Monitoring Report

###### PUBLICATION DATE  14.10.2020
VERSION  v. 1.1 
RELATED SUPPORT - TEMPLATE GUIDE Monitoring Report v. 1.1

This document contains the following Sections

Key Project Information

0 - Description of project

0 - Implementation of project

0 - Description of monitoring system applied by the project

0 - Data and parameters

0 - Calculation of SDG Impacts

0 - Safeguards Reporting

0 - Stakeholder inputs and legal disputes

### KEY PROJECT INFORMATION

##### Programme of Activity Information – (delete below table if N/A)

| GS ID of Programme | 10963 |
| --- | --- |
| Title of Programme | UpEnergy – Social and Climate Impact Programme |
| Version of POA-DD applicable to this monitoring report | 05 |
| Name and GS ID of fully Validated CPA/VPAs (i.e. non compliance check) | GS10967 Community Carbon Efficient Cooking Programme-VPA1 GS10968 Community Carbon Safe Water Drinking Programme-VPA2 GS10969 UpEnergy-Social and Climate Impact Programme- Cooking Devices VPA-3 GS10970 UpEnergy-Social and Climate Impact Programme- Cooking Devices VPA-4 GS10971 UpEnergy-Social and Climate Impact Programme- Cooking Devices VPA-5 GS11007 UpEnergy-Social and Climate Impact Programme- Cooking Devices VPA-6 by UpEnergy Group GS11008 UpEnergy-Social and Climate Impact Programme- Water filtration Devices VPA-7 by UpEnergy Group GS11210 UpEnergy-Social and Climate Impact Programme- Cooking Devices VPA-8 GS11211 UpEnergy-Social and Climate Impact Programme- Cooking Devices VPA-9 GS11245 UpEnergy-Social and Climate Impact Programme- Water filtration Devices VPA-10 GS11320 UpEnergy-Social and Climate Impact Programme- Safe Water VPA-11 GS11321 UpEnergy-Social and Climate Impact Programme- Safe Water VPA-12 GS11322 UpEnergy-Social and Climate Impact Programme- Cooking Devices VPA-13 GS11323 UpEnergy-Social and Climate Impact Programme- Cooking Devices VPA-14 GS11647 UpEnergy-Social and Climate Impact Programme- Cooking Devices VPA-15 GS12119 Community Carbon Efficient Cooking Programme-VPA 16 GS12120 Community Carbon Efficient Cooking Programme-VPA 17 GS12121 Community Carbon Efficient Cooking Programme-VPA 18 GS12122 Community Carbon Efficient Cooking Programme-VPA 19 |

##### Key Project Information

| GS ID (s) of Project (s) | 11211 |
| --- | --- |
| Title of the project (s) covered by monitoring report | UpEnergy – Social and Climate Impact Programme-Cooking Devices VPA-9 |
| Version number of the PDD/VPA-DD (s) applicable to this monitoring report | UpEnergy – Social and Climate Impact Programme-Cooking Devices VPA-9- Version 7.0 |
| Version number of the monitoring report | 3 |
| Completion date of the monitoring report | 28-08-2023 |
| Date of project design certification | 26-04-2023 |
| Date of Last Annual Report | NA |
| Monitoring period number | 1 |
| Duration of this monitoring period | 05/07/2022 – 28/02/2023 (both the days included) |
| Project Representative | UpEnergy Group and Community Carbon |
| Host Country | Mozambique |
| Activity Requirements applied | Community Services Activities Renewable Energy Activities Land Use and Forestry Activities/Risks & Capacities N/A |
| Methodology (ies) applied and version number | Technologies and Practices to Displace Decentralized Thermal Energy Consumption (TPDDTEC), version 03.1 |
| Product Requirements applied | GHG Emissions Reduction & Sequestration Renewable Energy Label N/A |

##### Table 1 - Sustainable Development Contributions Achieved

| Sustainable Development Goals Targeted | SDG Impact | Amount Achieved | Units/ Products |
| --- | --- | --- | --- |
| 13 Climate Action (mandatory) | Emission Reductions | 5,931 | tCO2(eq) |
| 1 (No poverty) | Percentage of users reporting money savings due to reduction in purchased fuel consumption in project | 90 | % |
| 3 (Good Health and Well being) | Percentage of users reporting reduction in smoke/PM after shifting to ICS in project | 90 | % |
| 5 (Gender Equality) | Percentage of users reporting time saving due to reduction in collected fuel consumption/cooking time | 90 | % |
| 7 (Affordable and Clean Energy) | Number of households with operational stoves | 9,356 | - |
| 8 (Decent Work and Economic Growth) | Total number of jobs created | 54 | - |
| 12 (Responsible Consumption and Production) | Average percentage fuel savings reported by users in the project | 42.98 | % |
| 15 (Life on Land) | Fuelwood equivalent savings reported by users in the project | 52.67 | tonnes-equivalent fuelwood/day |

##### Table 2 – Product Vintages

GS 11211 VPA 9- (ICS)

|  |  | Amount achieved |
| --- | --- | --- |
| Start Dates | End Dates | VERs |
| 05/07/2022 | 31/12/2022 | 4,312 |
| 01/01/2023 | 28/02/2023 | 1,619 |

#### SECTION A. DESCRIPTION OF PROJECT

##### A.1. General description of project

>>

The purpose of the VPA is to support clean cooking interventions such as energy-efficient biomass improved cookstoves (ICS that moves end-users up the energy ladder and reduce greenhouse gas (GHG) emissions from the burning of non-renewable woody biomass and/or charcoal for cooking in Mozambique.

The VPA is implemented by Community Carbon & UpEnergy Group is the Coordinating and Managing Entity (CME) of the PoA. UpEnergy will implement the programme in partnership with local partners and would ensure the last-mile distribution/installation of the Improved Cookstoves to the beneficiaries.

More than 2.5 billion people in world lack access to clean cooking facilities, relying instead on solid biomass, kerosene or coal as their primary cooking fuel. According to statistics household air pollution, mostly from cooking smoke, is linked to around 2.5 million premature deaths annually worldwide.

The lack of access to clean cooking remains very acute in sub-Saharan Africa with only 17% population having clean cooking access. The vast majority of people in sub-Saharan Africa thus rely on gathering biomass for cooking, in particular in rural areas, which dramatically damages health and impairs productivity improvements. Almost 490 000 premature deaths per year are related to household air pollution from the lack of access to clean cooking facilities, with women and children the worst affected. Forest degradation, sometimes leading to deforestation, is another serious consequence of the unsustainable harvesting of fuelwood. Improved Cookstoves (ICS) improve heat transfer efficiency as compared to the baseline conventional stoves, thereby reducing both the amount of fuel used by ICS and GHG emissions from the burning of non-renewable woody biomass and/or charcoal.

##### A.2. Location of project

>>

Host Party: Mozambique

Region/State/Province: All across Mozambique

City/Town/Community: All across Mozambique

The geographical location of Mozambique is depicted below. Mozambique officially the Republic of Mozambique is a country located in Southeastern Africa bordered by the Indian Ocean to the east, Tanzania to the north, Malawi and Zambia to the northwest, Zimbabwe to the west, and Eswatini (Swaziland) and South Africa to the southwest. with latitude18° 5’ 14.877” S and longitude 34° 45’ 0.7092” E. The geographical map of Mozambique is shown in below figure A.1.

Figure 1 - The physical/geographical boundary of the SSC-PoA: Mozambique

##### A.3. Reference of applied methodology

>>

- Technologies and Practices to Displace Decentralized Thermal Energy Consumption, version 3.1. https://globalgoals.goldstandard.org/standards/407_V3.1_EE_ICS_Technologies-and-Practices-to-Displace-Decentrilized-Thermal-Energy-TPDDTECConsumption-.pdf

- Requirements and Guidelines for carrying out usage surveys for projects implementing improved cooking devices. https://globalgoals.goldstandard.org/standards/RC_2020-IQ-TPDDTEC-3.1-and-Usage-Rate-Guidelines.pdf

##### A.4. Crediting period of project

>>

Crediting period is 05/07/2022 – 04/07/2037

Total length of crediting period is 15 years (5 years * 3)

#### SECTION B. IMPLEMENTATION OF PROJECT

##### B.1. Description of implemented project

>>

The VPA 9 involves the promotion and distribution of improved cookstoves (ICS) in Mozambique for use by households. The ICS disseminated through this programme replace the conventional inefficient biomass stove (3-stone fire)/traditional charcoal stoves with project stoves which combust biomass more efficiently or replaces use of biomass and improves heat transfer to pots, hence reducing use of non-renewable fuel and equivalent greenhouse gas emissions.

The VPAs involves marketing, distribution, and creating awareness for ICS for low-income households in Mozambique.

During the current monitoring period, only the distribution of ICS has been done, no distribution of any other project technology type such as ‘electric cooktop/cooker’ has been done in the monitoring period.

Improved Cookstoves (biomass)

The improved cookstoves reduce fuel consumption through use of a ceramic liner that increases combustion efficiency and retains heat. The stoves consist of a metal frame (called cladding) with interior ceramic liner for thermal insulation. The ICS has a provision that allows ash collection at the base. A thin layer of cement is placed between the cladding and the liner to bind the two. During use, a single pot rests at the top the stove. The fixed/inbuilt types are finished with either selected bricks, tiles or stainless steel. The movable types are of metallic stainless-steel cladding. The insulations include of pumice, mica, vermiculite and are bonded with ordinary cement. The smoke extracts through the chimney pipes which are metallic. The fitting stainless steel saucepans are of the thickness 3mm bottom and 2mm sides. The average lifespan of the improved cookstoves with residential use is 5 years.

a.	Examples of residential improved cookstoves are given below

Figure 3a: Residential Cookstoves

Improved Cookstoves

| Product Model | Dimension (In cm) | Avg. unit weight (In kg) | Thermal Efficiency (In %) | Estimated life (in years) | Fuel Type |
| --- | --- | --- | --- | --- | --- |
| Community Cookstoves | (Ø 28 x 26 H) | 11.00 | 38.00 | 5 | Charcoal |

The total number of ICSs (Community Cookstove) included under the VPAs are as follows:

| ICS Distributed | 10,395 |
| --- | --- |
| EPC Distributed | NIL |

The implementation of ICS is between 01/01/2022 to 28/02/2023.

B.1.1 Forward Action Requests

>>

Not Applicable

B.2. Post-Design Certification changes

>>

B.2.1. Temporary deviations from the approved Monitoring & Reporting Plan, methodology or standardized baseline

>>

Not Applicable

B.2.2. Corrections

>>

Not Applicable

B.2.3. Changes to start date of crediting period

>>

Not Applicable

B.2.4. Permanent changes from the Design Certified monitoring plan, applied methodology or applied standardized baseline

>>

Not Applicable

B.2.5. Changes to project design of approved project

>>

Not Applicable

#### SECTION C. DESCRIPTION OF MONITORING SYSTEM APPLIED BY THE PROJECT

>>

The monitoring system applied involves a number of key elements to ensure that the CME and VPA-Implementer have high-quality, unbiased and reliable information regarding the performance of the project.

Monitored Systems

- Total Distribution Record: The total distribution record documents the information listed below for the technologies implemented. A carbon waiver including a warranty card has been distributed with each stove distributed. The PP makes every effort to retrieve this information (paper form or electronically but cannot guarantee the collection of information for waivers and warranties with every stove due to challenges such as high rates of illiteracy and logistical challenges. The total distribution record has been kept electronically and with supporting evidence from paper records and/or electronic records and will be provided to the GS-VVB at verification. The Total Distribution Record contains:

- Unique identification of stove using stove serial number

- Name, address and telephone (as available)

- Date of distribution and model/type of project technology distributed

- Quantity of project technology distributed as evidenced by invoices

Frequency: Ongoing

- Project Database: Each VPA will have a specific Project Database that records each CEP crediting in that VPA. Every CEP listed in the Total Distribution Record will be transferred into the Project Database of this VPA as needed to expand the number of CEP deployed, until the maximum threshold for this VPA is reached. In addition to the information provided in the Total Distribution Record, the VPA-specific Project Database will record user details (enough for end-user identification and follow-up) for all, or a subset of all, appliances deployed. End-user details recorded are:

- Name

- Telephone, or address (as available)

- Type of stove (stove model) and fuel the ICS is replacing: Example – traditional or improved baseline stoves, or wood or charcoal fuel.

ICS with end-user details recorded here will be used to determine other information needed using as many samples as commensurate with representative sampling to calculate the emission reductions of the project, following TPDDTEC v3.1-page 28/29 guidance on Total Sales Record.

- Continued use of displaced traditional stoves

The replaced low efficiency devices are disposed of and not used within the boundary or within the region. Monitoring surveys conducted on households (HH) using CEP also investigate the extent to which baseline traditional stoves are still in use. Whenever it is found that a traditional stove is still used, even in a secondary role, the HH is encouraged to discard their traditional stove through the Disposal Policy. Evidence of use of traditional stoves in other kitchens is not possible as clients are encouraged to return the stove for additional warranty period for project stoves. In any case, the emissions reductions are calculated based on the total fuel usage in the Household (including all stoves), which takes into account any fuel usage in the pre-project devices.

- Disposal Policy: The project activity encourages full adoption of project stove distributed. To incentivize full adoption, the project proponents extend a full-year warranty in exchange for the disposal of a household’s traditional stove.

- Organizational structure of monitoring and inclusions:

| Person | Role |
| --- | --- |
| Project database administrator | The database administrator is responsible for updating and maintaining all electronic databases. Required competencies include experience with data management systems (eg. Excel, STATA, or SPSS), minimum 2 years working experience in a similar field, and at minimum a bachelor’s degree from an institution of higher education. |
| Monitoring team | The monitoring team is assigned by the PP to conduct the user interviews and appliance tests during the periodic sampling and reports the results to the database administrator. Proper training has been provided to all enumerators/monitoring team before every survey exercise. The skills and experience required for the data collection activities include: - Experience conducting surveys/tests - Experience conducting door-to-door surveys of biomass consumption. - Local language skills (especially important for input to questionnaire design and interviewing of end users) - English language skills - Cultural awareness - Numerical proficiency - Data entry skills |

- Quality Assurance/Quality control

As the Project Activity is intended to include multiple regions within a country with a high level of cultural diversity as well as different end user groups, there is no “one size fits all” approach for dealing with these issues. However, in order to avoid many of these problems the PP undertakes the following strategies, tailoring the specific approach to the local circumstances:

- Ensuring end user awareness. At the time of distribution, the CEP customer is made aware that they are required to participate in monitoring activities. This is done via a written statement (in English and local language where appropriate) on the carbon waiver form, or via alternative means such as training distribution personnel to explain the importance of monitoring to each customer.

- Questionnaire design. The design of the questionnaire it has been ensured that the questions are non-intrusive and easy to understand for both the interviewee and interviewer. For example, when conducting sampling to estimate the parameter µy a simplified approach has been designed to avoid the need for asking customers how much money they spend on fuel.

- Drawing on local knowledge. The local contractors hired by the PP in the country plays an important role in tailoring the approach to suit local circumstances. For example, in some instances, it is essential for a local person to conduct the interview in order to obtain accurate results, e.g. to explain to the end user that their old stove will not be removed if they admit to its continued use.

- Quality of contractors. The third parties hired by the PP to carry out sampling is required to demonstrate a high level of cultural awareness, local language skills and appropriate experience with data entry and data management. The PP ensures that contractors are adequately trained for the tasks they are contracted for (e.g., carrying out of KPTs in line with a methodology supported by an appropriate international body/standards). Training is also be provided on how to deal with non-responses, refusals and other problems should these occur.

- Fuel switch scenario

The potential increase in fuels with different carbon intensities (such as increased charcoal usage on users that consumed originally firewood prior to the purchase of the ICS) are accounted by monitoring consumption of biomass distinguishing the different fuels as charcoal and firewood.

#### SECTION D. DATA AND PARAMETERS

##### D.1. Data and parameters fixed ex-ante or at renewal of crediting period

>>

SDG13

| Data/parameter | EFb,CO2 |
| --- | --- |
| Unit | tCO2/TJ |
| Description | CO2 emission factor arising from use of fuels in baseline Scenario |
| Source of data | IPCC default value as per 2006 IPCC Guidelines for National Greenhouse Gas Inventories, volume 2, chapter 2 (Table2.5) |
| Value(s) applied | Fuelwood (Residential): 112 |
| Choice of data or Measurement methods and procedures | Default IPCC value for fuelwood is applied |
| Purpose of data | CO2 Emission calculation in baseline |
| Additional comment | - |

| Data/parameter | EFb,non-CO2 |
| --- | --- |
| Unit | tCO2/TJ |
| Description | Non-CO2 emission factor arising from use of fuels in baseline scenario |
| Source of data | IPCC default value considering AR5 GWP. |
| Value(s) applied | Fuelwood (Residential): 9.46 |
| Choice of data or Measurement methods and procedures | Default IPCC value for fuelwood is applied (AR5 GWP) |
| Purpose of data | Non-CO2 Emission calculation in baseline |
| Additional comment | - |

| Data/parameter | EFp,CO2 |
| --- | --- |
| Unit | tCO2/TJ |
| Description | CO2 emission factor arising from use of fuels in project Scenario |
| Source of data | IPCC default value as per 2006 IPCC Guidelines for National Greenhouse Gas Inventories, volume 2, chapter 2 (Table 2.5) |
| Value(s) applied | Fuelwood (Residential): 112 |
| Choice of data or Measurement methods and procedures | Default IPCC value is applied (AR5 GWP) |
| Purpose of data | CO2 Emission calculation in project scenario |
| Additional comment | - |

| Data/parameter | EFp,non-CO2 |
| --- | --- |
| Unit | tCO2/TJ |
| Description | Non-CO2 emission factor for methane arising from use of fuels in project Scenario |
| Source of data | IPCC default value considering AR5 GWP. |
| Value(s) applied | Fuelwood (Residential): 9.46 |
| Choice of data or Measurement methods and procedures | Default IPCC value for fuelwood is applied |
| Purpose of data | Non-CO2 Emission calculation in project scenario |
| Additional comment | - |

| Data/parameter | NCVb |
| --- | --- |
| Unit | TJ/ton |
| Description | Net calorific value of the fuels used in the baseline |
| Source of data | IPCC default 2006, volume 2, chapter 1 (Table 1.2) |
| Value(s) applied | Fuelwood: 0.0156 |
| Choice of data or Measurement methods and procedures | Default IPCC value for fuelwood is applied |
| Purpose of data | Emission calculation in project scenario |
| Additional comment | - |

| Data/parameter | NCVp |
| --- | --- |
| Unit | TJ/ton |
| Description | Net calorific value of the fuels used in the project |
| Source of data | IPCC default 2006, volume 2, chapter 1 (Table 1.2) |
| Value(s) applied | Fuelwood: 0.0156 |
| Choice of data or Measurement methods and procedures | Default IPCC value for fuelwood is applied |
| Purpose of data | Emission calculation in project scenario |
| Additional comment | - |

| Data/parameter | fNRB,i,y |
| --- | --- |
| Unit | Fractional non-renewability |
| Description | Non-renewability status of woody biomass fuel in scenario i during year y |
| Source of data | Assessment based on CDM Methodological tool 30: Calculation of the fraction of non-renewable biomass, Version 03.0 |
| Value(s) applied | 0.78 |
| Choice of data or Measurement methods and procedures | Assessment based on CDM Methodological tool 30: Calculation of the fraction of non-renewable biomass, Version 03.0 |
| Purpose of data | CO2 Emission calculation in project scenario |
| Additional comment | The fNRB value will remain fixed during the crediting period |

| Data / Parameter | Pb,y |
| --- | --- |
| Unit | tonnes/day |
| Description | Specific fuel consumption for an individual technology in baseline scenario b during year y converted to tons/day |
| Source of data | Baseline survey and KPT is conducted and value is fixed ex-ante for subsequent issuance. |
| Value(s) applied | 0.0131 |
| Choice of data or Measurement methods and procedures | Based on baseline Field Test updates. KPT conducted for 3 consecutive dates by measuring fuel and moisture in fuel. Compiled data and averaged the data of 3 days for specific fuel consumption. The details are shown in baseline KPT |
| Purpose of data | To calculate baseline emissions |
| Additional comment | This parameter is fixed ex-ante for first crediting period. The value of Charcoal would be converted into equivalent fuelwood, by using 1:6 conversion factor from charcoal to wood. Calibration details of the equipment used to assess baseline fuel consumption are as below Name of the equipment; Model/Type; Serial number; Calibration date; Validity of calibration. Counter Scale; Dahongying; UPE23031701; 17- 03-2023; 16-03-2024. Hanging Scale; N/A; 2023031702; 17-03-2023; 16-03-2024. Moisture Meter; Ryobi; V3210501620; 24-02-2023; 23-02-2024. |

SDG 1: No Poverty

| Data/parameter | HHSbaseline |
| --- | --- |
| Unit | Percentage (%) |
| Description | % users reporting money saving due to reduced fuel consumption in baseline |
| Source of data | - |
| Value(s) applied | 0 |
| Choice of data or Measurement methods and procedures | - |
| Purpose of data | SDG 1 impact calculation |

SDG 3: Good Health and Well Being

| Data/parameter | HHsmokebaseline |
| --- | --- |
| Unit | Percentage (%) |
| Description | % users reporting reduction in smoke/PM emissions while cooking on improved stove in baseline |
| Source of data | - |
| Value(s) applied | 0 |
| Choice of data or Measurement methods and procedures | - |
| Purpose of data | SDG 3 impact calculation |
| Additional Comment | - |

SDG 5: Gender Equality

| Data/parameter | HHtimebaseline |
| --- | --- |
| Unit | % |
| Description | % users reporting time savings for cooking /fuel collection |
| Source of data | - |
| Value(s) applied | 0 |
| Choice of data or Measurement methods and procedures | - |
| Purpose of data | SDG 5 impact calculation |
| Additional Comment | - |

SDG 7: Affordable and clean energy

| Data/parameter | HHcleanbaseline |
| --- | --- |
| Unit | Number |
| Description | Access to affordable and clean energy (Number of ICS operating) |
| Source of data | - |
| Value(s) applied | 0 |
| Choice of data or Measurement methods and procedures | - |
| Purpose of data | SDG 7 impact calculation |
| Additional Comment | - |

SDG 8: Decent Work and Economic Growth

| Data/parameter | EGbaseline |
| --- | --- |
| Unit | Number |
| Description | Quantitative Employment and income generation |
| Source of data | - |
| Value(s) applied | 0 |
| Choice of data or Measurement methods and procedures | - |
| Purpose of data | SDG 8 impact calculation |
| Additional Comment | - |

SDG 12: Responsible Consumption and Production

| Data/parameter | FCbaseline |
| --- | --- |
| Unit | tonnes/year |
| Description | Fuel consumption in baseline |
| Source of data | - |
| Value(s) applied | 4.78 |
| Choice of data or Measurement methods and procedures | This is calculated from the average fuel consumption per HH in baseline (tonnes eq. fuelwood/HH/day) ie 0.0131(MT/HH/day) * 365 day = 4.78 tonnes/year |
| Purpose of data | SDG 12 impact calculation |
| Additional Comment | - |

SDG 15: Life on Land

| Data/parameter | FC baseline |
| --- | --- |
| Unit | tonne-eq.fuelwood/HH/day |
| Description | Average Fuel consumption per HH in baseline |
| Source of data | Baseline survey |
| Value(s) applied | 0.0131 |
| Choice of data or Measurement methods and procedures | - |
| Purpose of data | SDG 15 impact calculation |
| Additional Comment | - |

##### D.2 Data and parameters monitored

>>

SDG 13

| Data / Parameter | Pp,y |
| --- | --- |
| Unit | tonnes/day |
| Description | Specific fuel consumption for an individual technology in project scenario p during year y converted to tons/day |
| Source of data | Project Kitchen Performance Test |
| Value(s) applied | 0.00747 |
| Measurement methods and procedures | Kitchen Performance Test is carried out in accordance with Annex 4 of the applied methodology TPDDTEC |
| Monitoring frequency | Every 2 years |
| QA/QC procedures | Transparent data analysis and reporting |
| Purpose of data | To calculate emission reductions |
| Additional comment | A single project fuel consumption parameter is weighted to be representative of the quantity of project technologies of each age being credited in a given project scenario. The value of Charcoal would be converted into equivalent fuelwood, by using 1:6 conversion factor from charcoal to wood. Calibration details of the equipment used to assess project fuel consumption are as below Name of the equipment; Model/Type; Serial number; Calibration date; Validity of calibration. Portable Weighing Scale; VC00315; 202351901; 19-05-2023; 18-05-2024. Moisture Meter; Ryobi Moisture Meter 2-In-1; HTMM210; 14-03-2023; 13-03-2024. Counter Scale; AM SUB25294; UPE23031703; 17-03-2023; 16-03-2024. |

| Data / Parameter | Up,y |
| --- | --- |
| Unit | Percentage |
| Description | Usage rate in project scenario p during year y |
| Source of data | Annual usage survey |
| Value(s) applied | 90.00% |
| Measurement methods and procedures | Ex-post monitoring and surveys carried out to determine the number of appliances still in operation by field survey |
| Monitoring frequency | Annual |
| QA/QC procedures | Transparent data analysis and reporting |
| Purpose of data | To calculate emission reductions |
| Additional comment | TPDDTEC Usage Requirements has been followed. |

| Data / Parameter | Np,y |
| --- | --- |
| Unit | Project technologies credited (units) |
| Description | Technologies in the project database for project scenario p through year y |
| Source of data | Total sales record |
| Value(s) applied | 10,395 |
| Measurement methods and procedures | The total number of appliances deployed during period y are tracked in the Project Database of the specific VPA, which is updated regularly. All appliances distributed will be recorded. Any appliance not recorded in the Project Database will not be credited for emission reductions. |
| Monitoring frequency | Continuous |
| QA/QC procedures | Transparent data analysis and reporting |
| Purpose of data | To calculate emission reductions |
| Additional comment | The total sales record is divided based on project scenario to create the project database |

| Data / Parameter | LEp,y |
| --- | --- |
| Unit | tCO2e per year |
| Description | Leakage in project scenario p during year y |
| Source of data | Baseline and monitoring surveys |
| Value(s) applied | 0 |
| Measurement methods and procedures | Not applicable |
| Monitoring frequency | Every two years |
| QA/QC procedures | Transparent data analysis and reporting |
| Purpose of data | To calculate emission reductions |
| Additional comment | The potential leakage probability is very low due to the following reasons: 1. The project developer encourages clients to return back their traditional stove by creating incentive such as additional warranty. Thus, it will not be reused outside the project boundary. 2. The project does not have any significant impact on NRB fraction in the project boundary Aggregate leakage can be assessed for multiple project scenarios, if appropriate. |

| Data / Parameter | µy |
| --- | --- |
| Unit | Fraction |
| Description | Adjustment to account for any continued use of pre-project devices (baseline stove) in the project scenario during the year y |
| Source of data | Monitoring/usage surveys |
| Value(s) applied | Value captured in the Pp,y |
| Measurement methods and procedures | Monitoring surveys to capture cooking habits and stove usage of households in the region, including quantification of use of baseline devices, by formulating questions and/or collecting evidences to determine the frequency of usage of both the project devices and baseline devices |
| Monitoring frequency | Annual |
| QA/QC procedures | - |
| Purpose of data | To calculate emission reduction |
| Additional comment | The project developer encourages clients to return back their traditional stove by creating incentive such as additional warranty. Emissions reductions are calculated based on the total fuel usage in the Household (including all stoves – baseline and project ICS), which takes into account any fuel usage in the pre-project devices. |

| Data / Parameter | ր𝑛𝑒𝑤,i |
| --- | --- |
| Unit | Percentage |
| Description | Efficiency of the project cookstove |
| Source of data | -Performance report -WBT Test |
| Value(s) applied | NA |
| Measurement methods and procedures | - |
| Monitoring frequency | Annual |
| QA/QC procedures | - |
| Purpose of data | To calculate project emissions |
| Additional comment | Since KPT has been conducted for the current monitoring period, no WBT has been conducted for this monitoring period. PP has not applied WBT based approach (ageing test approach) to calculate project fuel consumption for the current monitoring period. |

SDG 1: End poverty

| Data / Parameter | HHSproject |
| --- | --- |
| Unit | Percentage |
| Description | Percentage of households confirming money saving with improved cookstove |
| Source of data | Database |
| Value(s) applied | 90% |
| Measurement methods and procedures | The number of clean energy products distributed are recorded as part of the CME database which will be used to estimate this parameter |
| Monitoring frequency | Continuous |
| QA/QC procedures | - |
| Purpose of data | Monitoring of SDG 1 |
| Additional comment | - |

SDG 3: Good Health and Well-being

| Data / Parameter | HHsmokeproject |
| --- | --- |
| Unit | Percentage (%) |
| Description | Percentage of households confirming less smoke with the use of improved stove |
| Source of data | Monitoring Survey |
| Value(s) applied | 90% |
| Measurement methods and procedures | This parameter will be monitored as part of the monitoring survey. Users will be asked as part of monitoring survey if there is less smoke with the use of improved stove. |
| Monitoring frequency | Once every 2 years (biennially) |
| QA/QC procedures | - |
| Purpose of data | Monitoring of SDG 3 |
| Additional comment | The parameter will be measured qualitatively. |

SDG 5: Gender Equality

| Data / Parameter | HHtimeproject |
| --- | --- |
| Unit | Percentage (%) |
| Description | % HH reporting time saving associated with cooking and fuel collection |
| Source of data | Monitoring Survey |
| Value(s) applied | 90% |
| Measurement methods and procedures | This parameter will be monitored as part of the monitoring survey. Users will be asked as part of monitoring survey if they have been able to save time due to use of clean energy products |
| Monitoring frequency | Once every 2 years (biennially) |
| QA/QC procedures | - |
| Purpose of data | Monitoring of SDG 5 |
| Additional comment | The parameter will be measured qualitatively. |

SDG 7: Affordable and Clean Energy

| Data / Parameter | HHcleanproject |
| --- | --- |
| Unit | Number |
| Description | Access to affordable and clean energy (Number of ICS operating) |
| Source of data | CME Database |
| Value(s) applied | 9356 |
| Measurement methods and procedures | The number of improved cookstoves distributed will be recorded as part of the CME database which will be used to estimate this parameter |
| Monitoring frequency | Continuous |
| QA/QC procedures | - |
| Purpose of data | Monitoring of SDG 7 |
| Additional comment | - |

SDG 8: Decent Work and Economic Growth

| Data / Parameter | EGproject |
| --- | --- |
| Unit | Number |
| Description | Total no of jobs created |
| Source of data | CME Database |
| Value(s) applied | 54 |
| Measurement methods and procedures | CME will maintain a record of jobs created as a result of implementation of this programme in form employee list/ payroll system/ contracts/ pay slips |
| Monitoring frequency | Continuous |
| QA/QC procedures | - |
| Purpose of data | Monitoring of SDG 8 |
| Additional comment | - |

SDG 12: Responsible Consumption and Production

| Data / Parameter | FCproject |
| --- | --- |
| Unit | tonnes/HH/annum |
| Description | Consumption of non-renewable biomass per household |
| Source of data | Monitoring survey |
| Value(s) applied | 2.73 |
| Measurement methods and procedures | Recorded during KPT conducted in HH. |
| Monitoring frequency | Annually |
| QA/QC procedures | - |
| Purpose of data | Monitoring of SDG 12 and 15 |
| Additional comment | - |

SDG 15: Life on Land

| Data / Parameter | FCHHproject |
| --- | --- |
| Unit | Tonnes eq.fuelwood/day |
| Description | Average Fuel consumption per HH in project scenario |
| Source of data | - |
| Value(s) applied | 0.0131 |
| Measurement methods and procedures | - |
| Monitoring frequency | Annually |
| QA/QC procedures | - |
| Purpose of data | Monitoring of SDG 12 and 15 |
| Additional comment | - |

##### D.3. Comparison of monitored parameters with last monitoring period

Improved Cookstove

| Data/Parameter | Value obtained in this monitoring period | Value obtained last monitoring period |
| --- | --- | --- |
| Pp,y | 0.00747 | NA |
| Np,y | 10,395 | NA |
| Up,y | 90% | NA |

##### D.4. Implementation of sampling plan

>>

Project technologies which are part of the VPA have similar design and performance characteristics. All have same fundamental combustion technology and their respective thermal efficiencies or specific consumptions do not differ by more than +/-5% in absolute terms which can be verified by the technical specification sheet. Hence, these will be included under a single project scenario. Sampling will be undertaken separately for different project scenarios. This project has employed representative sampling in line with the requirements of the TPDDTEC version 3.1. If homogeneity of population can be demonstrated, or differences are taken into account (stratification) in the sample size calculation then 90/10 confidence/precision will be applied.

Baseline Survey and Kitchen Performance Test (KPT)

Baseline surveys and KPTs were conducted to establish the stove and fuel (wood/charcoal) usage trends. The survey also established kitchen regime characteristics and the type of baseline technology being replaced by the project stoves. The baseline study was done in the month of April 2023.

The sample size considered were as follows:

As per TPDDTEC version 3.1, the group size above 1000, minimum of 100 samples need to be carried out for baseline survey. Hence the number of samples surveyed was 187 (including 90 KPTs) which covered all administrative regions of Mozambique. Field questionnaires for both baseline survey and Kitchen performance test was designed to provide accurate representation, understanding of the types of baseline technologies etc. Quality assurance measures were put in place to ensure data quality for each phase of survey.

The baseline questionaries included name of the household, age group, number of people, contact details, geographical coordinates etc. The baseline survey also included current cooking device, type of fuel being used, etc. Baseline survey was conducted for 187 households (including 90 KPTs) and captured the information which is mentioned above. The kitchen performance questionnaire included fuel usage, moisture content of fuel, etc. The kitchen performance was conducted for 3 consecutive days to capture the fuel consumption and moisture content of the fuel. KPT was conducted for 90 households in all administrative regions which also includes the households surveyed for baseline study.

Result:

A total of 90 samples were considered (since the pilot data showed a Covariance of 0.4. This is as per page 58 of the TPDDTEC methodology). The quantity of biomass (charcoal) being consumed in each household surveyed is presented below. KPT was conducted for 90 households across Mozambique. The average consumption of fuel based on 90/10 precision, is tabulated below table:

| Fuel Type | Quantity of charcoal consumed MT/HH/day | Quantity of eq. firewood consumed, MT/HH/day |
| --- | --- | --- |
| Charcoal | 0.00218 | 0.0131 |

It can be seen from the above table that 0.0131 MT of firewood consumed every day from each household which is 4.78 MT/HH/yr.

Project monitoring survey

Description of the implemented sampling design and target population

The project usage monitoring conducted for a period of 3 weeks which was started on 04th July 2023 to 14th July 2023.

The monitoring survey investigates changes over time in a project scenario by surveying end users with project technologies on an annual basis. A usage survey determines the drop off rates as project technologies age and users switch back to the baseline technology.

Random sampling has been applied for determining the usage rate for each project scenario separately. The sample size determined has been distributed within each vintage category based on percentage of stoves in corresponding. The minimum total sample size is 100, with at least 30 samples for project technologies of each age being credited.

The required samples Vs actual samples monitored in the monitoring period is shown below. The total samples required is 100 which is in line with the TPDDTEC methodology.

| Technology | Vintage /Age | Required samples / vintage | Samples monitored |
| --- | --- | --- | --- |
| ICS | 2022 and 2023 | 100 samples and 30 per vintage | 160 (108 KPTs) |

As can be seen from the above table it is evident that the condition of minimum samples of overall sales and vintage split are as per the applied methodology TPDDTEC version 3.1.

All interviews in the monitoring and usage survey have been conducted by UpEnergy staff or by hired experts who were trained before and include expert observation by the interviewer within the kitchen in question. The GS cookstove usage rate guidelines have been followed in arriving at the usage rate for the current monitoring period.

The monitoring survey has been carried out along with the usage survey, to assess end-user characteristics such as technology use, fuel consumption and seasonal variation.

Project KPT

Collected data

For determining the fuel consumption in the baseline / project scenarios, the random sampling for KPTs was applied. The sample size determined was distributed within each age category based on percentage of stoves in corresponding age category. For determining the fuel consumption in the baseline / project scenario the KPT sample size determination shall be based on Kitchen Performance Test Protocol. Alternatively, the guideline given in TPDDTEC version 3.1 may be used for determination of sample size.

Project has carried out an independent sampling and hence the sample size is calculated based on the following table given in the TPDDTEC methodology version 3.1. The Covariance has been calculated based on data.

According to the initial study, a coefficient of variation (CoV) of 1.2 was determined. The methodology stated that 90 samples should be randomly selected from the ICS age group. Thus, as per the methodology, 108 samples for project KPTs were randomly selected considering the age group of ICS.

The project fuel consumption test can also be replaced with ageing test approach from second monitoring period.

In case, for future reporting periods, if PP opts for the ageing test approach instead of biennial project KPTs, annual WBTs would be conducted on a representative sample of each age group. The sample size would be calculated using student’s t-distribution approach for mean based parameter. The sample size is appropriately sampled from each age group and enough so that the results comply with the 90/10 rule. In case of not meeting the required confidence/precision, lower bound value will be used. The WBT shall be carried out along with the project KPTs prior to 1st issuance and then subsequently WBTs shall be carried out annually to monitor the degradation in the efficiency of the ICS. The WBTs should be conducted in the last 3 months of the monitoring period or after the monitoring period, provided it is representative of each age group. For this monitoring period, WBTs are not conducted.

Description of implemented single sampling design for monitoring parameters

The objective of the sampling plan for this VPA is to determine:

- Up,y: Usage rate in project scenario p during year y. This would be based on usage survey, to be conducted upon the representative sample of operational stoves.

- μy: Adjustment to account for any continued use of pre-project devices (baseline stove) in the project scenario during the year y. Survey questionnaires administered to a sample of end users will elicit visual inspections of the household and if necessary, an interview to confirm whether they are still using a baseline stove and, in that case, to obtain self-reported estimates of the amount of non-renewable biomass consumed per day in traditional stoves in parallel to the project stove.

- Pp,y: Specific fuel consumption for an individual technology in project scenario. The actual value has been determined based on the Kitchen Performance Test, which has been carried out in accordance with Annex 4 of the applied methodology TPDDTEC version 3.1.

The objective of the sampling was to obtain an unbiased and reliable estimate of the proportion or mean value of the above listed parameters over the course of the monitoring period, and with 90/10 confidence/precision as demonstrated in the ER sheet tabs “Sample Size Cal and Results_ICS”.

Even though a higher usage rate (93.03%) has been demonstrated through the usage surveys conducted, PP has applied the maximum cap of 90% on stove model wise usage rate, complying with the requirements pertaining to “good practice” as explained below:

Step 1: Defining stove use and non-use:

As evidenced from the usage survey forms, usage of project stove (details like stove usage, stove usage frequency in a day and week, average hours of cooking in a day, whether baseline stove still in use, etc.) are captured.

Step 2. Household Usage Survey:

The project developer has carried out an in-person usage survey to determine stove use. The sample size considered for the usage survey is 160, which is in line with the requirements mentioned in the applicable methodology. The usage survey included the following key elements.

i: Kitchen observation: The surveyors have visited the household during the time of survey, to gather objective information to support the usage survey findings (e.g., if the stove is warm to the touch, ashes present etc.). This can be substantiated from section 1.4 of the usage survey which clearly states that the surveyor should observe the stove which is operational (Ash/char/spillage observed near the stove, Is it warm to touch).

ii. Interview with the primary cook: The surveyor interviewed the primary cook of the household to gather information on stove use patterns. This can be substantiated from section 1.2 of the usage survey.

iii. Photos of the cooking area(s): Surveyors have taken photographs of the project stoves to gather visual data on the status of the stove. This can be substantiated from section 1.4 of the usage survey which clearly states that the surveyor should take 2 to 3 photos inside the kitchen along with project stove and other stoves, if any. Sample photographs have been shared with the VVB.

iv. GPS coordinates: GPS co-ordinates are captured during the monitoring survey additionally, as per the guideline, photographs taken under iii. above may also be used to meet this requirement.

Step 3. Verification checks:

As required by the guideline (5-10% of the surveyed households), 11 number of ICS users’ verification checks have been performed by the PP prior to verification by the VVB. The details collected through telephone have been recorded and shared with the VVB.

Step 4: Field team training and supervision:

The PD has provided training and supervision necessary to ensure field teams have the capacity required to complete usage survey successfully. The necessary documents like training attendance sheet, pictures of the training have been shared with the VVB during the verification process.

Step 5: End-User Training and follow up visits:

The PD has provided end-user training on project stove use via demonstrations during ICS sale and follow-up visits. This can be substantiated from section 1.4 of the usage survey through the response to the question ‘Was the training on project stove usage demonstrated’.

Step 6: Awareness campaign:

The PD has organized several awareness campaigns to make end-user aware about the benefits of continuous use of project stove and key product attributes through ‘last mile distribution’ campaign, end user training. Pictures from last mile distribution have been shared with the VVB.

Data obtained from the samples were used to estimate proportions and mean values for the parameters described above. The values were then being factored into the emissions reduction calculations. For the demonstration of required confidence/precision being met, please refer to the ER calculation sheet for detailed calculation.

Demonstration of whether the samples were randomly selected and are representative of the population

The samples were randomly selected using Stratified Random Sampling across the VPA population. Random numbers were generated using online random number generator for each stratum and the ICS corresponding to the random numbers obtained, were selected as samples to be monitored. Under Stratified Random Sampling, the entire target population has an equal chance of being selected, thus the samples selected were deemed to be representative of population.

Demonstration of whether the required confidence/precision has been met:

Reliability for Usage fraction:

Reliability for project fuel consumption:

#### SECTION E. CALCULATION OF SDG IMPACTS

##### E.1. Calculation of baseline value or estimation of baseline situation of each SDG Impact

>>

The Baseline value for the current monitoring period has been calculated as follows:

SDG 13: Climate Action

The baseline emissions in case of different fuel in project and baseline scenario is calculated using the below equation:

Baseline Emissions

BEb,y = Bb,y *(( fNRB,b,y * EFb,fuel,CO2) + EFb,fuel,non-CO2) * NCVb,fuel

Where:

| BEb,y | Emissions for baseline scenario b during the year y in tCO2e |
| --- | --- |
| Bb,y | Quantity of fuel consumed in baseline scenario b during year y, in tons |
| f NRB,b,y | Fraction of biomass used in year y for baseline scenario b that can be established as non-renewable biomass, 0.78 |
| NCVb,fuel | Net calorific value of the fuel that is substituted or reduced (IPCC default for wood fuel, 0.0156 TJ/ton) |
| EFb,fuel,CO2 | CO2 emission factor of the fuel that is substituted or reduced. 112 tCO2/TJ for Wood/Wood Waste, or the IPCC default value of other relevant fuel |
| EFb,fuel,nonCO2 | Non-CO2 emission factor of the fuel that is reduced, 9.46 tCO2/TJ |

Where:

| Bb,y | Quantity of fuel consumed in baseline scenario b during year y, in tons |
| --- | --- |
| Pb,y | Specific fuel consumption for an individual technology in baseline scenario b during year y converted to tons/day, 0.0131 tonnes/HH/day |
| Np,y | Project technology-days in the project database for project scenario p through year y (775,100 technology days) |

Hence for ICS

Bb,y = 0.0131 * 775,100 = 10,154 tonnes

And BEb,y = 10154 * (0.78 * 112) + 9.46) * 0.0156 = 15,336 tCO2e

SDG 1: No Poverty

1.4.1 Proportion of population living in households with access to basic services

HHSBaseline       % HH reporting money saving due to reduced fuel consumption in baseline = 0

Justification: There is no money savings due to reduced fuel consumption the baseline scenario as the traditional cookstove burn inefficiently which increases the cost need to purchase more fuel for cooking.

SDG 3: Good Health and Well Being

3.9.1 Mortality rate attributed to household and ambient air pollution

HHsmokebaseline % HH reporting reduction in smoke/PM emissions while cooking on traditional stove in baseline = 0

Justification: Before project intervention the household smoke is not reduced, and baseline scenario is assumed as “0” in terms of reduction in smoke/PM emissions while cooking on traditional cookstove.

SDG 5: Gender Equality

5.4.1 Proportion of time spent on unpaid domestic and care work, by sex, age and location

HHtimebaseline	% HH reporting time saving due to reduced collected fuel consumption / cooking time in baseline = 0

Justification: The amount of time saving due to reduced collected fuel consumption/cooking is NIL as it involves the baseline scenario technology is traditional cookstove.

SDG 7: Affordable and Clean Energy

7.1.2 Proportion of population with primary reliance on clean fuels and technology

HHcleanbaseline Access to affordable and clean energy (% of operating ICS units under Baseline) = 0

Justification: There is no ICS distribution in the baseline scenario.

SDG 8: Decent Work and Economic Growth

8.5.1 Average hourly earnings of female and male employees, by occupation, age and persons with disabilities

EGbaseline	Quantitative Employment and income generation (Number of person (male and female) hired under Baseline) = 0

Justification: The employment opportunities are created after project implementation started, thus in the baseline scenario the employment creation is NIL.

SDG 12: Responsible Consumption and Production

12.2.2 - Domestic material consumption, domestic material consumption per capita, and domestic material consumption per GDP

FCBaseline	Average fuel consumption per HH in Baseline = 0.0131 tonnes/day/HH (0.0131*365= 4.78 tonnes/day/HH)

Justification: The average fuel consumption in the baseline scenario based on baseline studies are at 4.78 tonnes/year/HH for all VPAs under this monitoring period.

SDG 15: Life on Land

15.2 By 2020, promote the implementation of sustainable management of all types of forests, halt deforestation, restore degraded forests and substantially increase afforestation and reforestation globally

FCbaseline	Average fuel consumption per HH in Baseline = 0.0131 tonnes/day/HH (0.0131*365= 4.78 tonnes/day/HH)

Justification: The average fuel consumption in the baseline scenario based on baseline studies are at 4.78 tonnes/year/HH for all VPAs under this monitoring period.

##### E.2. Calculation of project value or estimation of project situation of each SDG Impact

>>

SDG 13: Climate Action

Project emissions due to biomass consumption is applicable for this project. For the current monitoring period, ICS project emissions are calculated as follows.

PEp,y = Bp,y * ((fNRB,p,y * EFp,fuel,CO2) + EFp,fuel,non-CO2) * NCVp,fuel

Bp,y = Np,y * ((Pp,y * Up,y) + (Pb,y * (1 – Up,y)))

Where:

| PEp,y | Emissions for project scenario p during the year y in tCO2e |
| --- | --- |
| Bp,y | Quantity of fuel consumed in project scenario p during year y, in tons |
| f NRB,y | Fraction of biomass used in year y that can be established as non-renewable biomass, 0.78 |
| NCVp,fuel | Net calorific value of the project fuel that is substituted or reduced (IPCC default for wood fuel, 0.0156 TJ/ton) |
| EFp,fuel,CO2 | CO2 emission factor of the fuel that is substituted or reduced. 112 tCO2/TJ for Wood/Wood Waste, or the IPCC default value of other relevant fuel |
| EFp,fuel,nonCO2 | Non-CO2 emission factor of the fuel that is reduced, 9.46 tCO2/TJ |
| Np,y | Project technology-days in the project database for project scenario p through year y, 775,100 |
| Pp,y | Specific fuel consumption for an individual technology in project scenario p during year y converted to tons/day, 0.00747 tonnes/HH/day |
| Up,y | Cumulative usage rate for technologies in project scenario j during year y, based on cumulative installation rate and drop-off rate, 90% |
| Pb,y | Specific fuel consumption for an individual technology in baseline scenario b during year y converted to tons/day, 0.0131 |

Hence

Bp,y 		= 775,100 * ((0.00747 * 90.00%) + (0.0131 * (1 – 90.00%))) = 6,226 tonnes

PEp,y (ICS) 	= 6,226 * ((0.78 * 112) + 9.46) * 0.0156 = 9,404 tCO2e

The overall GHG reductions achieved by the project activity are then calculated as follows:

ERy = ∑BEb,y - ∑PEp,y - ∑LEp,y

Where:

ERy	Emission reduction for total project activity in year y (tCO2e/yr)

PEp,y	 Emissions for project scenario p during the year y in tCO2e

BEp,y	 Baseline emissions for baseline scenario b in year y (tCO2e/yr)

LEp,y	 Leakage for project scenario p in year y (tCO2e/yr)

ERy= 15,336-9,404 = 5,931

The other SDGs impacts of this VPA (SDG 1, SDG 3, SDG 5, SDG 7, SDG 8, SDG 12 and SDG 15) will be estimated as below.

SDG 1

| SDG 1-No Poverty |  |  |
| --- | --- | --- |
| HHSbaseline | % users reporting money saving due reduced fuel consumption in baseline | 0% |
| HHSproject | % users reporting money saving due reduced fuel consumption in in project | 90% |
| Percentage of users reporting money savings due to reduction in purchased fuel consumption in project | (HHSproject-HHSbaseline) | 90% |

SDG 3

| SDG 3-Good health and well being |  |  |
| --- | --- | --- |
| HHsmokebaseline | % users reporting reduction in smoke after shifting to ICS in baseline | 0 |
| HHsmokeproject | % users reporting reduction in smoke after shifting to ICS in project | 90% |
| Percentage of households reporting reduction in smoke/PM after shifting to ICS in project | (HHsmokeproject-HHsmokebaseline) | 90% |

SDG 5

| SDG 5-Gender Equality |  |  |
| --- | --- | --- |
| HHtimebaseline | % users reporting time saving due to reduced collected fuel consumption / cooking time in baseline | 0% |
| HHtimeproject | % users reporting time saving due to reduced collected fuel consumption / cooking time in project | 90% |
| Percentage of users reporting time saving due to reduction in collected fuel consumption / cooking time in project | (HHtimeproject-HHtimebaseline) | 90% |

SDG 7

| SDG 7-Affordable and Clean Energy |  |  |
| --- | --- | --- |
| HHcleanbaseline | Number of household having access to affordable and clean energy functional / operational ICS units under Baseline | 0 |
| HHcleanproject | Number of household having access to affordable and clean energy functional / operational ICS units under Project | 9,356 |
| Percentage of ICS distributed and operating under the project as an indicator of providing clean technology (relative to baseline stoves). | (HHcleanproject-HHcleanbaseline) | 9,356 |

SDG 8

| SDG 8-Decent work and economic growth |  |  |
| --- | --- | --- |
| EGbaseline | Generation of employment in baseline scenario | 0 |
| EGproject | Generation of employment in project scenario | 54 |
| Total number of jobs created | (EGproject -EGbaseline) | 54 |

SDG 12

| SDG 12 Responsible consumption and Production |  |  |
| --- | --- | --- |
| FCbaseline | Quantity of biomass saved per tonnes per year in the baseline | 4.78 |
| FCproject | Quantity of biomass saved per tonnes year in the project | 2.73 |
| Average percentage of fuel savings reported by users in the project | (1-FCbaseline/FCproject) | 42.98% |

SDG 15:

| SDG 15 Life on land |  |  |
| --- | --- | --- |
| FCHHbaseline | Average Fuel consumption per HH in baseline (tonnes eq. fuelwood/HH/day) | 0.0131 |
| FCHHproject | Average Fuel consumption per HH in project (tonnes eq. fuelwood/HH/day) | 0.0075 |
| Fuelwood equivalent savings reported by users in the project | (FCHHbaseline-FCHHproject) * Number of operational ICS ((tonnes eq. fuelwood/day)) | 52.67 |

##### E.3. Calculation of leakage

>>

Leakage is not applicable for the current monitoring period.

Emission Reductions

ERy = ΣBE,b,y - ΣPE,p,y - ΣLEp,y

Where:

| ERy | Emission reduction for total project activity in year y (tCO2e/yr) |
| --- | --- |
| PEp,y | Emissions for project scenario p during the year y in tCO2e, |
| BEp,y | Baseline emissions for baseline scenario b in year y (tCO2e/yr), |
| LEp,y | Leakage for project scenario p in year y (tCO2e/yr), 0 |

Hence

ERy for ICS = 15,336 – 9,404 = 5,931 tCO2e (round down value)

##### E.4. Calculation of net benefits or direct calculation for each SDG Impact:

| SDG | SDG Impact | Baseline estimate | Project estimate | Net benefit |
| --- | --- | --- | --- | --- |
| 13 | Emission Reductions | 0 tCO2e | 5,931 tCO2e | 5,931 tCO2e |
| 1 | Percentage of users reporting money savings due to reduction in purchased fuel consumption in project | 0% | 90% | 90% |
| 3 | Percentage of users reporting reduction in smoke/PM after shifting to ICS in project | 0% | 90% | 90% |
| 5 | Percentage of users reporting time saving due to reduction in collected fuel consumption/cooking time | 0% | 90% | 90% |
| 7 | Number of households with operational stoves | 0 | 9356 | 9356 |
| 8 | Total number of jobs created | 0 | 54 | 54 |
| 12 | Average percentage fuel savings reported by users in the project | 4.78 | 2.73 | 42.98 % |
| 15 | Fuelwood eq savings reported by users in the project | 0.0131 | 0.0075 tonnes-equivalent fuelwood/day | 0.0056 tonnes-equivalent fuelwood/day |

##### E.5. Comparison of actual SDG Impacts with estimates in approved PDD:

| SDG | Values estimated in ex-ante calculation of approved PDD for this monitoring period | Actual values achieved during this monitoring period |
| --- | --- | --- |
| 13 | 18,905 tCo2e | 5,931 tCo2e |
| 1 | 100% | 90% |
| 3 | 100% | 90% |
| 5 | 95% | 90% |
| 7 | 5,959 | 9356 |
| 8 | 40 | 54 |
| 12 | 55% | 42.98% |
| 15 | 52.14 | 52.67 |

E.5.1. Explanation of calculation of value estimated ex-ante calculation of approved PDD for this monitoring period

>>

Based on ex-ante tables in the PDD

Baseline Emissions

BEb,y = Bb,y*((fNRB,y*EFb,fuel,CO2)+ EFb,fuel,nonCO2)*NCVb,fuel

Where:

| BEb,y | Emissions for baseline scenario b during the year y in tCO2e |
| --- | --- |
| Bb,y | Quantity of fuel consumed in baseline scenario b during year y, in tons |
| f NRB,y | Fraction of biomass used in year y for baseline scenario b that can be established as non-renewable biomass |
| NCVb,fuel | Net calorific value of the fuel that is substituted or reduced (IPCC default for wood fuel, 0.0156 TJ/ton) |
| EFb,fuel,CO2 | CO2 emission factor of the fuel that is substituted or reduced. 112 tCO2/TJ for Wood/Wood Waste, or the IPCC default value of other relevant fuel |
| EFb,fuel,nonCO2 | Non-CO2 emission factor of the fuel that is reduced |

Bb,y = Np,y * Pb,y

Where:

| Bb,y | Quantity of fuel consumed in baseline scenario b during year y, in tons |
| --- | --- |
| Pb,y | Specific fuel consumption for an individual technology in baseline scenario b during year y converted to tons/day |
| Np,y | Project technology-days in the project database for project scenario p through year y |

Project Emissions

PEp,y = Bp,y * ((fNRB,y * EFp,fuel,CO2) + EFp,fuel,nonCO2) * NCVp,fuel

Bp,y = Np,y * ((Pp,y * Up,y) + (Pb,y * (1 – Up,y)))

Where:

| PEp,y | Emissions for project scenario p during the year y in tCO2e |
| --- | --- |
| Bp,y | Quantity of fuel consumed in project scenario p during year y, in tons |
| f NRB,y | Fraction of biomass used in year y that can be established as non-renewable biomass |
| NCVp,fuel | Net calorific value of the project fuel that is substituted or reduced (IPCC default for wood fuel, 0.0156 TJ/ton) |
| EFp,fuel,CO2 | CO2 emission factor of the fuel that is substituted or reduced. 112 tCO2/TJ for Wood/Wood Waste, or the IPCC default value of other relevant fuel |
| EFp,fuel,nonCO2 | Non-CO2 emission factor of the fuel that is reduced |
| Np,y | Project technology-days in the project database for project scenario p through year y |
| Pp,y | Specific fuel consumption for an individual technology in project scenario p during year y converted to tons/day |
| Up,y | Cumulative usage rate for technologies in project scenario j during year y, based on cumulative installation rate and drop-off rate |
| Pb,y | Specific fuel consumption for an individual technology in baseline scenario b during year y converted to tons/day |

Emission Reductions

The overall GHG reductions achieved by the project activity are then calculated as follows:

ERy = ∑BEb,y - ∑PEp,y - ∑LEp,y

Where:

| ERy | Emission reduction for total project activity in year y (tCO2e/yr) |
| --- | --- |
| PEp,y | Emissions for project scenario p during the year y in tCO2e |
| BEp,y | Baseline emissions for baseline scenario b in year y (tCO2e/yr) |
| LEp,y | Leakage for project scenario p in year y (tCO2e/yr) |

| Year | Total | Ex-ante ER (pro-rated) |
| --- | --- | --- |
| Year-1 | 28,872 | 18,905 |

##### E.6. Remarks on increase in achieved SDG Impacts from estimated value in approved PDD

>>

Not Applicable

#### SECTION F. SAFEGUARDS REPORTING

>>

A completed Safeguarding Principles Assessment is in Appendix 1 of VPA DD, no ongoing monitoring is applicable to this monitoring period.

#### SECTION G. STAKEHOLDER INPUTS AND LEGAL DISPUTES

##### G.1. List all Inputs and Grievances which have been received via the Continuous Input and Grievance Mechanism together with their respective responses/mitigations.

>>

An Expression book has been placed at office of community carbon in Mozambique. Stakeholders are free to voice their concerns via the Grievance Expression Book. By maintaining feedback book at the local office, it is ensured that stakeholders that don’t have access to electronic media for expressing concerns / grievances are also able to share their concerns / feedback. Additionally, the end users always have an option to revert to the salesperson (representative of distribution/retail partners etc.) in case of any feedback / complaints with the product post distribution.

But no inputs and grievances received during the monitoring period.

##### G.2. Report on any stakeholder mitigations that were agreed to be monitored.

>>

Not applicable

##### G.3. Provide details of any legal contest that has arisen with the project during the monitoring period

>>

No legal contest during the monitoring period.

Revision History

| Version | Date | Remarks |
| --- | --- | --- |
| 1.1 | 14 October 2020 | Hyperlinked section summary to enable quick access to key sections Improved clarity on Key Project Information Section for POA monitoring Forward action request section Improved Clarity on SDG contribution/SDG Impact term used throughout Clarity on safeguard reporting Clarity on design changes Leakage section added for VER/CER projects Addition of Comparison of monitored parameters with last monitoring period Provision of an accompanying Guide to help the user understand detailed rules and requirements |
| 1.0 | 10 July 2017 | Initial adoption |

The start date of the project (01/01/2022) is based on the initial pilot distribution, whereas the start of the monitoring period is based on the actual full-scale distribution and is in line with the start of the crediting period start date mentioned in the VPA-DD.

The start date of the project (01/01/2022) is based on the initial pilot distribution, whereas the start of the monitoring period is based on the actual full-scale distribution and is in line with the start of the crediting period start date mentioned in the VPA-DD.

The charcoal fuel consumption will be equivalently calculated in terms of fuel-wood using 1:6 ratio allowed by methodology.

Please refer to the ER sheet for further details

Refer to the ER sheet tab “ER Calculation” for detailed calculations

Refer to the ER sheet tab “ER Calculations” for detailed calculation

Refer to ER sheet tab “SDGs” for detailed calculation

Refer to ER sheet tab “SDGs” for detailed calculation

Refer to ER sheet tab “SDGs” for detailed calculation

Refer to ER sheet tab “SDGs” for detailed calculation

Refer to ER sheet tab “SDGs” for detailed calculation

Refer to ER sheet tab “SDGs” for detailed calculation

Refer to ER sheet tab “SDGs” for detailed calculation

- Whenever emission reductions are capped, both the original and capped values used for calculations must be transparently reported.  Use brackets to denote original values.