Key Project Information & project Design Document (PDD)

###### PUBLICATION DATE  14.10.2020
VERSION  v. 1.2 
RELATED SUPPORT

###### - TEMPLATE GUIDE Key Project Information & Project Design Document v.1.2

This document contains the following Sections

Key Project Information

SECTION A – Description of project

SECTION B - Application of approved Gold Standard Methodology (ies) and/or    demonstration of SDG Contributions

SECTION C – Duration and crediting period

SECTION D – Summary of Safeguarding Principles and Gender Sensitive Assessment

SECTION E – Outcome of Stakeholder Consultations

Appendix 1 – Safeguarding Principles Assessment (mandatory)

Appendix 2 - Contact information of Project participants (mandatory)

Appendix 3 - LUF Additional Information (project specific)

Appendix 3 - Summary of Approved Design Changes (project specific)

### KEY PROJECT INFORMATION

| GS ID of Project | GS11539 |
| --- | --- |
| Title of Project | Bundling of Household biogas plants for thermal energy applications |
| Time of First Submission Date | 27/01/2022 |
| Date of Design Certification | N/A |
| Version number of the PDD | 3.0 |
| Completion date of version | 08/06/2022 |
| Project Developer | Greneity Infocom Service Private Limited |
| Project Representative | Greneity Infocom Service Private Limited |
| Project Participants and any communities involved | Greneity Infocom Service Private Limited Samagra Vikas Mission |
| Host Country (ies) | India |
| Activity Requirements applied | Community Services Activities Renewable Energy Activities Land Use and Forestry Activities/Risks & Capacities N/A |
| Scale of the project activity | Micro scale Small Scale Large Scale |
| Other Requirements applied | N/A |
| Methodology (ies) applied and version number | AMS-I.E.: Switch from non-renewable biomass for thermal applications by the user -Version 12.0 |
| Product Requirements applied | GHG Emissions Reduction & Sequestration Renewable Energy Label N/A |
| Project Cycle: | Regular Retroactive |

Table 1 – Estimated Sustainable Development Contributions

| ## Sustainable Development Goals Targeted | ## SDG Impact (defined in B.6.) | ## Estimated Annual Average | ## Units or Products |
| --- | --- | --- | --- |
| ## 13 | ## Emission reductions | ## 36,002 | ## tCO2e |
| ## 3 | ## Good health & well being | ## 9,237 users are expected to have positive impact on health and illness due to the project. | ## Number of affected users |
| ## 7 | ## Affordable and clean energy | ## 9,237 users are expected to access clean and affordable energy. | ## Number of affected users |

#### SECTION A. DESCRIPTION OF PROJECT

##### A.1 Purpose and general description of project

## The project activity is targeting to bundle 9,237 household biogas plants located in rural parts of Chhattisgarh and Bihar states of India commissioned from March 2021 until March 2022.

## The project biogas plants replace commonly used inefficient wood fired mud stoves thereby replacing firewood consumption.

## The household biogas plants utilize cattle dung in the digester to produce biogas which is burnt in gas stoves for household cooking’s. This leads to reduction of greenhouse gas emissions by displacing conventionally used non-renewable biomass with renewable biogas. In addition, the hygienic conditions in the rural areas will be improved by an appropriate disposal of waste. Further, residue from the bio digesters can be used as organic fertilizer and will improve soil fertility in rural areas.

## Project activity will contribute towards sustainable development by replacing firewood with biogas generated from the biodigesters.

Pre project Scenario:

Household survey was conducted to assess the baseline fuel and quantity used. As per the Survey, firewood is found to be the primary fuel for domestic cooking needs in both the states. Usage of inefficient firewood leads to indoor air pollution and land use patterns have been showing a decrease in forest land cover and increase in degraded land. Increasing pressure from human and livestock population and indiscriminate and illegal exploitation of forest resources are among factors that have led to further intensification of the problem. A trend of forests turning into open scrubs has been observed. Degradation of forest lands has exacerbated the already existing problem of desertification. There is a need to maintain adequate forest cover in the state to mitigate climate change effects.

Project Scenario:

Project activity is targeting to bundle 9,237 household biogas plants installed in rural areas of Chhattisgarh and Bihar.

Expected Implementation Schedule

|  | 2021 | 2022 |
| --- | --- | --- |
| Chhattisgarh | 5448 | 1052 |
| Bihar | 2418 | 319 |

Eligibility of the project under Gold Standard

>> The project is eligibility under GS4GG as the project activity involves dissemination of household biogas which is eligible under Gold Standard pursuant to clause 3.1.1 of the GS4GG Principles and Requirements document. The eligibility compliance under GS4GG is detailed as follows;

| Eligibility Criteria Category | Eligibility criterion - Required condition | Justification |
| --- | --- | --- |
| 1. Types of Project | Eligible projects shall include physical action/implementation on the ground. Pre-identified eligible project types are identified in the Eligibility Principles and Requirements section. | The project is already implemented since its start date. As per section 3.1.1 (a) of GG4GG Community Services Activity Requirements ver.1.2,” Renewable energy types such as solar (photovoltaic and solar thermal electricity generation), tidal/wave, wind, hydropower, geothermal, waste to energy and renewable biomass that are connected to mini grid3 or off grid solutions for targeted users and/or applications.” Since, project activity includes utilization of cow dung as feed to bio-digesters along with other organic waste to generate biogas Hence, it meets the criterion. Thus, the project technology/measure falls under GG4GG Community Services Activity Requirements. Section 4.1.3 of the GS4GG Principles and Requirements document states following for automatic eligibility for a project “A Project type is automatically eligible for Gold Standard Certification if there are Gold Standard approved Activity Requirements and/or Impact Quantification Methodologies associated with it or it’s referenced in the Gold Standard Product Requirements.” The Gold Standard has published Community Services Activity Requirements; therefore, the project activity falls under the list of Pre-identified eligible project and is automatically eligible for Gold standard certification. |
| 2. Location of Project | Projects may be located in any part of the world. | Location of the project is the states of Chhattisgarh and Bihar, India. |
| 3. Project Area, Project Boundary and Scale | The Project Area and Project Boundary shall be defined. Projects may be developed at any scale although certain rules, requirements and limitations may apply under specific Activity Requirements, Impact Quantification Methodologies and Products Requirements. In order to avoid double counting the Project shall not be included in any other voluntary or compliance standards programme unless approved by Gold Standard (for or compliance standard programme of a similar nature, the Project shall demonstrate that there is no double counting of impacts at design and performance certification (for example use of similar technology or practices through which the potential arises for double counting or misestimation of impacts amongst projects) example through dual certification). Also, if the Project Area overlaps with that of another Gold Standard or other voluntary | The boundary for the project activity in terms of a geographical area is defined as the territorial boundary of the states of Chhattisgarh and Bihar, India. The project falls under waste handling and disposal with emission reductions 36,002 tCO2 with installed energy output of 20.80 MWth (Below the threshold of 45 MWth). Hence, the project falls under small scale projects. To avoid inclusion of any bio-digester which is a part of another registered carbon project/ programme, all the 9,237 bio-digesters under this project activity have a unique ID number to uniquely identify the bio-digesters avoiding any double counting and trace its user, later during monitoring and verification. |
| 4. Host Country Requirements | Projects shall be in compliance with applicable Host Country’s legal, environmental, ecological and social regulations. | The project activity is in compliance with the host country’s legal, environmental, ecological & social regulation. |
| 5. Contact Details | As part of the Project Documentation the Project Developer shall provide (i) name and (ii) contact details of all Project Participants; AND in case of an organisation (iii) the legal registration details and (iv) documentation by the governing jurisdiction that proves that the entity is in good standing (defined as being a legal or other appropriate entity registered in or allowed to operate within the required jurisdiction and with no evidence of insolvency or legal/criminal notices placed against it or any of its Directors). Gold Standard retains the right (at its own discretion) to refuse use of the Standard where reputational concerns are highlighted. | Name and Contact details of organisation is given in the Appendix 1. |
| 6. Legal Ownership | Full and uncontested legal ownership of any Products that are generated under Gold Standard Certification, (for example carbon credits) shall be demonstrated. Where such ownership is transferred from project beneficiaries this must be demonstrated transparently and with full, prior and informed consent (FPIC). Note that for certain Project types there is a requirement for full and uncontested legal land title/tenure to be demonstrated. These are contained within specific Activity or Product Requirements. All projects shall immediately report to Gold Standard any land title/tenure disputes arising. | The project activity includes installation of household biogas plants in the rural areas of Chhattisgarh and Bihar. The owners of the biodigester have transferred their all legal rights, interests, credits, entitlements, benefits or allowances arising from or in connection with any greenhouse gas emissions reductions arising from the operation of the digester (Emission Reduction) in the lieu of free operation and maintenance of the plant to Samagra Vikas Mission. Same is mentioned in the Commissioning certificates co-signed by the respective plant owners. Also, Samagra Vikas Mission has signed agreement with Greneity Infocom Service Pvt Ltd to take all necessary action required for registering the project under suitable Voluntary Standard along with complete assistance during operation and maintenance of the biogas plants included in the project activity. Also feedback has been collected during the physical local stakeholder consultation meeting as well as SFR round for the project activity. |
| 7. Other Rights | As well as legal title and ownership, the Project Developer shall also demonstrate where required uncontested legal rights and/or permissions concerning changes in use of other resources required to service the Project (for example, access rights, water rights etc.). Any known disputes or contested rights must be declared immediately to Gold Standard by the Project Developer and resolved prior to further project implementation in affected areas. | Not Applicable. |
| 8. Official Development Assistance (ODA) Declaration | All Project Developers applying for project activities located in a country named by the OECD Development Assistance Committee’s ODA recipient list and seeking Gold Standard Certification for carbon credits shall declare the Official Development Assistance (ODA) support. The Project Developer shall follow the GHG Emissions Reduction & Sequestration Product Requirements and submit the declaration at the time of Design Certification. | No ODA is involved in the Project activity. A declaration is being submitted. |

Eligibility under Gold Standard Community Services Activity (CSA) Requirements

>>

As per section 3.1.1 of GS4GG Principles & Requirements, Eligibility criteria is defined below:

| Eligibility Criteria Category | Eligibility criterion - Required condition | Justification |
| --- | --- | --- |
| 1. Eligible Project Types | All CSA Projects shall lead to climate change mitigation and/or adaptation by providing or improving access to services/resources at the household or community or institution level. Eligible services include electricity and energy, water and sanitation, waste management, housing, etc. | The project is already implemented since its start date. As per section 3.1.1 (a) of GG4GG Community Services Activity Requirements ver.1.2,” Renewable energy types such as solar (photovoltaic and solar thermal electricity generation), tidal/wave, wind, hydropower, geothermal, waste to energy and renewable biomass that are connected to mini grid3 or off grid solutions for targeted users and/or applications.” Since, project activity includes utilization of cow dung as feed to bio-digesters along with other organic waste to generate biogas Hence, it meets the criterion. |
| 2. GENERAL ELIGIBILITY CRITERIA - Type of project | (a) Renewable energy: Renewable energy types such as solar (photovoltaic and solar thermal electricity generation), tidal/wave, wind, hydropower, geothermal, waste to energy and renewable biomass that are connected to mini grid3 or off grid solutions for targeted users and/or applications. | As per section 3.1.1 (a) of GG4GG Community Services Activity Requirements ver.1.2,” Renewable energy types such as solar (photovoltaic and solar thermal electricity generation), tidal/wave, wind, hydropower, geothermal, waste to energy and renewable biomass that are connected to mini grid3 or off grid solutions for targeted users and/or applications.” Since, project activity includes utilization of cow dung as feed to bio-digesters along with other organic waste to generate biogas Hence, it meets the criterion. |
| 3. GENERAL ELIGIBILITY CRITERIA – Project Area, Boundary and scale | Project Area and Boundary shall be defined in line with the applicable Impact Quantification Methodologies and Product Requirements. | The boundary for the project activity in terms of a geographical area is defined as the territorial boundary of the states of Chhattisgarh and Bihar, India. The project falls under waste handling and disposal with emission reductions 36,002 tCO2 with installed energy output of 20.80 MWth (Below the threshold of 45 MWth). Hence, the project falls under small scale projects. To avoid inclusion of any bio-digester which is a part of another registered carbon project/ programme, all the 9237 bio-digesters under this project activity have a unique ID number to uniquely identify the bio-digesters avoiding any double counting and trace its user, later during monitoring and verification. |
| 4. GENERAL ELIGIBILITY CRITERIA – Legal Ownership | (a) Projects involving the distribution of a large number of devices for services such as heating, cooking, lighting, electricity generation, water treatment technology such as water filter, etc. shall provide a clear description of the ownership of the Products that are generated under Gold Standard Certification all along the investment chain. In line with the FPIC requirement, the proofs that end-users are aware of and willing to give up their rights on Products shall be provided. (b) The transfer of Product ownership shall be discussed during local stakeholder consultations for projects. | The project activity includes installation of household biogas plants in the rural areas of Chhattisgarh and Bihar. The owners of the bio-digester have transferred their all legal rights, interests, credits, entitlements, benefits or allowances arising from or in connection with any greenhouse gas emissions reductions arising from the operation of the digester (Emission Reduction)in the lieu of free operation and maintenance of the plant to Samagra Vikas Mission. Same is mentioned in the Commissioning certificates co-signed by the respective plant owners. Also, Samagra Vikas Mission has signed agreement with Greneity Infocom Service Pvt Ltd to take all necessary action required for registering the project under suitable Voluntary Standard along with complete assistance during operation and maintenance of the biogas plants included in the project activity. Also feedback has been collected during the physical local stakeholder consultation meeting as well as SFR round for the project activity. |

PP confirms that the host country, region, locality or state does not have an emission reduction cap enforced OR have the possibility to trade emissions that include the scope of the proposed project activity.

Legal ownership of products generated by the project and legal rights to alter use of resources required to service the project

>> The project activity is the household level biogas plants and the owner of the technology is the particular household using biogas plants. The owners of a digester signed an agreement  with Samagra Vikas Mission by transferring all legal rights, interests, credits, entitlements, benefits or allowances arising from or in connection with any greenhouse gas emissions reductions arising from the operation of the digester (Emission Reduction), and agrees to take all necessary action required to ensure the transfer of those Emission Reductions to the Samagra Vikas Mission or its nominee, including executing any relevant documents.

##### A.2 Location of project

A.2.1.	Host Country

>> India

### A.2.2. Region/State/Province etc.

>> State: Chhattisgarh and Bihar

Geo coordinates for Bihar is 25° 36' 45.6372'' N and 85° 9' 31.9500'' E and Chhattisgarh is  21° 17' 42.4752'' N and 81° 49' 41.6352'' E.

##### A.3 Technologies and/or measures

>>

The existing traditional stoves used in the baseline are simple structures made from clay or having stone or metal tripods with poor combustion air supply or flue gas ventilation system i.e. without a grate or a chimney. These stoves use non-renewable biomass (firewood). The target group of the project are households with at least one head of cattle (generally cows or buffalos) who currently use non-renewable woody biomass (firewood) for cooking purposes. The project aims to utilize the methane produced from household biogas digesters to replace the current non-renewable energy sources that is used by households.

The project activity involves bundling of household bio-digesters with Deenbandhu model technology installed in the state of Chhattisgarh and Bihar. The bio-digester models of various sizes (2m3,3m3). The major feed cattle dung is mixed with water and fed into the plant through the inlet chamber of the plant. This waste is converted into biogas with the help of a special type of anaerobic bacteria. The digested material, which comes out of the plant, is enriched manure.

Deenbandhu Model:

The major feed cow dung and other organic waste material is mixed with water and fed into the plant through the inlet chamber of the plant. This waste is converted into biogas with the help of a special type of anaerobic bacteria. The digested material, which comes out of the plant, is enriched manure.

The main feature of a Deenbandhu biogas plant is the fixed underground digester chamber, constructed with a layer of bricks and an additional layer of cement mortar forming the roof above. Connected to the underground chamber is an inlet tank (labelled on diagram as “Mixing Tank”), through which manure is fed into the plant. The manure then ferments separating the slurry from the methane gas which rises and collects at the top of the digester tank, and is released through the gas outlet pipe. The slurry passes into the outlet tank where it is ejected from the plant and can be used as fertilizer on the field

Figure1: Deenbandhu type Biogas plant

## The cattle dung otherwise would have been left to decay in open which leads to methane emissions and by utilizing the same for generation of biogas leads to capture of methane and utilize as fuel. The biogas thus generated replaces firewood that otherwise would have been used for cooking and heating purposes. The project is expected to reduce 36, 002 tCO2 per annum. The projects operational life time is expected to be around 20 years.

The project contributes directly in achieving the SDG#3 & 7 in addition to SDG#13 as required by Principle-1 of GS4GG. The project will have following benefits:

| - Environmental Benefits: Reduction in firewood consumption and emission of greenhouse gases, forest and biodiversity conservation (SDG#13). - Health Benefits: Sufficiently enhance indoor air quality thereby improving health of family members and reducing incidences of smoke and fire related injuries (SDG#3). - Social Benefits: The project will provide affordable and clean fuel compared to baseline scenario (SDG #7) |  |
| --- | --- |

##### A.4 Scale of the project

The project falls under waste handling and disposal with emission reductions of 36, 002  tCO2 per annum with installed energy output of 20.80  MWthermal (below the threshold of 45 MWthermal). Hence, the project falls under small scale projects.

##### A.5 Funding sources of project

No public funding from parties included in Annex I to the UNFCCC, is available to the project. No Official Development Assistance involves in the project activity.

SECTION B. APPLICATION OF APPROVED GOLD STANDARD METHODOLOGY (IES) AND/OR DEMONSTRATION OF SDG CONTRIBUTIONS

##### B.1. Reference of approved methodology (ies)

Methodology	: AMS-I.E - Switch from non-renewable biomass for thermal applications by the user

Version	: Version 12

Reference:

https://cdm.unfccc.int/methodologies/DB/BLVN9ULDP1FRUVS2LYWW6WPYN9W78E

##### B.2. Applicability of methodology (ies)

>>

AMS-I. E. "Switch from Non-Renewable Biomass for Thermal Applications by the User" Version 12.0 is applicable to the proposed project because the project meets all applicability criteria. The applicability of the methodology is outlined as below:

| Criteria AMS-I.E. | Applicability |
| --- | --- |
| Project participants4 are able to show that non-renewable biomass has been used in the project region since 31 December 1989, using survey methods or referring to published literature, official reports or statistics. | Survey was conducted to check since when villagers were using firewood. It is evident from the survey that all villagers have been using firewood for more than 35 years hence meets the criteria. |
| In the case that technologies using renewable biomass are used under the project activity, this methodology is applicable where all emissions related to processing of biomass are fully accounted for and biomass is sourced from biomass residues and/or a dedicated plantation of the CDM project activity, meeting the following conditions: (a) For projects that use biomass residues, prior to the implementation of the project activity, the biomass residues have not been collected and used but been left for decay and would, in the absence of the project activity, continue to be left for decay; and (b) For projects that use biomass residues from a production process (e.g. production of sugar or wood panel boards), the implementation of the project does not result in an increase of the processing capacity of raw input (e.g. sugar, rice, logs, etc.) or in other substantial changes (e.g. product change) in this process; and (c) The biomass used by the project facility is not stored for more than one year; and (d) In the case biomass from dedicated plantations are used, the applicability conditions of TOOL16 “Project and leakage emissions from biomass” are satisfied. | Biomass is not used in the project activity. Hence, the conditions are not applicable. |
| For electric cook stoves with integrated renewable energy device or with grid connected renewable energy system employing net metering, project participants shall demonstrate that, on an annual basis, at least 80% of the electricity generated is consumed by the electric cook stoves (i.e. 20% or less of electricity is consumed by other loads connected). | The project does not involve any electric cook stove and hence the condition is not applicable for the project. |
| For electric cook stoves, in all cases under paragraph 2(d) above where back-up diesel generators are used, this methodology is only applicable when no more than 1% of total electricity supply occurs from back up diesel generators on an annual basis. | The project does not involve any electric cook stove and hence the condition is not applicable for the project. |
| Under this methodology, emission reductions cannot be claimed only due to fuel-switch aspect and proposed project activities shall introduce new renewable energy based technologies, i.e. technology switch is also involved. | The project activity involves technology switch from conventional firewood based cooking system to renewable biogas based cooking system. Hence, the criteria are met. |
| Project participants shall describe in the PDD/PoA-DD the proposed method for distribution of project devices and how the double counting of emission reductions has been addressed, for example, using methods such as unique identifications of product and end-user locations (e.g. programme logo), to prevent double counting of emission reductions from the project devices (e.g. between end users, distributors and producers of stoves, producers of renewable energy, producers of processed renewable biomass). | Each of the bio-digesters shall be allocated a unique id against each end users. End user and project implementer shall have an agreement to avoid any double counting. |
| For project activities introducing bio ethanol cook stove, project participants shall demonstrate that the bioethanol cook stove are designed, constructed and operated to the requirements (e.g. with regard to safety) of a relevant national or local standard or comparable literature. Latest guidelines issued by a relevant national authority or an international organization may also be used. | The Project is using biogas digesters and bio- ethanol is not used and safety requirement does not arise here. Hence, the criteria is met. |

The section 3.1.1 of GS4GG Principles & Requirements states that ‘A Project type is automatically eligible for Gold Standard Certification if there are Gold Standard published Activity Requirements and/or Gold Standard Approved Methodologies associated with it or as referenced in Gold Standard Product Requirements’. The GS has published Community Services Activity Requirement (Version 1.2, October 2019) therefore the proposed project falls under the automatic eligibility list of projects. The projects lead to climate change mitigation by providing access to resources (biogas) to households. The project falls under ‘Renewable energy” type- Waste management and handling: Management of animal waste (cattle dung) to deliver biogas.

##### B.3. Project boundary

>> The project boundary is the physical, geographical site of the use of biomass or the renewable energy. Therefore, the project boundary encompasses the sum of all the 9, 237 physical geographical sites of all individual biogas plants (digester system, pipe leading to the stove and the stove itself) realized by the project activity. However, the baseline emissions from methane avoidance have been excluded to be conservative.

Cattle dung Anaerobic Bio-Digester Biogas stove Organic Manure to be used in field as fertilizer

## For the purpose of GHG mitigation/sequestration following table shall be completed (delete if not required)

| Source |  | GHGs | Included? | Justification/Explanation |
| --- | --- | --- | --- | --- |
| Baseline scenario | Firewood | CO2 | Yes | The major source of emissions in the baseline due to burning of firewood |
|  |  | CH4 | No | Excluded for simplification, this is conservative. |
|  |  | N2O | No | Not applicable for the project activity |
| Project scenario | Fire wood | CO2 | Project emissions form continued use of fire-wood shall be accounted. |  |
| Leakage scenario | The use/diversion of non-renewable woody biomass saved under the project activity by non-project households/users | CO2 | Yes | Default option as per methodology to be applied. |
|  |  | CH4 | No | Not applicable for the project activity |
|  |  | N2O | No | Not applicable for the project activity |

##### B.4. Establishment and description of baseline scenario

>>As per AMS I.E- Switch from non-renewable biomass for thermal applications by the user, Version 12 ‘the baseline scenario would be the use of fossil fuels for meeting similar thermal energy needs’.

A project specific baseline survey reveals the alternative option in absence of biogas use is firewood-based cooking practice and therefore, firewood based thermal application is considered as baseline scenario.

The project activity involves the installation of anaerobic biodigesters for the production of biogas which replaces non-renewable biomass, used as a fuel for household cooking purposes. A baseline survey has been conducted on January 2021 to estimate the average firewood consumption prior to project activity at user point following sampling standard as per UNFCCC ‘Sampling and surveys for CDM project activities and programmes of activities’ version 08 and guideline ‘Sampling and surveys for CDM project activities and programmes of activities’ version 04.

The baseline survey was performed by a third party covering targeted population of Bihar and Chhattisgarh. A 90% confidence interval and 10% error margin applied for sample calculation.

To understand the cooking method pattern especially with focus on targeted population in the rural areas of Bihar and Chhattisgarh, to establish a baseline with respect to the fuel used for cooking; and to facilitate a clean cooking programme for the states of Bihar and Chhattisgarh. As part of this objective, a primary survey of households was carried out to gauge the extent of the use of non-renewable biomass for cooking in households. The primary survey covers the targeted rural households in the states of Bihar and Chhattisgarh, India.

To understand the level of detail and data points required, a conclusive research methodology, through structured face-to-face interviews, was used to arrive at statistically-valid estimates for the relevant parameters. In consonance with the objective of the study, the following methodology was adopted for carrying out the assignment.

• Preparation of detailed Questionnaire for household survey

• Mobilization of appropriate teams for village and household survey

• Orientation of the team members about the survey objectives

• Collection of primary and secondary data.

• Compilation and analysis of the collected data

Sampling Plan:

To establish a statistically representative sample size, from the total rural targeted households in Bihar and Chhattisgarh, Guidelines for sampling and surveys for CDM project activities and programmes of activities (Ver04.0, CDM-EB67-A06-GUID) issued by UNFCCC, is used. As per the guidelines provided by UNFCCC, for the representative sampling methods, sample size shall be chosen for a 90/10 precision (90% confidence interval and 10% margin of error) for parameter values used to determine emission reductions. 50% of the proportion was considered.

The project activity is envisaging to implement in the states of Bihar and Chhattisgarh, India. The sample population has homogeneous characteristics which are as follows:

o Basic functioning of the bio digesters is same irrespective of their sizes.

o All of the bio digesters are used by villagers

o Main usage of the bio digesters is to suffice domestic needs

Hence, a simple random sampling is performed. However, proportionate allocation of samples is done against the targeted population of each district. The following equation is used for arriving the sample size.

Where:

n= Sample size

N = Total number of biogas system of type i installed under the project

p = expected proportion(50%)

1.645 = represents the 90% confidence required

0.1 = represents the 10% relative precision

Sample size arrived is 263 , however, on a conservative note, the project developer, has decided to go for 300 household sampling. The total sample size decided is 300 households.

Baseline assessment:

This study examines the patterns of domestic energy consumption of targeted households in 2 states of, India over the period of past three and half decades. Potential Fuels for cooking in the survey area are as follows;

Kerosene: Dependence on kerosene as a primary fuel for cooking or for lighting has decreased in India, and it is used much less for cooking than for lighting. It is evident from the survey, that none of the households utilize Kerosene as the primarily source of energy for cooking owing to is higher price and limited quantity is available is Fair price shop which is not sufficient to meet their energy demands. Prices in the open market shop is very high and hence making it out of the reach of the local population.

LPG: During this survey it was found that Fuel wood has remained the principal component of rural domestic energy. From the survey conducted it is very evident that, In India, the transition to modern fuel is slow and still a large number of households are using traditional fuels for cooking and lighting. So, it is important to understand the household level behavior towards use of different types of traditional and modern fuel.  More than 80 per cent of the households still bank upon the traditional energy sources for cooking i.e. firewood With the slow rate of growth of LPG, the Government of India launched the Pradhan Mantri Ujjwala Yojana (PMUY) in May 2016 to facilitate underprivileged households' access to LPG for cooking (Cabinet Committee on Economic Affairs 2018). The scheme aimed to empower women and safeguard health. But still, LPG has not become the primarily source of fuel because of the following reasons:

•High upfront costs

• High refill costs

•Extended refilling time

•Lack of adequate infrastructure

During the survey it was found, there were lack of LPG distributors within a reasonable distance and therefore can be considered as major problem, as carrying cylinders over long distances which is exhausting, especially in view of the lack of roads in some rural areas are acting as the main barriers for the non-adoption of LPG.

Firewood: It was found during survey firewood provided the main fuel source for home heating, domestic hot water, and food preparation. The forest has been the dominant source for the supply of wood. However, due to increase in demand, fuel wood is also sourced from Trees outside forest, shrubs, dead logs. Being cost effective, relative affordability and easily accessible makes its primary fuel choice.

The baseline survey study concludes that, firewood is the primary source of fuel in targeted rural households in the various districts of Bihar and Chhattisgarh, India.

The kitchen performance test: The kitchen performance test is performed to understand an average consumption of fuelwood usage per household. The kitchen performance test allows the testers to understand the rate of daily fuelwood consumption per household, as they are used in the normal household environment over a period of time. The KPT is a prolonged test conducted with the willing cooperation of the individual families. More than 90% of the approached families are willing to participate in the kitchen performance test. It is done by conducting daily measurement as families use the traditional stove for a period of 3 days. Results are compiled at the end of the test period. The kitchen performance test household data helps to derive the monthly and yearly consumption of biomass usage per household.

Total number of individuals in a household reflects the rationality of the quantity of biomass fuel consumed.

Average number of individuals in a household reflects the rationality of the quantity of biomass fuel consumed.

| District | Average Household size |
| --- | --- |
| Chhattisgarh | 5.20 |
| Bihar | 5.50 |

For 3 experimental days, it is evidenced that the average frequency for food preparation is 2-3 times a day. The practice of additional preparation of food items/thermal applications viz. snacks, preparation of tea, hot water etc is also been carried out.

In case of experimental households, in field, the remaining quantity of the measured fuel was weighed back on daily basis and noted down. However, for the analytical purpose, the average daily consumption for the three experimental days is taken. The survey results on household’s fuel consumption are as follows.

| Districts | Annual Fuelwood Use (Tons) |
| --- | --- |
| Chhattisgarh | 4.40 |
| Bihar | 4.20 |

##### B.5. Demonstration of additionality

As described in section A.2 above, the project falls under GG4GG Community Services Activity Requirements. As per Annex-B Positive list under ‘GG4GG Community Services Activity Requirements’ the project meets the criteria 3 ‘Project activities solely composed of isolated units where the users of the technology/measure are households or communities or institutions and where each unit results in <= 600 MWh of energy savings per year or <=600 tonnes of emission reductions per year’ as explained below:

Emission reduction:

Considering a biogas plant can save around 4.40 ton/HH/year of firewood in Chhattisgarh and 4.20 ton/HH/year of firewood in Bihar which is used for cooking in baseline, the emission reduction as per applied methodology (AMS-I.E, version 12) is calculated as below:

| By=NHH×(BCBL,HH,y-BCPJ,HH,y) |  |
| --- | --- |

Where,

| By | = | Quantity of woody biomass that is substituted or displaced in tonnes |
| --- | --- | --- |
| NHH | = | Number of households in the project activity, number. It is considered 1 considering a single unit. |
| BCBL, HH,y | = | Average annual consumption of woody biomass per household before the start of the project activity, tonnes/household/year |
| BCPJ, HH,y | = | If it is found that pre-project devices were not completely displaced but continue to be used to some extent, average annual consumption of woody biomass per household in the pre-project devices during the project activity, tonnes/household/year. It is considered conservatively that entire firewood is replaced. |

Baseline emissions - Chhattisgarh,

By = 1* (4.40 -0)

= 4.40 tonnes

=6500*4.40

= 28,600 tonnes

Therefore, emission reduction is

BEy=By×fNRB,y×NCVbiomass×EFprojected_fossil_fuel

= 28,600*92% (most conservative value) * 0.0156* 64.4

= 26,434 tCO2per year

For Bihar,

Baseline emissions- Bihar,

By = 1* (4.20 -0)

= 4.20 tonnes

= 2737 * 4.2

= 11, 495

Therefore, emission reduction is

BEy=By×fNRB,y×NCVbiomass×EFprojected_fossil_fuel

= 11, 495 *99% (most conservative value)* 0.0156* 64.4

= 11, 432.8  tCO2per year

Therefore, with conservative assumptions, the emission reduction for an individual unit is far below 600 tonnes of emission reduction per year. Hence, the proposed project activity falls under the ‘Positive List’ and hence under Principle 5 – Financial Additionality & Ongoing Financial Need, the project is considered deemed additional and therefore not required to prove Financial Additionality at the time of Design Certification.

B.5.1 Prior Consideration

As per GS4GG rule for retroactive projects, project documents need to submit to GS within one year of the project start date to meet prior consideration. Only biogas plants commissioned from one year prior to GS submission shall be considered.

B.5.2 Ongoing Financial Need

Ongoing Financial Need shall be demonstrated at Design Certification Renewal (Refer clause 3.5.2.2 of GS4GG ‘principle and requirements’).

##### B.6. Sustainable Development Goals (SDG) outcomes

Relevant Target/Indicator for each of the three SDGs

| SDGs | Targets |
| --- | --- |
|  | The project will contribute to the SDG target 3.9 “By 2030, substantially reduce the number of deaths and illnesses from hazardous chemicals and air, water and soil pollution and contamination”. The project replaces firewood consumption with biogas. Therefore, the indoor air quality at user point will improve and will contribute to the SDG target. The indicator would be 3.9.1 ‘Mortality rate attributed to household and ambient air pollution’ |
|  | The project will contribute towards below SGD targets: - By 2030, ensure universal access to affordable, reliable and modern energy services (7.1) - By 2030, increase substantially the share of renewable energy in the global energy mix (7.2) |
|  | The project will contribute towards below SDG target (13.2): - Integrate climate change measures into national policies, strategies and planning |

B.6.1 Explanation of methodological choices/approaches for estimating the SDG Impact

>>SDG 13:

As per “AMS I.E- Switch from non-renewable biomass for thermal applications by the user, Version 12,  the baseline emissions (BEy) are calculated as:

| BEy=By×fNRB,y×NCVbiomass×EFprojected_fossil_fuel |  |
| --- | --- |

Where,

| BEy | = | Baseline emissions during the year y in t CO2e |
| --- | --- | --- |
| By | = | Quantity of woody biomass that is substituted or displaced in tonnes |
| fNRB,y | = | Fraction of woody biomass used in the absence of the project activity in year y that can be established as non-renewable biomass (fNRB) |
| NCVbiomass | = | Net calorific value of the non-renewable woody biomass that is substituted (IPCC default for wood fuel, 0.0156 TJ/tonne) |
| EFprojected_fossilfuel | = | Emission factor for the substitution of non-renewable woody biomass by similar consumers. Use a value of 64.4 tCO2/TJ for South Asia. |

Byis determined by using option (a) paragraph 27 (a) of the methodology as follows:

“Calculated as the product of the number of households multiplied by the estimate of average annual consumption of woody biomass per household that is displaced by the project activity (tonnes/household/year)”;

| By=NHH×(BCBL,HH,y-BCPJ,HH,y) |  |
| --- | --- |

Where,

| NHH | = | Number of households in the project activity, number |
| --- | --- | --- |
| BCBL, HH,y | = | Average annual consumption of woody biomass per household before the start of the project activity, tonnes/household/year |
| BCPJ, HH,y | = | If it is found that pre-project devices were not completely displaced but continue to be used to some extent, average annual consumption of woody biomass per household in the pre-project devices during the project activity, tonnes/household/year |

BCBL,HH,y has been determined as per third party survey for the sample of households explained in section B.4 above.

Fraction of woody biomass used in the absence of the project activity in year y that can be established as non-renewable biomass (fNRB,y) is determined as per methodological tool ‘Calculation of the fraction of non-renewable biomass’ version 02 as follows:

The fraction of woody biomass that can be established as non-renewable, is:

Where:

fNRB = Fraction of non-renewable biomass (fraction or %)

NRB = Quantity of non-renewable biomass (t/yr)

RB = Quantity of renewable biomass (t/yr)

Estimation of consumption of woody biomass (H/Bold,total) is done following paragraph 11 (a) of the tool ‘Official Statistics’.

Bihar

As per Forest Survey of India report 2011, annual wood fuel consumption in household sector for the state of Bihar = 11,940,000 (Ton/year)

As per Forest Survey of India report 2011, non-domestic wood consumption (Annual fuelwood consumption in household sector and consumption of wood in House construction, Furniture and Agriculture) for the state of Bihar = 30,156,330 (ton/year)

As per Forest Survey of India report 2011, Total wood consumption for the state of Bihar = 42,096,330 (ton/year)

Chhattisgarh

As per Forest Survey of India report 2011, annual wood fuel consumption in household sector for the state of Chhattisgarh = 5,744,000 (Ton/year)

As per Forest Survey of India report 2011, non-domestic wood consumption (Annual fuelwood consumption in household sector and consumption of wood in House construction, Furniture and Agriculture) for the state of Chhattisgarh = 333,23070 (ton/year)

As per Forest Survey of India report 2011, Total wood consumption for the state of Chhattisgarh = 39,067,070 (ton/year)

Procedure to estimate RB:

Renewable biomass (RB) in the country/region/area is estimated using the equation below:

Where:

MAIforest,i = Mean Annual Increment of woody biomass growth per hectare in subcategory i of forest areas (t/ha/yr).

MAIother,i = Mean Annual Increment of woody biomass growth per hectare in subcategory i of other wooded land areas (t/ha/yr). This value is not considered due to non-availability of data.

Fforest,i = Extent of forest in sub-category i (ha). India state of of forest report, 2019 has been referred for this parameter.

Fother,i = Extent of other wooded land in sub-category i (ha). This is not used following footnote 4 of the tool.

Pforest = Extent of non-accessible area (e.g. protected area where extraction of wood is prohibited, geographically remote area) within forest areas (ha). This parameter is optional and not considered.

Pother = Extent of non-accessible area (e.g. protected area where extraction of wood is prohibited, geographically remote area) within other wooded land areas (ha). This parameter is optional and not considered.

I = Sub-category i of forest areas and other wooded land areas

| State | Chhattisgarh | Bihar |
| --- | --- | --- |
| MAIforest,i | 0.5 | 0.5 |
| Fforest,i | 6331000 | 622000 |
| Renewable biomass (RB) | 31,58,000 | 311000 |

The fraction of woody biomass that can be established as non-renewable, is:

fNRB =NRB NRB+RB

Accordingly, the fNRB would be:

| State | H | RB | NRB (H-RB) | fNRB |
| --- | --- | --- | --- | --- |
| Chhattisgarh | 39,067,070 | 31,58,000 | 3,59,09,070 | 92% |
| Bihar | 42,096,330 | 311,000 | 41,785,330 | 99% |

fNRB is calculated for Chhattisgarh to be 92% and for Bihar is 99%.

| SDG 7: |  |
| --- | --- |
| Baseline estimation: All users used fire-wood based cooking system which is not clean energy. | 9,237 users are accessed to clean energy sources. This shall be monitored. It is assumed that with 100% operational status all users are accessed to clean energy source. |
| SDG 3: |  |
| Prone to illness due to smoke released from burning of fire-wood. | Reduction in smoke related illness due to use of biogas instead of firewood. Since, biogas is smokeless; 9,237 users are expected to get rid of kitchen smoke related illness. |

Project Emissions (PEy):

SDG 13:

As per applied methodology AMS-I.E, version 12, project emissions are accounted for below activities:

- CO2 emissions from on-site consumption of fossil fuels due to the project activity

- CO2 emissions from electricity consumption by the project activity

- Methane emission from solid waste disposal or waste water

- Project emissions related to cultivation of feedstock

- Project emissions from transportation

The project activity does not involve any of the above activity and hence, project emissions for the project activity is not applicable. However, while determining By as per equation 3 of the applied methodology, firewood consumed by pre-project devices during the project activity shall be monitored and applied ex-post. This is to be accounted.

Leakage Emissions (LEy):

SDG 13:

Leakage emissions (related to the non-renewable woody biomass saved by the project activity shall be assessed based on ex post surveys of users and the areas from which this woody biomass is sourced (using 90/30 precision for a selection of samples). The following potential source of leakage shall be considered: The use/diversion of non-renewable woody biomass saved under the project activity by non-project households/users that previously used renewable energy sources. If this leakage assessment quantifies an increase in the use of non-renewable woody biomass used by the non-project households/users that is attributable to the project activity, then By is adjusted to account for the quantified leakage. Alternatively, By is multiplied by a net to gross adjustment factor of 0.95 to account for leakages, in which case surveys are not required.

PP has opted default option, and By shall be adjusted with adjustment factor of 0.95 to account leakage.

Emission reductions:

SDG13:

Emission reductions are to be estimated based on the equation below:

ERy = BEy – PEy - LEy

B.6.2 Data and parameters fixed ex ante

13 (Climate Action)- Specific indicator- 13.2.1

| Data/parameter | fNRB,y |
| --- | --- |
| Unit | - |
| Description | Fraction of woody biomass saved by the project activity during year y that can be established as non-renewable biomass |
| Source of data | Calculated following requiremnets of Methodological tool: Calculation of the fraction of non-renewable biomass version 02 |
| Value(s) applied | 92% for Chhattisgarh and 99% for Bihar |
| Choice of data or Measurement methods and procedures | Methodological tool: Calculation of the fraction of non-renewable biomass |
| Purpose of data | Baseline emissions estimation. |
| Additional comment | This value will be fixed for the crediting period and will be reviewed during renewal of crediting period. |

| Data/parameter | NHH |
| --- | --- |
| Unit | Number |
| Description | Number of households in the project activity in year y |
| Source of data | As per PP’s project database |
| Value(s) applied | 9237 |
| Choice of data or Measurement methods and procedures | Project database as per the commissioning reports and end user agreements. |
| Purpose of data | Baseline emissions estimation. |
| Additional comment | This value will be fixed for the crediting period and will be reviewed during renewal of crediting period. |

| Data/parameter | BCBL,HH,y |
| --- | --- |
| Unit | tonnes/household/year |
| Description | Average annual consumption of woody biomass per household before the start of the project activity |
| Source of data | Baseline survey |
| Value(s) applied | 4.40 and 4.20 |
| Choice of data or Measurement methods and procedures | The survey was conducted to know the firewood consumption pattern in line with the methodology in the potential households that uses traditional cookstoves (baseline technologies). Survey was conducted following UNFCCC sampling standard ‘Sampling and surveys for CDM project activities and programmes of activities’ version 9 and guideline ‘Sampling and surveys for CDM project activities and programmes of activities’ version 04. A 90% confidence interval and 10% error margin applied for sample calculation. |
| Purpose of data | Baseline emissions estimation. |
| Additional comment |  |

| Data/parameter | NCVbiomass |
| --- | --- |
| Unit | TJ/tonne |
| Description | Net calorific value of the non-renewable woody biomass that is substituted |
| Source of data | AMS-I.E, version 12 |
| Value(s) applied | 0.0156 TJ/tonne |
| Choice of data or Measurement methods and procedures | IPCC default for wood fuel |
| Purpose of data | Baseline emissions estimation. |
| Additional comment |  |

| Data/parameter | 𝐸𝐹𝑝𝑟𝑜𝑗𝑒𝑐𝑡𝑒𝑑_𝑓𝑜𝑠𝑠𝑖𝑙𝑓𝑢𝑒l |
| --- | --- |
| Unit | tCO2/TJ |
| Description | Emission factor for the substitution of non-renewable woody biomass |
| Source of data | Default value as per AMS-I.E, version 12 |
| Value(s) applied | 64.4 tCO2/TJ |
| Choice of data or Measurement methods and procedures | Default value as per AMS-I.E, version 12 |
| Purpose of data | Baseline emissions estimation. |
| Additional comment |  |

B.6.3 Ex ante estimation of SDG Impact

>>

SDG 13:

Baseline Emissions:

| BEy=By×fNRB,y×NCVbiomass×EFprojected_fossil_fuel Where, |  |
| --- | --- |
| By=NHH×(BCBL,HH,y-BCPJ,HH,y) Total Baseline emissions = 35,979.00 tCO2/year |  |

BCPj,HH,y (if it is found that pre-project devices were not completely displaced but continue to be used to some extent, average annual consumption of woody biomass per household in the pre-project devices during the project activity, tonnes/household/year) is considered zero during design certification stage. The same shall be monitored ex-post certification.

The fraction of woody biomass that can be established as non-renewable, is:

Accordingly, the fNRB would be: 92% for Chhattisgarh and 99% for Bihar

Accordingly, BEy would be:

| BEy=By×fNRB,y×NCVbiomass×EFprojected_fossil_fuel |  |
| --- | --- |

Project Emissions (PEy): Nil as explained in section B.6.2 above.

Leakage Emissions: By is multiplied by a net to gross adjustment factor of 0.95 to account for leakages.

Emission reductions 	= BEy – PEy – LEy

## SDG 3:

| ## Baseline estimation | ## Ex-ante estimation |
| --- | --- |
| ## Prone to illness due to smoke released from burning of fire-wood | ## Reduction in smoke related illness due to use of biogas instead of firewood. ## Since, biogas is smokeless; 100% users are expected to get rid of kitchen smoke related illness. Ex-post monitoring methods are discussed in section B.7.1 below. |

## SDG 7:

| ## Baseline estimation | ## Ex-ante estimation |
| --- | --- |
| ## All users used fire-wood based cooking system which is not clean energy. | ## 100% users are accessed to clean energy sources. This shall be monitored. ## It is assumed that with 100% operational status all users are accessed to clean energy source. Ex-post monitoring methods are discussed in section B.7.1 below. |

B.6.4 Summary of ex ante estimates of each SDG Impact

| Year | Baseline estimate | Leakage | Net benefit |
| --- | --- | --- | --- |
| Year 1 | 37, 874 | 1, 895 | 35,979 |
| Year 2 | 37, 874 | 1, 895 | 35,979 |
| Year 2 | 37, 874 | 1, 895 | 35,979 |
| Year 4 | 37, 874 | 1, 895 | 35,979 |
| Year 5 | 37, 874 | 1, 895 | 35,979 |
| Total | 189, 368 | 9,475 | 179, 895 |

| Total number of crediting years |
| --- |
| Annual average over the crediting period | 37, 874 | 1895 | 35979 |

##### B.7. Monitoring plan

B.7.1 Data and parameters to be monitored

Greneity infocom Service Pvt Ltd (hereafter Greneity) facilitates the End User families to set up village level institutions to take care of repairs, maintenance and the social controls/peer support needed to cope with various exigencies that crops up.

Biogas Project Management Unit

A dedicated team is set up within Greneity for management and monitoring of the Biogas Project. This Project Management & Monitoring Unit consist of the following staff:

Biogas Field Workers/RETs:

Biogas field workers oversee the operation & maintenance of biogas units.

Each field worker is in-charge of approximately 500 units. They report to the Project Manager and their tasks are to:

- Inspect the bio-digesters in periodic intervals to ensure proper functioning of all the plants

- Identify local suppliers of material for village clusters

- Record the working of bio-digesters in specially designed formats and handover the data to manager on a regular basis

- Assist in selection of Volunteers, train and support them

- Contract Masons and attend to major repairs that cannot be handled by the village level institutions

Management system at village level:

Village level Volunteers maintain the Monitoring log Books where the reasons for non-usage are recorded. Every participating village have a Volunteer to monitor usage of biogas units. Volunteer(s)/mason(s) will be responsible for monitoring of usage and is the first to identify dysfunctional units. The Volunteer either prompts the End User to set right a problem or bring it to the notice of the Biogas Technician.

Each Biogas Unit is marked with a unique Identification Number and date of construction.

A Monitoring log Book is maintained for the project activity. If any Biogas Unit is faulty or not functional, the problem report is automatically passed on to Volunteer(s)/mason(s). There is a continuous database maintained of all the Biogas Units not operational on a regular basis where number of non-operational days, and the reasons of non-operation are recorded. Mostly, volunteers are able to handle the problem arises, in case of major issues, Biogas technicians are informed to handle the situation.

Information flow diagram is as follows;

SDG 13 (Climate Action) -Specific indicator- 13.2.1

| Data / Parameter | BCPJ,HH,y |
| --- | --- |
| Unit | tonnes/household/year |
| Description | Average annual consumption of woody biomass per household in the pre-project devices during the project activity, if it is found that pre-project devices were not completely displaced but continue to be used to some extent. |
| Source of data | Survey |
| Value(s) applied | To be monitored |
| Measurement methods and procedures | Monitoring shall consist of estimation of all project devices or a representative sample thereof, at least once every two years(biennial) |
| Monitoring frequency | At least once every two years (biennial) |
| QA/QC procedures | Third party survey to be conducted following standard sampling approach. |
| Purpose of data | Baseline Emissions estimation |
| Additional comment | -- |

| Data / Parameter | NHH,y |
| --- | --- |
| Unit | Number |
| Description | Number of households (biogas system) in the project activity in operational per year. |
| Source of data | Survey |
| Value(s) applied | To be monitored |
| Measurement methods and procedures | Monitoring consists of checking of representative sample, to ensure that bio-digesters operating. Calculated as NHH *Py, where NHH is the number of biodigesters installed in the project and Py is Proportion of Bio-digesters operational estimated based on the sample survey. |
| Monitoring frequency | At least once every two years (biennial) |
| QA/QC procedures | Third party survey to be conducted following standard sampling approach. |
| Purpose of data | Baseline Emissions estimation |
| Additional comment | -- |

SDG 3 (Good health and well-being) - Specific indicator 3.9.1

| Data / Parameter | Improvement in health and decrease in illness |
| --- | --- |
| Unit | Qualitative |
| Description | Indoor air quality at usage point |
| Source of data | Sampling survey/annual usage survey/monitoring survey |
| Value(s) applied | To be monitored |
| Measurement methods and procedures | Improvement in health and decrease in illness will be assessed through interview with end users due to project implementation. Users opinion on indoor air quality due to biogas usage shall be collected during monitoring survey. In addition, training to the operation and maintenance technicians and field supervisors to be provided to increase awareness in safe operation and handling emergency situations. |
| Monitoring frequency | At least once every two years (biennial) |
| QA/QC procedures | Third party survey to be conducted following standard sampling approach. Publicly available data may be referred. |
| Purpose of data | Sustainable development assessment |
| Additional comment | -- |

SDG 7 (Affordable and clean energy) - Specific indicator 7.1.2

| Data / Parameter | Access to affordable and clean energy services |
| --- | --- |
| Unit | Numbers |
| Description | Number of biogas system operational under the project activity |
| Source of data | Biogas User Survey |
| Value(s) applied | To be monitored |
| Measurement methods and procedures | Sample survey to confirm if project biogas systems are operational. Operational status will confirm that the users are accessed to affordable and clean energy. |
| Monitoring frequency | At least once every two years (biennial) |
| QA/QC procedures | Required sample size shall be determined following UNFCCC sampling standard |
| Purpose of data | Sustainable development assessment |
| Additional comment | -- |

B.7.2 Sampling plan

- >> Sampling Approach:

- Objectives and reliability requirements: The objective of the sampling plan is to achieve unbiased and reliable estimates of the proportion or the mean value of the key variables over the crediting period. The methodology applied for the project (AMS.I.E version 12) requires the project proponent achieving 95 percent confidence interval and a 10 percent margin of error when biennial inspection is chosen and when annual inspection is chosen 90 per cent. The table below provides the monitoring parameters that will be monitored annually:

| Parameter | Type | Description |
| --- | --- | --- |
| NHH | Proportional parameter | Number of households (biogas system) in the project activity in operational per year |
| BCPJ,HH,y | Mean value parameter | Average annual consumption of woody biomass per household in the pre-project devices during the project activity, if it is found that pre-project devices were not completely displaced but continue to be used to some extent. |

- Target Population: The target population for different parameters discussed in the table above are given below:

  - For the proportional parameter; the target population is the bio-digester users listed in the project database.

  - For the mean value parameter; the target population is the total number of operational biogas digester for which the emission reductions will be accounted for the monitoring period in question.

- Sampling frame: All the households with biogas digester within the project will be the sampling frame.

- Sampling Method: A simple random sampling will be adopted for estimating the sample size for the monitoring surveys. Simple random sampling is suited to populations that are homogenous (EB 86 annex 04).

- Sample Size: The calculation of the required sample size for each parameter will be calculated at 90/10 confidence/precision as required for the annual monitoring. The sample size is determined using the Guidelines for Sampling and Surveys for CDM Project activities and Programme of Activities Ver. V4.0 (EB86, Annex 4).

Proportional Parameters (Operational status)

The minimum sample size to determine number of biogas system in operation using the procedure outlined in para 12 of appendix 1, EB 86 Annex 4, Guidelines for Sampling and Surveys for CDM Project activities and Programme of Activities Ver. 4.0.

n≥1.6452N×p(1-p)N-1×0.12×p2+1.6452p(1-p)

Where:

n= Sample size

N = Total number of biogas system of type i installed under the project

p = expected proportion (50%)

1.645 = represents the 90% confidence required

0.1 = represents the 10% relative precision

n≥1.6452NVN-1×0.12+1.6452V

Out of the sample calculated above, conservative number of sample will be taken

- Data:

- Field Measurements:

- Checking of a representative sample of biogas system within the project to ensure that they are still operating (NHH).

The selection of households will be done using rand function in excel to generate random number and taking households from first up to the number of sample required by arranging the random number in ascending or descending order.

- Determine average annual consumption of woody biomass per household in the pre-project devices during the project activity, if it is found that pre-project devices were not completely displaced but continue to be used to some extent.

- The survey will be conducted annually with the objective to target 10 percent precision and to achieve 90 percent confidence.

- Quality Assurance/Quality Control:

A survey questionnaire will be prepared to seek responses of operating status (yes or no) of biogas digesters within the project activity. Users will be asked to provide their estimates on consumption of woody biomass after project activity. The survey will be performed by the project developer appointing a third party.

- Analysis:

The project developer will collect, compile and analyze the data to derive the number of bio-digesters within the project still operational and avenarge firewood consumption at each bio-digester user (if any). The developer will prepare “monitoring report” based on the survey report.

- Implementation:

The survey questionnaire will be prepared, pre-tested and field personnel will be trained in conducting the survey to ensure the quality of data collected and the survey will be carried out as per defined frequency covering required data information. The schedule for implementing the sampling effort shall be defined prior to the field activity.

B.7.3 Other elements of monitoring plan

>> N/A

#### SECTION C. DURATION AND CREDITING PERIOD

##### C.1. Duration of project

C.1.1 Start date of project

>> 01/03/2021

C.1.2 Expected operational lifetime of project

>> 20 Years

##### C.2. Crediting period of project

C.2.1 Start date of crediting period

>>01/03/2021. The date represents first batch of biogas digesters installed within the project activity.

C.2.2 Total length of crediting period

>>The project falls under GS community service activity and hence eligible for total 15 years issuance with certification renewal every 5 years. So the length of crediting period will be 5 year* 3 times.

#### SECTION D. SUMMARY OF SAFEGUARDING PRINCIPLES AND GENDER SENSITIVE ASSESSMENT

##### D.1 Safeguarding Principles that will be monitored

A completed Safeguarding Principles Assessment is in Appendix 1, ongoing monitoring is summarised below.

| Principles | Mitigation Measures added to the Monitoring Plan |
| --- | --- |
| N/A |  |

##### D.2. Assessment that project complies with GS4GG Gender Sensitive requirements

| Question 1 - Explain how the project reflects the key issues and requirements of Gender Sensitive design and implementation as outlined in the Gender Policy? | The project respects the key gender issues and requirements of gender-sensitive design and implementation of the project. SDG#5 is one of the impact areas of the project. The project is aimed to replace conventional wood fired mud stoves technology with biogas primarily for cooking. This will result in reducing use of firewood consumption or replacement of complete firewood usage at user place. Primarily in rural areas of India, cooking activity at household level is managed by women. Therefore, women are more exposed to the indoor air pollution and the associated hazard. Women in rural areas in most cases are also responsible for taking care of their children specially infants who need mother’s support most of the time are bound to accompany their mother in kitchen. This situation leads to enhanced exposure of the women and children to kitchen smoke and associated health consequences. Since the project aims to replace the polluting traditional cooking stoves with biogas cooking system, the primary beneficiary would be the women and children. Furthermore, the project is focused to the economically dis-advantaged group of people, which also justifies the dimension of social inclusion in the project design. |
| --- | --- |
| Question 2 - Explain how the project aligns with existing country policies, strategies and best practices | The Government of India reaffirms its commitment to work for the realization of constitutional guarantee of equality, social justice and non-discrimination on the basis of sex, caste, community, language and religion. Ministry of women & child development, Govt. of India has taken various measures for gender equality/socio-economic development/empowerment of women. Out of these, the project positively contributes towards the national mission for empowerment of women through improvement of health and attaining vision for empowerment of women under national policy for women 2016 (Women participation will be ensured in the efficient use and spreading the use of solar energy, biogas, smokeless chulas and other technological applications to have positive influence on their life styles and a long term impact on meeting sustainable development goals) |
| Question 3 - Is an Expert required for the Gender Safeguarding Principles & Requirements? | Not required. |
| Question 4 - Is an Expert required to assist with Gender issues at the Stakeholder Consultation? | Not required. |

#### SECTION E. SUMMARY OF LOCAL STAKEHOLDER CONSULTATION

##### E.1 Summary of stakeholder mitigation measures

Greneity and its partner NGOs invited all the relevant stake holders like Gold standard supporting NGOs, DNA, local Government officials, elected representatives and the villagers for this meeting.

The local stake holder consultation meeting was successfully held on 12-03-2022 as per the details below:

| Date of Meeting | Place |
| --- | --- |
| 12th March 2022, 10 AM onwards | Village Chendra Block Vikaskhand Odhmi Surajpur District Chhattisgarh, India. |
| 12th March 2022, 10 AM onwards | Village Benipur Block Kurtha Arwal District Bihar, India. |

The project idea and its components were thoroughly discussed, feedback from the participants has been documented and sustainable development aspects were discussed during the proceedings. Participants got clarification on many of the project aspects.

Village Sarpanch and Pradhan explained about the biogas program of the Government and its limitations and implementation style. They stated that the present project is much useful to the farming community in general and women. Further, the positive impact on the women health by using the clean fuel technology is also explained.

RET’s introduced the present project to the participants and clarified many of their queries. The RET, also explained the transfer of ownership of Carbon credits, to provide free operation and maintenance of the biogas plant. Staff members of the Samagra Vikas Mission recorded attendees’ details in a special register. Further, RET’s explained how this Project activity is supporting the installation of biogas plants to rural households and provide participating households with clean, convenient, and efficient means for cooking and heating purposes. They also provided insights on how the monitoring of installed domestic biogas units could lead to positive sustainable development goals.

They had distributed feedback forms among the participants and collected once they were duly filled. The minutes of the meeting were recorded, and meeting photos were taken to document the event.

The following were the sample questions raised in the meeting:

How long the project will be monitored and maintained?

Basic qualification required to be technician?

The following were the sample comments from the feedback forms:

1. The meeting is well organized and very informative.

2. The project is a needed and useful for both farmers and women.

3. There is nothing to dislike in the project.

The participants overwhelmingly supported the project, and no negative comments were raised.

Stakeholder feedback round has been conducted virtually by inviting the stakeholders through email with stakeholders being sent the project documentation to submit feedback. Stakeholder feedback round was initiated on 08-02-2022 for 60 days.

However, no comments were received during this period.

##### E.2 Final continuous input / grievance mechanism

| Method | Include all details of Chosen Method (s) so that they may be understood and, where relevant, used by readers. |
| --- | --- |
| Continuous Input / Grievance Expression Process Book (mandatory) | The project details and grievance logbook has been maintained in the panchayat office. |
| GS Contact (mandatory) | help@goldstandard.org |
| Other |  |

### Appendix 1 - Safeguarding Principles Assessment

Complete the Assessment below and copy all Mitigation Measures for each Principle into SECTION D above. Please refer to the instructions in the Guide to Completing this Form.

| Assessment Questions/ Requirements | Justification of Relevance (Yes/potentially/no) | How Project will achieve Requirements through design, management or risk mitigation. | Mitigation Measures added to the Monitoring Plan (if required) |
| --- | --- | --- | --- |
| Principle 1. Human Rights |  |  |  |
| - The Project Developer and the Project shall respect internationally proclaimed human rights and shall not be complicit in violence or human rights abuses of any kind as defined in the Universal Declaration of Human Rights - The Project shall not discriminate with regards to participation and inclusion | No | 1. India adopted ‘The protection of human rights Act’ 1993 and the project is bound to follow the rules and regulation of host country. In addition, India has ratified ‘International Convention on the Elimination of All Forms of Racial Discrimination :1969’ ‘International Covenant on Civil and Political Rights :1976’, ‘International Covenant on Economic, Social and Cultural Rights :1976’ ‘Convention on the Elimination of All Forms of Discrimination against Women :1981’. Therefore, the project developer and the project do respect nationally and internationally proclaimed human rights and shall not be complicit in violence or human rights abuses of any kind. 2. India has ratified ‘International Convention on the Elimination of All Forms of Racial Discrimination :1969’, ‘Convention on the Elimination of All Forms of Discrimination against Women :1981’ in addition to its national human rights Act’ 1993. Therefore, the project will not discriminate with regards to participation and inclusion. | Not applicable. |
| Principle 2. Gender Equality |  |  |  |
| - The Project shall not directly or indirectly lead to/contribute to adverse impacts on gender equality and/or the situation of women - Projects shall apply the principles of non-discrimination, equal treatment, and equal pay for equal work - The Project shall refer to the country’s national gender strategy or equivalent national commitment to aid in assessing gender risks - (where required) Summary of opinions and recommendations of an Expert Stakeholder(s) | No | The project happens in individual households. It does not involve any women workforce which may lead to sexual harassment The project does not has any scope to apply gender strategy as such. Although the project positively contributes towards the national mission for empowerment of women through improvement of health and attaining vision for empowerment of women under national policy for women 2016. | Not applicable. |
| Principle 3. Community Health, Safety and Working Conditions |  |  |  |
| - The Project shall avoid community exposure to increased health risks and shall not adversely affect the health of the workers and the community | No | The project leads to safe working condition and improvement in health as it will replace firewood as fuel with biogas which is clean and safe | Not Applicable |
| Principle 4.1 Sites of Cultural and Historical Heritage |  |  |  |
| Does the Project Area include sites, structures, or objects with historical, cultural, artistic, traditional or religious values or intangible forms of culture? | No | The project area covers households which does not have any structures, or objects with historical, cultural, artistic, traditional or religious values or intangible forms of culture. Hence, not applicable | Not applicable |
| >> |  |  |  |
| Principle 4.2 Forced Eviction and Displacement |  |  |  |
| Does the Project require or cause the physical or economic relocation of peoples (temporary or permanent, full or partial)? | No | The project activity takes place within individual households. Therefore, no relocation of people takes place | Not applicable |
| >> |  |  |  |
| Principle 4.3 Land Tenure and Other Rights |  |  |  |
| - Does the Project require any change, or have any uncertainties related to land tenure arrangements and/or access rights, usage rights or land ownership? - For Projects involving land use tenure, are there any uncertainties with regards to land tenure, access rights, usage rights or land ownership? | No | The project area covers households which does not have any structures, or objects with historical, cultural, artistic, traditional or religious values or intangible forms of culture. Hence, not applicable | Not applicable |
| >> |  |  |  |
| Principle 4.4 - Indigenous people |  |  |  |
| Are indigenous peoples present in or within the area of influence of the Project and/or is the Project located on land/territory claimed by indigenous peoples? | No | No. The project involves household biogas digesters. Therefore, it does not involve any influence towards indigenous people | Not applicable |
| >> |  |  |  |
| Principle 5. Corruption |  |  |  |
| - The Project shall not involve, be complicit in or inadvertently contribute to or reinforce corruption or corrupt Projects | No | The project benefits households with clean fuel (biogas). There is no corruption provision in the project activity. | Not applicable |
| Principle 6.1 Labour Rights |  |  |  |
| - The Project Developer shall ensure that all employment is in compliance with national labour occupational health and safety laws and with the principles and standards embodied in the ILO fundamental conventions - Workers shall be able to establish and join labour organisations - Working agreements with all individual workers shall be documented and implemented and include: - Working hours (must not exceed 48 hours per week on a regular basis), AND - Duties and tasks, AND - Remuneration (must include provision for payment of overtime), AND - Modalities on health insurance, AND - Modalities on termination of the contract with provision for voluntary resignation by employee, AND - Provision for annual leave of not less than 10 days per year, not including sick and casual leave. - No child labour is allowed (Exceptions for children working on their families’ property requires an Expert Stakeholder opinion) - The Project Developer shall ensure the use of appropriate equipment, training of workers, documentation and reporting of accidents and incidents, and emergency preparedness and response measures | No | The project does not require labour force for implementation of the project. Trained technicians are involved in construction and operation and maintenance of plants. Therefore, no forced labour is involved in the project. No child labour is involved | Not applicable |
| Principle 6.2 Negative Economic Consequences |  |  |  |
| - Does the project cause negative economic consequences during and after project implementation? | No | The project has lifetime of 20 years and expected to run beyond the project certification period. With carbon finance it is further strengthened to operate 100% project systems with continuous maintenance | Not applicable |
| >> |  |  |  |
| Principle 7.1 Emissions |  |  |  |
| Will the Project increase greenhouse gas emissions over the Baseline Scenario? | No | No. The project will replace firewood use with biogas. Hence, it will reduce greenhouse gas emissions over the Baseline Scenario. | Not applicable |
| >> |  |  |  |
| Principle 7.2 Energy Supply |  |  |  |
| Will the Project use energy from a local grid or power supply (i.e., not connected to a national or regional grid) or fuel resource (such as wood, biomass) that provides for other local users? | No | No. The project uses inhouse cattle dung only. | Not applicable |
| >> |  |  |  |
| Principle 8.1 Impact on Natural Water Patterns/Flows |  |  |  |
| Will the Project affect the natural or pre-existing pattern of watercourses, ground-water and/or the watershed(s) such as high seasonal flow variability, flooding potential, lack of aquatic connectivity or water scarcity? | No | No. Water in huge quantity is not required for the project which can impact the ground water level or any seasonal flow. Water is needed to mix with cattle dung while feeding in digester | Not applicable |
| >> |  |  |  |
| Principle 8.2 Erosion and/or Water Body Instability |  |  |  |
| - Could the Project directly or indirectly cause additional erosion and/or water body instability or disrupt the natural pattern of erosion? - Is the Project’s area of influence susceptible to excessive erosion and/or water body instability? | No | No. Water in huge quantity is not required for the project which can impact the ground water level or any seasonal flow. Water is needed to mix with cattle dung while feeding in digester | Not applicable |
| >> |  |  |  |
| Principle 9.1 Landscape Modification and Soil |  |  |  |
| Does the Project involve the use of land and soil for production of crops or other products? | No | No. Since the project takes place in individual households, no net degradation in existing landscape function and services takes place | Not applicable |
| >> |  |  |  |
| Principle 9.2 Vulnerability to Natural Disaster |  |  |  |
| Will the Project be susceptible to or lead to increased vulnerability to wind, earthquakes, subsidence, landslides, erosion, flooding, drought or other extreme climatic conditions? | No | No. The project happens at individual households. There is no activity which can affect adversely the natural system to cause earthquake, landslides, erosion, flooding, draught or other extreme climatic conditions | Not applicable |
| >> |  |  |  |
| Principle 9.3 Genetic Resources |  |  |  |
| Could the Project be negatively impacted by or involve genetically modified organisms or GMOs (e.g., contamination, collection and/or harvesting, commercial development, or take place in facilities or farms that include GMOs in their processes and production)? | No | Not applicable. The project does not involve any crop production or cultivation | Not applicable |
| >> |  |  |  |
| Principle 9.4 Release of pollutants |  |  |  |
| Could the Project potentially result in the release of pollutants to the environment? | No | No. The project do not release any pollutants to the environment | Not applicable |
| >> |  |  |  |
| Principle 9.5 Hazardous and Non-hazardous Waste |  |  |  |
| Will the Project involve the manufacture, trade, release, and/ or use of hazardous and non-hazardous chemicals and/or materials? | No | Not applicable. The project does not involve any production process | Not applicable |
| >> |  |  |  |
| Principle 9.6 Pesticides & Fertilisers |  |  |  |
| Will the Project involve the application of pesticides and/or fertilisers? | No | No | Not applicable |
| >> |  |  |  |
| Principle 9.7 Harvesting of Forests |  |  |  |
| Will the Project involve the harvesting of forests? | No | Not applicable. The project happens at individual households | Not applicable |
| >> |  |  |  |
| Principle 9.8 Food |  |  |  |
| Does the Project modify the quantity or nutritional quality of food available such as through crop regime alteration or export or economic incentives? | No | Not applicable | Not applicable |
| >> |  |  |  |
| Principle 9.9 Animal husbandry |  |  |  |
| Will the Project involve animal husbandry? | No | Not applicable | Not applicable |
| >> |  |  |  |
| Principle 9.10 High Conservation Value Areas and Critical Habitats |  |  |  |
| Does the Project physically affect or alter largely intact or High Conservation Value (HCV) ecosystems, critical habitats, landscapes, key biodiversity areas or sites identified? | No | Not applicable | Not applicable |
| >> |  |  |  |
| Principle 9.11 Endangered Species |  |  |  |
| a. Are there any endangered species identified as potentially being present within the Project boundary (including those that may route through the area)? - Does the Project potentially impact other areas where endangered species may be present through transboundary affects? | No | Not applicable | Not applicable |
| >> |  |  |  |

### Appendix 2- Contact information of project participants

| Organization name | Greneity Infocom Service Pvt Ltd |
| --- | --- |
| Registration number with relevant authority | CIN-U74900DL2013PTC253219 under Companies Act,1956 |
| Street/P.O. Box | Dilshad Garden |
| Building | 137-B, Pocket-A |
| City | Delhi |
| State/Region | Delhi |
| Postcode | 110095 |
| Country | India |
| Telephone | +91-9886252062 |
| E-mail | Shivani.garg@greneity.com |
| Website | www.greneity.com |
| Contact person | Shivani Garg |
| Title | Managing Director |
| Salutation | Ms. |
| Last name | Garg |
| Middle name |  |
| First name | Shivani |
| Department |  |
| Mobile | +91- 9886252062 |
| Direct tel. |  |
| Personal e-mail | Shivani.garg@greneity.com |

### Appendix 4-Summary of Approved Design Changes

Please refer to Design Change Requirements for more information on procedures governing Design Changes

##### Revision History

| Version | Date | Remarks |
| --- | --- | --- |
| 1.2 | 14 October 2020 | Hyperlinked section summary to enable quick access to key sections Improved clarity on Key Project Information Inclusion criteria table added Gender sensitive requirements added Prior consideration (1 yr rule) and Ongoing Financial Need added Safeguard Principles Assessment as annex and a new section to include applicable safeguards for clarity Improved Clarity on SDG contribution/SDG Impact term used throughout Clarity on Stakeholder Consultation information required Provision of an accompanying Guide to help the user understand detailed rules and requirements |
| 1.1 | 24 August 2017 | Updated to include section A.8 on ‘gender sensitive’ requirements |
| 1.0 | 10 July 2017 | Initial adoption |

Commissioning certificates co-signed by the respective plant owners and Samagra Vikas Mission.

https://www.ctae.ac.in/images/editorFiles/file/CTAE-2017/Biogas%20FAQ_English.pdf  (Refer the page no. 06 point 25 of the document.)

https://fsi.nic.in/cover_2011/chapter7.pdf

https://fsi.nic.in/cover_2011/chapter7.pdf

CDM Methodological Tool -30: Calculation of the fraction of non-renewable biomass, version 03

https://fsi.nic.in/isfr19/vol2/isfr-2019-vol-ii-chhattisgarh.pdf (Page no. 44)

https://fsi.nic.in/isfr19/vol2/isfr-2019-vol-ii-bihar.pdf (Page no. 34)

https://moef.gov.in/wp-content/uploads/2019/06/Pacific.pdf (page no. 16 of Asia-Pacific Forestry Sector Outlook Study II India Outlook Study 2020)

- Error! Hyperlink reference not valid.

- Error! Hyperlink reference not valid.

http://pib.nic.in/newsite/PrintRelease.aspx?relid=132945

http://wcd.nic.in/sites/default/files/women%20empowerment%20poliy_Final_17May.pdf