Key Project Information & project Design Document (PDD)

###### PUBLICATION DATE  14.10.2020
VERSION  v. 1.2 
RELATED SUPPORT

###### - TEMPLATE GUIDE Key Project Information & Project Design Document v.1.2

This document contains the following Sections

Key Project Information

SECTION A – Description of project

SECTION B - Application of approved Gold Standard Methodology (ies) and/or    demonstration of SDG Contributions

SECTION C – Duration and crediting period

SECTION D – Summary of Safeguarding Principles and Gender Sensitive Assessment

SECTION E – Outcome of Stakeholder Consultations

Appendix 1 – Safeguarding Principles Assessment (mandatory)

Appendix 2 - Contact information of Project participants (mandatory)

Appendix 3 - LUF Additional Information (project specific)

Appendix 3 - Summary of Approved Design Changes (project specific)

### KEY PROJECT INFORMATION

| GS ID of Project | GS11202 |
| --- | --- |
| Title of Project | BIM Wind Power |
| Time of First Submission Date | 25/05/2021 |
| Date of Design Certification | Not Available |
| Version number of the PDD | 1.6 |
| Completion date of version | 21/08/2023 |
| Project Developer | AC Energy Corporation (an Ayala Company) |
| Project Representative | AC Energy Corporation (an Ayala Company) BIM Wind Power Joint Stock Company |
| Project Participants and any communities involved | AC Energy Corporation (an Ayala Company) BIM Wind Power Joint Stock Company |
| Host Country (ies) | Viet Nam |
| Activity Requirements applied | Community Services Activities Renewable Energy Activities Land Use and Forestry Activities/Risks & Capacities N/A |
| Scale of the project activity | Micro scale Small Scale Large Scale |
| Other Requirements applied | N/A |
| Methodology (ies) applied and version number | ACM0002 - Grid-connected electricity generation from renewable sources (version 20.0) |
| Product Requirements applied | GHG Emissions Reduction & Sequestration Renewable Energy Label N/A |
| Project Cycle: | Regular Retroactive |

Table 1 – Estimated Sustainable Development Contributions

| ## Sustainable Development Goals Targeted | ## SDG Impact (defined in B.6.) | ## Estimated Annual Average | ## Units or Products |
| --- | --- | --- | --- |
| ## 13 – Climate Action | ## Emission Reduction | ## 251,419 | ## tCO2/year |
| ## 8 – Decent Work and ## Economic Growth | ## Full and productive employment and ## decent work for all (All employees have access to training, healthcare, insurances and better income) | ## 33 | ## employees |
| ## 7 – Affordable and Clean Energy | ## Ensure access to affordable, reliable, ## sustainable and modern energy for ## all (MWh of renewable energy generated) | ## 305,500 | ## MWh |

#### SECTION A. DESCRIPTION OF PROJECT

##### A.1 Purpose and general description of project

>>

BIM Wind Power Project (hereafter “Proposed Project” or “Project Activity”) is a greenfield grid-connected wind power plant which is located in Quan the 1 village, Phuoc Minh commune, Thuan Nam district, Ninh Thuan province, Vietnam.

The proposed project is owned by BIM Wind Power Join Stock company (hereafter “BIM Wind Power JSC”) which is a joint venture between BIM group (Vietnam) and AC Energy Corporation (Philippines). In addition, AC Energy Corporation (an Ayala Company) is a Gold Standard project developer of the proposed project activity. The project activity has a total installed capacity of 88MW, with a predicted power generation of 305,500 MWh per annum. The project activity involves the installation of 22 Wind Turbine Generators (WTGs), comprising a total installed capacity of 88 MW.

Prior to the implementation of the project activity, electricity in Vietnam is generated mainly from fossil fuel sources and is solely distributed to consumers by Vietnam Electricity (EVN) via the unique national electricity grid. The baseline scenario of the project activity is the same as the scenario existing prior to the start of implementation of the project activity.

The entire electricity generated by the project activity will be fed into the Vietnam national grid (hereafter referred as “the grid”) without any Greenhouse Gas (GHG) emissions. The operational lifetime of the wind power project is 25 years. The electricity generation will result 251,419 tCO2e/year and total emission reduction will be 1,257,095 tCO2e for the first crediting period of 5 years.

The project activity contributes to the substantiable development as follows:

Economic well-being

Vietnam has been continuously facing power shortage due to increasing demand and inadequate supply. Hence, the government has laid out plans to nearly double the country's installed generation capacity in its base scenario, to cope with the expected surge in power demand. The project will contribute towards country’s power demand by supplying electricity directly to the national grid. Furthermore,

- The project will reduce the dependency of Vietnam on coal, fossil fuel and electricity importation as well as negative impact on the foreign exchange. It will contribute to strengthen the renewable energy sector, especially wind energy sector in Vietnam.

- The project gives a boost to the wind energy sector and push R&D in this field thus improving national economy through innovations.

- The project contributes to poverty alleviation through employment opportunities during project construction & operation as well as towards nation’s as well as commune’s tax revenues. The commune has been diverted the tax to improve the local commune to build roads, community buildings and parks.

Social well-being

- The project has provided / will provide job opportunities to local people during implementation, commissioning and operation of the wind project.

- Frequent visits by engineers, professionals and industrialists to the project sites during project execution will create positive impacts on the economy of the communes through significant income by opening restaurants & convenient shops by the locals in the project area and improve residents’ lifestyle.

- The wind project supports the “revised Power Development Plan VIII (PDP VIII)” in increasing the electricity generation by using renewable energy sources. Also, development of infrastructure like road network and street lightings etc.

Environmental well-being

- Wind energy is one of the cleanest renewable sources and does not involve any fossil fuel. Thus, the wind project contributes to environmental well-being without causing any negative impact on health and surrounding environment.

- The wind project does not contribute to the carbon emissions, harmful pollutants and suspended airborne particulate matter associated with coal and fossil fuel fired power plants.

In conclusion the project activity will contribute positively towards sustainable development of Viet Nam.

The chronology of project implementation as follows:

| Date | Significant Actions |
| --- | --- |
| 29/05/2020 | - Date of EPC (Engineering, Procurement & Construction) contract signed – start date of the project - Date of investment decision |
| 25/06/2020 | License for grid connection |
| 02/07/2020 | Engineering consultancy service agreement signed with K2 Management |
| 10/09/2020 | Date of construction started (commencement with internal road) |
| 13/11/2020 | Full Services Agreement (FSA) signed with GE Vietnam Limited |
| 12/03/2021 | Approval on the investment policy by Ninh Thuan Province People’s Committee |
| 25/03/2021 | Authority’s approval on submission of Environmental Protection Plan (EPP) report. |
| 26/03/2021 | Power Purchase Agreement (PPA) – Approval in Principle |
| 29/03/2021 | Date of Loan Facility Agreement signed |
| 26/05/2021 | First submission to GS for project listing |
| 30/09/2021 | Commencement of commercial operation |
| 18/10/2021 | Completion of project listing |

All the supporting documents for the above key actions are submitted together with this consultation report.

Eligibility of the project under Gold Standard

>>

The project activity complies with all the eligibility criteria as per section 3.1.1 of GS4GG Principles & Requirements, version 1.2 as follows:

| - Types of Project As per section 4. Eligibility Principles and Requirements of GS4GG Principles & Requirements, version 1.2 |  |
| --- | --- |
| Eligibility Criteria | Compliance by Project Activity |
| A Project type is automatically eligible for Gold Standard Certification if there are approved Gold Standard Activity Requirements and/or Gold Standard Impact Quantification Methodologies associated with it or as referenced in Gold Standard Product Requirements. These are published to the Gold Standard website and shall be followed where provided for a given Project type | The project type is a large-scale wind project which is an eligible project type as per section 2. Eligible Project Types under Renewable Energy Activity Requirements (Version 1.3). The project activity applies CDM methodology ACM0002 - Grid-connected electricity generation from renewable sources (Version 20.0), which is an approved methodology under Gold Standard. |
| For Project types that are not automatically eligible, a Project Developer may submit to Gold Standard for approval. This shall be done at minimum as part of the Preliminary Review, though it is recommended to engage with Gold Standard earlier to establish the criteria and requirements for approval | The project is automatically eligible as per “GS Activity Requirements”. |
| Project types applying for Gold Standard approval are referred to the Gold Standard Vision and Mission. The Project Developer shall demonstrate how the Project would contribute to these and how the Gold Standard for the Global Goals Requirements would be met in their application for approval | Wind project is an approved project type as per “GS Activity Requirements” and does not require approval from Gold Standard. |
| In reviewing a new Project type for approval, Gold Standard may establish new Requirements to be met by the Project in order to achieve Gold Standard Design Certification and ongoing Gold Standard Certified Project status. Where required, Gold Standard shall engage expert peer reviewers to establish these Requirements, at the Project Developer’s expense | Wind project is an approved project type as per “GS Activity Requirements” and does not require approval from Gold Standard. |
| Gold Standard does not support Project types associated with geo-engineering or energy generated from fossil fuel or nuclear, fossil fuel switch, or any project that supports, enhances or prolongs such energy generation. In certain cases, concerning energy efficiency involving fossil fuels (for example, LPG stoves), an exception is made. This is captured in the relevant Activity Requirements, Gold Standard Approved Methodologies and/or Product Requirements | This project activity is not associated with geo-engineering or energy generated from fossil fuel or nuclear, fossil fuel switch. Also, the project activity does not support, enhance or prolong such energy generation. |
| As per section 2. Eligible Project Types & 3. General Eligibility Criteria of “Renewable Energy Activity Requirements”, version 1.3 |  |
| New Gold Standard Verified Emission Reductions (GS VER) or Gold Standard labels for Certified Emission Reductions (GS CER), Renewable Energy projects connected to national or a regional electricity grid must be located in a; - Least Developed Country (LDC), Small Island Developing State (SIDS) or a Land Locked Developing Country (LLDC). - Low Income and Low Middle-income country where the penetration level of the proposed Renewable Energy Technology type is less than 5% of the total grid installed capacity, at the time of the first submission to preliminary review | - Not applicable - The proposed project is located at Viet Nam which is categorized as Low Middle-Income Country as per the “The World Bank” data. The penetration level of wind energy technology is 1.11% (i.e.538 MW) of the total installed capacity (i.e.48,573 MW) of Viet Nam. This is less than 5% as per the requirement. |
| - Certain Impact Quantification methodologies allow projects to account for a Suppressed Demand scenario when establishing a baseline. In such cases, the application of the Suppressed Demand baseline is limited to small scale and microscale projects. | The proposed project doesn’t use Suppressed Demand Baseline. |
| - A single Renewable Energy project may potentially pursue any number and combination of Certified Impact Statements or Products. However, certain Product Requirements, which supersede the generic requirements stated in this document can set restrictions on co-issuance of Certified Impact statements or Products. For instance, GS VERs or GS CERs with REC labels cannot be claimed for the same MWh | The proposed project will claim only GS-VER for the MWh supplied to the national grid. |
| - Location of Project The project activity is located at Quan the 1 village, Phuoc Minh commune, Thuan Nam district, Ninh Thuan province, Viet Nam. The GPS coordinates of each tower is provided under section A.2 below. |  |
| - Project Area, Project Boundary and Scale The project area is 30.8 hectares. As per chosen CDM methodology ACM0002 (version 20.0), The spatial extent of the project boundary includes the project activity and all power plants connected physically to the electricity system that the proposed project is also connected. The detailed on the project boundary is presented in Section B.3 of this PDD. The installed capacity of the project activity is 88 MW so it’s a large-scale project. |  |
| - Host Country Requirements The proposed project activity meets all the legal, environmental, ecological and social regulations through obtaining various approvals & license from authorities such as EPP approval before implementation, Land use, construction & grid connection approvals, Investment related license and so on. Also, the project activity contributes to the national and local level sustainable development as defined under section A.1. |  |
| - Contact Details The contact details of the Project Participants are included under Appendix 2 of this GS PDD. |  |
| - Legal Ownership BIM Wind Power Joint Stock company is the legal owner who has full legal right to control and operate the project activity. All the legal ownership documents such as business registration, investment license, PPA, etc. will be uploaded in sustain-cert for design review. |  |
| - Other Rights None. |  |
| - Official Development Assistance (ODA) Declaration No public funds or ODA is involved in this project activity. |  |

| Eligibility criteria defined under PDD guide |  |
| --- | --- |
| Eligibility Criteria | Compliance by Project Activity |
| Confirm that the project is not registered with any other voluntary or compliance schemes. | The proposed project activity is not registered with any other voluntary or compliance schemes. It has been verified through cross checking the project information against with the other projects from same province registered with any of the other voluntary schemes. |
| Demonstrate the activity is NOT located in a host country, region, locality or state that has an emission reduction cap enforced OR has the possibility to trade emissions that include the scope of the proposed project | The host country of proposed project activity is Vietnam which is a non-annex country. Hence, no emission reduction cap enforced as well as no emission trading system implemented in the host country as per the Decree 06/2022/ND-CP, Article 17, 1c). |
| Demonstrate that no potential for double counting of impacts if the Project Area overlaps with that of another Gold Standard or other voluntary or compliance standard programme of a similar nature. | Since project owner keeps confidential and secured all the project related documents, the same nature of project will be unlikely pursue with any other voluntary or compliance standard programme. Also, project owner will continuously monitor through searching in different registries for similar scope of project registered from same location. The project activity’s documents are clearly defined the technical aspects and location of each WTG tower which are covered under this project scope. Hence, overlapping is unlikely occurred in the proposed project activity. |
| Demonstrate that the project is in compliance with applicable Host Country’s legal, environmental, ecological and social regulations | The proposed project activity is obtaining all the authorities’ approvals to comply with legal, environmental, ecological and social regulations before begin the implementation. |

Legal ownership of products generated by the project and legal rights to alter use of resources required to service the project

>>

BIM Wind Power Joint Stock Company is the legal owner of the proposed project and has legal rights concerning changes in the use of resources required to service the project.

##### A.2 Location of project

>>

The proposed project is located in Quan the 1 village, Phuoc Minh commune, Thuan Nam district, Ninh Thuan province, Vietnam.

The geo-coordinates of WTGs are as follow:

| WTGs | Latitude | Longitude |  | WTGs | Latitude | Longitude |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | 11.42808 | 108.89705 |  | 12 | 11.36809 | 108.88341 |
| 02 | 11.42335 | 108.89862 |  | 13 | 11.42992 | 108.88878 |
| 03 | 11.41843 | 108.90005 |  | 14 | 11.42374 | 108.88870 |
| 04 | 11.41177 | 108.89693 |  | 15 | 11.41279 | 108.88321 |
| 05 | 11.40585 | 108.90154 |  | 16 | 11.40760 | 108.87862 |
| 06 | 11.40599 | 108.89685 |  | 17 | 11.40356 | 108.89021 |
| 07 | 11.4011 | 108.90311 |  | 18 | 11.39760 | 108.88936 |
| 08 | 11.39993 | 108.89673 |  | 19 | 11.38729 | 108.89384 |
| 09 | 11.39377 | 108.89604 |  | 20 | 11.40708 | 108.88453 |
| 10 | 11.38113 | 108.89075 |  | 21 | 11.40085 | 108.88158 |
| 11 | 11.37640 | 108.88635 |  | 22 | 11.40197 | 108.87415 |

##### A.3 Technologies and/or measures

>>

The proposed project activity involves the implementation of 88 MW grid-connected wind power plant with 22 GE158-4MW WTGs to convert wind energy into rotating energy of the blades and converting that rotating energy into electrical energy by the generator, which will be supplied to the national grid through the transmission line.

The electricity supplied to the grid will be monitored through the energy meter installed at switching station. They are digital meters with an accuracy level of Class 0.2 to measure the export and import of electricity of BIM Wind Power Plant.

The technical parameters of the major equipment as follows:

| Parameters | Value | Units |
| --- | --- | --- |
| Turbines model | GE–158- | – |
| Rated power output | 4 | MW |
| Total number of Turbines | 22 | – |
| Hub Height | 120.9 | m |
| Rotor diameter | 158.0 | m |

##### A.4 Scale of the project

>>

As per section 9.1.2 of “GHG Emission Reduction & Sequestration Requirements”, all projects exceeding the small-scale thresholds are defined as large scale. The small-scale threshold is 15 MW(e) for the renewable energy projects in accordance with “CDM project standard for project activities”.

The total installed capacity of the project activity is 88 MW which is higher than 15 MW. Hence, the proposed project is Large Scale.

##### A.5 Funding sources of project

>>

Based on the investment policy approved by people’s committee of Ninh Thuan for the total investment and the loan facility agreement for debt loan, the funding sources for the total investment are as follows:

Equity – 30%

Debt Loan – 70%

The project doesn’t receive any ODA funding.

#### SECTION B. APPLICATION OF APPROVED GOLD STANDARD METHODOLOGY (IES) AND/OR DEMONSTRATION OF SDG CONTRIBUTIONS

##### B.1. Reference of approved methodology (ies)

>>

| Applied Methodology |  |
| --- | --- |
| ACM0002: Grid-connected electricity generation from renewable sources (Version 20.0) |  |
| Related Tools | Applicable to Proposed Project |
| Tool01 – Tool for the demonstration and assessment of additionality (version 07.0.0) | To demonstrate the project additionality as referred in section B.5 |
| Tool07 – Tool to calculate the emission factor for an electricity system (version 07.0) | To define the baseline scenario and grid emission factor as referred in section B.4. |

##### B.2. Applicability of methodology (ies)

>>

| Eligibility Criteria as per ACM0002 | Compliance by Project Activity |
| --- | --- |
| - This methodology is applicable to grid-connected renewable energy power generation project activities that: - Install a Greenfield power plant; - Involve a capacity addition to (an) existing plant(s); - Involve a retrofit of (an) existing operating plants/units; - Involve a rehabilitation of (an) existing plant(s)/unit(s); or - Involve a replacement of (an) existing plant(s)/unit(s). | The proposed project activity is a greenfield grid connected renewable (wind) energy power plant. |
| - The methodology is applicable under the following conditions: - The project activity may include renewable energy power plant/unit of one of the following types: hydro power plant/unit with or without reservoir, wind power plant/unit, geothermal power plant/unit, solar power plant/unit, wave power plant/unit or tidal power plant/unit; - In the case of capacity additions, retrofits, rehabilitations or replacements (except for wind, solar, wave or tidal power capacity addition projects) the existing plant/unit started commercial operation prior to the start of a minimum historical reference period of five years, used for the calculation of baseline emissions and defined in the baseline emission section, and no capacity expansion, retrofit, or rehabilitation of the plant/unit has been undertaken between the start of this minimum historical reference period and the implementation of the project activity. | The proposed project activity is a new installation of wind power plant. It doesn’t involve any capacity additions/ retrofits/ rehabilitations/ replacements the existing plant. |
| - In case of hydro power plants, one of the following conditions shall apply: | Not applicable. Proposed project acidity is a wind power plant. |
| - In the case of integrated hydro power projects, project proponent shall: | Not applicable. Proposed project acidity is a wind power plant. |
| - The methodology is not applicable to: - Project activities that involve switching from fossil fuels to renewable energy sources at the site of the project activity, since in this case the baseline may be the continued use of fossil fuels at the site; - Biomass fired power plants/units. | Not applicable. The proposed project activity neither involve switching from fossil fuels to renewable energy sources nor biomass fired power plant. |
| - In the case of retrofits, rehabilitations, replacements, or capacity additions, this methodology is only applicable if the most plausible baseline scenario, as a result of the identification of baseline scenario, is “the continuation of the current situation, that is to use the power generation equipment that was already in use prior to the implementation of the project activity and undertaking business as usual maintenance”. | The proposed project activity is a new installation of wind power plant. It doesn’t involve any capacity additions/ retrofits/ rehabilitations/ replacements the existing plant. |

##### B.3. Project boundary

>>

As per the chosen methodology ACM0002, the spatial extent of the project boundary includes the BIM Wind Power Plant and all power plants connected physically to the national electricity grid to which the proposed project is also connected.

The flow diagram of the project boundary is shown in Figure below.

WTGs

Transformers

Switching Station with energy meters

National Grid

Fossil Fuel based power plants

Wind flow

GHG Emissions

Project Boundary

| Source |  | GHGs | Included? | Justification/Explanation |
| --- | --- | --- | --- | --- |
| Baseline scenario | CO2 emissions from electricity generation in fossil fuel fired power plants that are displaced due to the project activity | CO2 | Yes | Main emission source |
|  |  | CH4 | No | Minor emission source |
|  |  | N2O | No | Minor emission source |
| Project Activity | For dry or flash steam geothermal power plants, emissions of CH4 and CO2 from non-condensable gases contained in geothermal steam | CO2 | No | The proposed project activity is a wind power plant and thereby all these sources are not applicable. |
|  |  | CH4 | No |  |
|  |  | N2O | No |  |
|  | For binary geothermal power plants, fugitive emissions of CH4 and CO2 from non-condensable gases contained in geothermal steam | CO2 | No |  |
|  |  | CH4 | No |  |
|  |  | N2O | No |  |
|  | For binary geothermal power plants, fugitive emissions of hydrocarbons such as n-butane and isopentane (working fluid) contained in the heat exchangers | CO2 | No |  |
|  |  | CH4 | No |  |
|  |  | N2O | No |  |
|  | CO2 emissions from combustion of fossil fuels for electricity generation in solar thermal power plants and geothermal power plants | CO2 | No |  |
|  |  | CH4 | No |  |
|  |  | N2O | No |  |
|  | For hydro power plants, emissions of CH4 from the reservoir | CO2 | No |  |
|  |  | CH4 | No |  |
|  |  | N2O | No |  |

##### B.4. Establishment and description of baseline scenario

>>

As per chosen methodology ACM0002 (Version 20.0), section 5.2.1, clause 22:

If the project activity is the installation of a Greenfield power plant, the baseline scenario is electricity delivered to the grid by the project activity would have otherwise been generated by the operation of grid-connected power plants and by the addition of new generation sources, as reflected in the combined margin (CM) calculations described in “TOOL07: Tool to calculate the emission factor for an electricity system”.

Since the proposed project activity involves the installation of greenfield grid connect wind power plant, the baseline scenario of the project is to provide an equal amount of electricity provided by the national grid where the proposed project is also connected. EVN is the sole operator of the Viet Nam national electricity grid where all the power plants in Viet Nam are physically connected to and the proposed project activity is not outside of that system.

The combined margin emission factor of the national grid (EFgrid,CM,y) is calculated as per "Tool to calculate the emission factor for an electricity system (Version 07.0)" and used to calculate the baseline emissions of the proposed project activity. Data to calculate EFgrid,CM,y for wind power projects is published (CV 263/BDKH) by Department of Climate Change - Ministry of Natural Resources and Environment on 12/03/2020.

##### B.5. Demonstration of additionality

As per chosen methodology ACM0002 (Version 20.0), “Tool for the demonstration and assessment of additionality (Version 07.0.0)” shall be used to demonstrate the additionality of the proposed project activity step-by-step as follows:

Step 0: Demonstration whether the proposed project activity is the first-of-its-kind

The proposed project is an onshore wind power project located in Vietnam and it’s not a first-of-its-kind.

Step1. Identification of alternatives to the proposed project activity consistent with current laws and regulations

In accordance with the chosen methodology ACM0002 version 20.0, paragraph 22 states:

“if the project activity is the installation of a Greenfield power plant, the baseline scenario is electricity delivered to the grid by the project activity would have otherwise been generated by the operation of grid connected power plant and by the addition of new generation sources.”

Since the baseline scenario is defined by the chosen methodology, no further analysis is required to carry out to identify the plausible alternatives. However, the identification of plausible alternatives has been considered in order to demonstrate the compliance towards the Tool 01:

Sub-step 1a: Define alternatives to the project activity

In the absence of the proposed project activity, plausible and credible project activities to the proposed project activity are as below:

- The proposed project activity not undertaken as a GS-VER project activity

- Continuation of the current situation (no project activity or other alternatives undertaken)

Sub-step 1b: Consistency with mandatory laws and regulations

The above plausible alternatives do not have any obstacles due to national or local laws and regulations since the proposed project is obtaining all the necessary authorities’ approval throughout its implementation and operation, it doesn’t face any difficulties due to mandatory laws and regulations. In terms of the second alternatives, it’s currently under operation and thereby no effort is required. Hence these alternatives are consistent with mandatory laws and regulations.

Since wind projects are green power without polluting the environment, no concern to operate from pollution control board. Second alternative has been chosen as an appropriate baseline alternative for this project activity in line with the methodology. Hence, both alternatives are found to comply with the mandatory laws and regulations taking into account the enforcement of the legislations in the region or country and EB decisions on national and/or sectoral policies and regulations.

Step 2: Investment Analysis

Determine whether the proposed project activity is not:

- The most economically or financially attractive; or

- Economically or financially feasible, without the revenue from the sale of emission reductions.

The following steps are used to determine the proposed project activity is not financially attractive or feasible.

Sub-step 2a: Determine appropriate analysis method

As per “Tool for the demonstration and assessment of additionality” (version 07.0.0), paragraph 32 states:

“If the CDM project activity and the alternatives identified in Step 1 generate no financial or economic benefits other than CDM related income, then apply the simple cost analysis (Option I). Otherwise, use the investment comparison analysis (Option II) or the benchmark analysis (Option III).”

Since the project owner couldn’t obtain the financial data regarding the investment of at least one alternative available, Options I & II are not applicable for the proposed project activity. Hence, the most appropriate financial analysis would be Option III (benchmark analysis) where the returns on equity investment from the project activity is compared with nominal benchmark.

Sub-step 2b: Option III. Apply benchmark analysis

The benchmark analysis approach has been chosen to assess the additionality of the proposed project activity.

Identified Financial/Economic Indicator:

The expected return on equity (Equity IRR) was selected as the financial indicator to assess the financial additionality of the proposed project activity as per Methodological tool for Investment analysis (version 10.0).

Selection of Appropriate Benchmark:

The methodological tool for Investment Analysis, Version 10.0, paragraph 16 states:

“In situations where an investment analysis is carried out in nominal terms and the available IRR benchmarks are in real terms, project participants shall convert the real term values of benchmarks to nominal values by adding the inflation rate. The inflation rate shall be obtained from the inflation forecast of the central bank of the host country for the duration of the crediting period. If this information is not available, the target inflation rate of the central bank shall be used. If this information is also not available, then the average forecasted inflation rate for the host country published by the IMF (International Monetary Fund World Economic Outlook) or the World Bank for the next five years after the start of the project activity shall be used.”

As stated above the nominal benchmark is computed as follows:

Nominal Benchmark = {(1+Real Benchmark) × (1+Inflation Rate)} – 1

Whereas:

| Real Benchmark | : | The default benchmark for Group 1 for Vietnam under Appendix of TOOL27 (Version 10.0) is 12.6% |
| --- | --- | --- |
| Inflation rate | : | The inflation rate of 2019 was obtained from State Bank of Vietnam and used 2.79% |

Nominal Benchmark = {(1+12.6%) × (1+2.79%)} – 1

= {115.74%} – 1

Nominal Benchmark = 15.74%

Sub-step 2c: Calculation and comparison of financial indicators (only applicable to Options II and III)

As per the Methodological Tool: Investment Analysis, Version 09.0, the general issued in calculation and presentation are discussed as follows:

| General issues as per investment analysis tool | Justification |
| --- | --- |
| - The period of assessment should not be limited to the proposed crediting period of the CDM project activity. Both project internal rate of return (IRR) and equity IRR calculations should reflect the period of expected operation of the underlying project activity (technical lifetime) and if a shorter period than the technical lifetime is chosen, the investment analysis shall be conducted for at least 10 years and include the fair value of the project activity assets at the end of the assessment period. The IRR calculation may include the cost of major maintenance and/or rehabilitation if these are expected to be incurred during the period of assessment. | The project activity has chosen 25 years (project lifetime) for the IRR calculation. Fair value is considered as zero in accordance with the following explanation: As per Annex 2 of “GUIDING REGULATION ON MANAGEMENT, USE AND DEPRECIATION OF FIXED ASSETS” issued by Ministry of Finance, Straight-line depreciation method defines: Where, the default value for maximum time of depreciation for wind power assets is 20 years which is mentioned under Annex 1 in the same guideline. Since BIM wind power has been chosen 25 years for computation, no fair value is considered. |
| - The fair value of any project activity assets at the end of the assessment period shall be included as a cash inflow in the final year. The fair value should be calculated in accordance with local accounting regulations where available, or international best practice. It is expected that such fair value calculations will include both the book value of the asset and the reasonable expectation of the potential profit or loss on the realization of the assets. | As referred in the previous justification, the IRR calculation has been used the entire project lifetime of 25 years, zero fair value is considered. |
| - The discount rate used in the investment comparison analysis shall be determined following the requirements as set out in this tool for the calculation of IRR benchmarks in section 6 below. | No discount is considered in the IRR computation. |
| - The weighted average costs of capital (WACC) and the cost of equity provided in the Appendix or calculated using Capital Asset Pricing Model (CAPM) are post-tax IRR benchmarks, and investment analysis shall be conducted with post-tax cash flows. Depreciation, and other non-cash items related to the project activity, which have been deducted in estimating gross profits on which tax is calculated, shall be added back to net profits for the purpose of calculating the financial indicator (e.g. IRR, NPV). The cash flow effects of taxation should be included in the IRR/NPV calculation. | The investment analysis has been conducted with the post-tax cash flows. There are no depreciation and other non-cash items are not considered in the calculation. |
| - Input values used in all investment analysis shall be valid and applicable at the time of the investment decision taken by the project participant. The DOE is therefore expected to validate the timing of the investment decision and the consistency and appropriateness of the input values with this timing. The DOE should also validate that the listed input values have been consistently applied in all calculations. | The input values used in all investment analysis has been valid and applicable at the time of the investment decision taken by the project participant. The date of EPC signed on 29th May 2020 is considered as the investment decision date Therefore, the default benchmark for IRR has been used from Methodological Tool: Investment Analysis, Version 10.0. |
| - In the case of project activities for which implementation ceases after the commencement and where implementation is recommenced due to consideration of the CDM the investment analysis should reflect the economic decision-making context at point of the decision to recommence the project. Therefore, capital costs incurred prior to the revised project activity start date can be reflected as the recoverable value of the assets, which are limited to the potential reuse/resale of tangible assets | This is not applicable as the implementation of project activity is not ceased but delayed due to pandemic outbreak. |
| - Project participants shall supply spreadsheet versions of all investment analysis. All formulas used in this analysis shall be readable and all relevant cells shall be viewable and unprotected. The spreadsheet will be made available to the Board, UNFCCC secretariat and others contracted to assess the request for registration on behalf of the Board including assigned members of the Registration and Issuance Team. In cases where the project participant does not wish to make such a spreadsheet available to the public an exact read-only or PDF copy shall be provided for general publication. In case the project participant wishes to black-out certain elements of the publicly available version, a clear justification for this shall be provided to the secretariat by the DOE when requesting registration. | The spreadsheet is available during the validation of the proposed project activity. |

The values used in investment analysis were valid and applicable at the time of the investment decision taken by the management. The key values used to calculate the return on equity as follows:

Exchange Rate: 1USD = 23,261 VND (Source: State Bank of Vietnam)

| Description of Parameters | Values | Unit | Reference |
| --- | --- | --- | --- |
| Total installed capacity | 88 | MW | Investment Policy |
| Project lifetime | 25 | years | Appendix B of PPA |
| Electricity Generation |  |  |  |
| Annual generation (kWh) | 306 | mil kWh | Energy Yield Assessment Report |
| Annual degradation per year | 0.1% | % |  |
| Electricity Tariff | 0.0850 | USD/kWh | PPA |
| O&M Cost |  |  |  |
| O&M Cost with VAT (Y1 - Y5) | 1.12 | mil USD | O&M Cost in USD |
| O&M Cost with VAT (Y6 - Y8) | 1.69 | mil USD |  |
| O&M Cost with VAT (Y9 - Y15) | 1.93 | mil USD |  |
| O&M Cost without VAT (Y1 - Y5) | 1.02 | mil VND | O&M Contract (Section 7.1 - Fixed Annual Payment) |
| O&M Cost without VAT (Y6 - Y8) | 1.54 | mil VND |  |
| O&M Cost without VAT (Y9 - Y15) | 1.76 | mil VND |  |
| Inflation rate | 3.857% | % | International Monetary Fund (IMF) |
| Insurance |  |  |  |
| Full year premium | 0.323 | mil USD | Policy Endorsement Note |
| Project Costing |  |  |  |
| Total Project cost | 154.54 | mil USD | Investment Policy approved in 12 Mar 2021 |
| Equity Share | 30% | % | Calculated value |
| Debt Share | 70% | % | Calculated value |
| Equity Amount | 46.36 | mil USD | Calculated value |
| Debt (Bank Loan) |  |  |  |
| Debt Amount | 108.50 | mil USD | Project Loan Facility Agreement |
| Interest Rate | 8% | % |  |
| Income Tax |  |  |  |
| VAT | 10% | % | https://moj.gov.vn |
| CIT | 20% | % | https://moj.gov.vn |

Outcome of Investment Analysis:

By considering all the above values, the equity IRR for the proposed project activity as follows:

| Nominal Benchmark (Investment Analysis Tool) | Equity IRR without carbon revenue |
| --- | --- |
| 15.74% | 12.49% |

The equity IRR is estimated based on the project activity’s lifetime of 25 years, cash outflows and cash inflows into the project activity. From the above table, the estimated equity IRR is below than the nominal Benchmark and thereby justifying that the investments are not financially attractive. Therefore, the project activity is an additional and not a Business-As-Usual (BAU).

Sub-step 2d: Sensitivity Analysis

As per the Methodological Tool: Investment Analysis, Version 10.0, paragraph 27 states:

“Variables, including the initial investment cost, that constitute more than 20% of either total project costs or total project revenues should be subjected to reasonable variation (all parameters varied need not necessarily be subjected to both negative and positive variations of the same magnitude), and the results of this variation should be presented in the PDD and be reproducible in the associated spreadsheets.”

The project investment is already fixed, and the electricity tariff has been already committed with national grid for 20 years. Therefore, both values are not subjected to variation. Hence, only power generation and O&M expenses are subjected to reasonable variation. However, following factors have been considered into sensitivity analysis:

- Total project investment

- Electricity tariff

- Power generation

- Operation & Maintenance (O&M) costs

The results of sensitivity analysis are as follows:

| Variations | -20% | -10% | -5% | 0% | 5% | 10% | 20% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Total Investment | 15.83% | 14.04% | 13.24% | 12.49% | 11.78% | 11.11% | 9.86% |
| Electricity Tariff | 2.54% | 7.89% | 10.25% | 12.49% | 14.64% | 16.73% | 20.80% |
| Power Generation | 2.54% | 7.89% | 10.25% | 12.49% | 14.64% | 16.73% | 20.80% |
| O&M Costs | 13.01% | 12.75% | 12.62% | 12.49% | 12.36% | 12.22% | 11.96% |

| Variations | Calculated IRR | Benchmark IRR | Required variation to exceed benchmark |
| --- | --- | --- | --- |
| Total Investment | 12.49% | 15.74% | 20% |
| Electricity Tariff |  |  | 8% |
| Power Generation |  |  | 8% |
| O&M Costs |  |  | 100% (not feasible) |

Total Project Investment

The equity investment by project owner was 46.36 million USD which is 70% of the total project investment cost. To assess the additionality of this project, if equity decreased by 20% (more than 9 mil USD), then equity IRR would exceed the benchmark. As per the contracts/agreements in place, relevant purchase orders are already made, thus the equity investment is impossible to vary in the future.

Electricity Tariff

The electricity tariff is extracted from the Decision No: 39/2018/QD-TTg as referred by the in-principle approval on the Power Purchase Agreement (PPA) signed by EVN (Vietnam Electricity). The equity IRR is exceeding the benchmark when the electricity tariff increased up to 8% which is not possible for next 20 years as per clause 7 of Article 3 of 39/2018/QD-TTg.

Power Generation

Increase in power generation would lead to increase in electricity supply to grid and thereby revenue. The equity IRR is exceeding the benchmark, if the power generation increase 8% which is 24,440 MWh of total estimated power generation 305,500 MWh per annum. The energy yield assessment was conducted based on the on-site wind measurement (covered minimum 12 calendar months period), equipment specification, potential losses and whether condition as per geographical location. Hence, the possibility of increase in power generation is unlikely in the future.

Operation & Maintenance Costs

The analysis shows that the equity IRR would exceed the benchmark, if the O&M costs decrease more than 100% which is not reasonable. Hence, the O&M costs associated with the project has no impact on the financial returns of the equity.

Step 3: Barrier Analysis

Barrier analysis is not considered.

Step 4: Common practice analysis

Common practice analysis has been carried out as per TOOL24: Common Practice, (Version 03.1). As per the methodological tool, the following definitions are explained in order to evaluate the project as per the stepwise approach for common practice:

| Definitions as per methodological tool | Justification |  |
| --- | --- | --- |
| - calculate applicable capacity or output range as +/-50% of the total design capacity or output of the proposed project activity. | +50% installed capacity Installed capacity -50% installed capacity | : 132 MW : 88 MW : 44 MW |
| - identify similar projects (both CDM and non-CDM) which fulfil all of the following conditions: - The projects are located in the applicable geographical area; - The projects apply the same measure as the proposed project activity; - The projects use the same energy source/fuel and feedstock as the proposed project activity, if a technology switch measure is implemented by the proposed project activity; - The plants in which the projects are implemented produce goods or services with comparable quality, properties and applications areas (e.g. clinker) as the proposed project plant; - The capacity or output of the projects is within the applicable capacity or output range calculated in Step 1; - The projects started commercial operation before the project design document (CDM-PDD) is published for global stakeholder consultation or before the start date of proposed project activity, whichever is earlier for the proposed project activity. | Identification of similar projects (CDM & Non-CDM) is carried out as follows: - The applicable geographical area is Vietnam (host country) and thereby all projects located in Vietnam are considered - The proposed project activity is a green-field renewable energy project and uses measure (b) “Switch of technology with or without change of energy source including energy efficiency improvement as well as use of renewable energies”. Therefore, all power generation plants based on renewable energy are considered. - The proposed project activity utilizes wind energy source. Hence, only wind energy-based power plants have been considered. - The project activity produces renewable electricity by using wind energy source; therefore, all wind power plants are considered. - The capacity range of the projects is within the applicable capacity range from 44 to 132 MW - The project start date was 29th May 2020. Numbers of Similar projects (CDM and non-CDM) identified, which fulfil above-mentioned conditioned are: In accordance with table shown below, there was 3 wind power plants identified before the investment decision made on 29th May 2020 for the proposed project activity. |  |
| - within the projects identified in Step 2, identify those that are neither registered CDM project activities, project activities submitted for registration, nor project activities undergoing validation. Note their number Nall. | Within the projects identified in Step 2, no projects are registered as CDM projects. Hence: Nall = 3 |  |
| - within similar projects identified in Step 3, identify those that apply technologies that are different to the technology applied in the proposed project activity. Note their number Ndiff. | As per the definitions provided in Common Practice Analysis tool for the “different technologies”, the project activities have been categorized from the different technologies on the following basis: 12. c) Size of Installation (power capacity)/energy savings Since project activity is large scale project, small and micro scale projects are considered as different technology project. Based on this criterion, there are no different technology project out of the similar identified projects in Step 3. Ndiff = 0 |  |
| - calculate factor F=1-Ndiff/Nall representing the share of similar projects (penetration rate of the measure/ technology) using a measure/ technology similar to the measure/ technology used in the proposed project activity that deliver the same output or capacity as the proposed project activity. | F=1-Ndiff/Nall F = 1- 0/3 = 1 |  |
| As per paragraph 18, The proposed project activity is a “common practice” within a sector in the applicable geographical area if the factor F is greater than 0.2 and Nall-Ndiff is greater than 3. | Since F is 1 which is >0.2 as well as Nall-Ndiff is 3 which is not greater than 3. Hence, the project activity is not a “common practice” in the applicable geographical area. |  |

Following list of wind power plants have been identified as pe the step 2 above:

| S/N | Name | Project Type | Year of Commencement |  | Capacity (MW) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Construction | Operation |  |
| 0 | Proposed Project | On-shore | 2020 | 2021 | 88 |
| 1 | Cầu Đất wind power plant | On-shore | Mar 2019 | Sep 2020 | 50 |
| 2 | Trà Vinh 1 | On-shore | Apr 2019 | Sep 2020 | 48 |
| 3 | Duyên Hải wind farm | On-shore | 2019 | 2020 | 48.3 |

The above explanation/justification clearly shows that the proposed project activity is unlikely to be financially attractive as well as not a common practice in host country; hence the proposed project activity is additional.

B.5.1 Prior Consideration

>>

BIM Wind Power Joint Stock Company is a joint venture between BIM group (Vietnam) and AC Energy Corporation (Philippines). The GS-VER is not only known by BIM but also has always been considered part of BIM’s mandate and business model in any of its current and future projects. This is further evidence by the fact that the BIM has successfully achieved GS-VER certification (GS7525 – BIM Solar Farm) in early 2020 for its existing and fully owned 3 solar plants (325MW + 50MW + 30MW). The revenue from GS-VER will partially cover the O&M costs during operational period. It’s clear that the project owner considered the carbon revenue prior to project implementation.

B.5.2 Ongoing Financial Need

>>

Not applicable

##### B.6. Sustainable Development Goals (SDG) outcomes

Relevant Target/Indicator for each of the three SDGs

| Sustainable Development Goals Targeted | Most relevant SDG Target | SDG Impact |
| --- | --- | --- |
|  |  | Indicator (Proposed or SDG Indicator) |
| 13 Climate Action (mandatory) | Target 13.3: Integrate climate change measures into national policies, strategies grid planning | Indicator 13.2.2. GHG emission reductions per year |
| 8 Decent Work and Economic Growth | Target 8.5: By 2030, achieve full and productive employment and decent work for all women and men, including for young people and persons with disabilities, and equal pay for work of equal value | Indicator 8.5.1 – Average hourly earnings of female and male employees, by occupation, age and persons with disabilities |
| 7 Affordable and Clean Energy | Target 7.2: By 2030, increase substantially the share of renewable energy in the global energy mix | Indicator 7.2.1 – Renewable energy share in the total final energy consumption (MWh of electricity generation) |

B.6.1 Explanation of methodological choices/approaches for estimating the SDG Impact

>>

| SDG | Methodological Approaches | QA/QC |
| --- | --- | --- |
| SDG 13: Climate Action | The emission reduction parameter is calculated through net electricity supplied to grid and grid emission factor. The grid emission factor is published (CV 263/BDKH) by Department of Climate Change - Ministry of Natural Resources and Environment and explained in section B.6.2. This is in line with “Tool to calculate the emission factor for an electricity system, version 7”. The emission reductions are calculated as per the chosen methodology in section B.6.3. | As per the chosen methodology, equations: (11): BEy=EGPj,y×EFgrid,CM,y (17): ERy=BEy×PEy Where: BE = Baseline emission EG = Power generation EF = Grid emission factor ER = Emission reduction PE = Project emission were used to calculate the baseline emission & emission reduction and it is further explained in section B.6.4 as well as used in emission reduction spreadsheet. This parameter is calculated and thereby no QA/QC procedure is required. |
| SDG 8: Decent Work and Economic Growth | The average hourly earnings of female/male employees, by occupation, age and persons with disabilities has been checked through employment contract, payslip and payroll records. Randomly choose employees with designation from payroll records and compare their wages, gender, experience and abilities. | This parameter is not required any QA/QC procedure. The DOE can confirm this parameter with review employee’s records and interview with project owner or employees on site |
| SDG 7: Affordable and Clean Energy | Electricity produced and supplied to the grid is monitored through energy meters. A SCADA (Supervisory control and data acquisition) in place to control and monitor the plant operation. Net electricity will be calculated by project owner on monthly basis and provided in the monthly/quarterly/biyearly/yearly report or equivalent. The other parameters used to reach net electricity supplied to grid are mentioned in monitoring plan. | This parameter is monitored monthly basis and value of parameter will be cross checked with invoices to national grid. The electricity meters will be verified by either EVN or calibrated as per supplier recommendations. |

B.6.2 Data and parameters fixed ex ante

Copy the table for each piece of data and parameter; use headings to group parameter tables by SDG

SDG13

| Data/parameter | EFgrid,OM,y |
| --- | --- |
| Unit | tCO2/MWh |
| Description | Operating Margin CO2 emission factor for the electricity system in year y |
| Source of data | http://dcc.gov.vn/van-ban-phap-luat/1102/Nghien-cuu,-xay-dung-he-so-phat-thai-(EF)-cua-luoi-dien-Viet-Nam-nam-2021-(k%C3%A8m-CV-1278/BDKH-TTBVTOD).html |
| Value(s) applied | 0.9239 |
| Choice of data or Measurement methods and procedures | The latest EFgrid,OM,y for 2021 was calculated and published (1278/BDKH-TTBVTOD) by Department of Climate Change - Ministry of Natural Resources and Environment on 31/12/2022 as per “Tool to calculate the emission factor for an electricity system (Version 07.0)” |
| Purpose of data | To compute combined margin CO2 emission factor for the electricity system |
| Additional comment | None |

SDG13

| Data/parameter | EFgrid,BM,y |
| --- | --- |
| Unit | tCO2/MWh |
| Description | Build Margin CO2 emission factor for the electricity system in year y |
| Source of data | http://dcc.gov.vn/van-ban-phap-luat/1102/Nghien-cuu,-xay-dung-he-so-phat-thai-(EF)-cua-luoi-dien-Viet-Nam-nam-2021-(k%C3%A8m-CV-1278/BDKH-TTBVTOD).html |
| Value(s) applied | 0.5202 |
| Choice of data or Measurement methods and procedures | The latest EFgrid,BM,y for 2021 was calculated and published (1278/BDKH-TTBVTOD) by Department of Climate Change - Ministry of Natural Resources and Environment on 31/12/2022 as per “Tool to calculate the emission factor for an electricity system (Version 07.0)” |
| Purpose of data | To compute combined margin CO2 emission factor for the electricity system |
| Additional comment | None |

SDG13

| Data/parameter | EFgrid,CM,y |
| --- | --- |
| Unit | tCO2/MWh |
| Description | Combined Margin CO2 emission factor for the electricity system in year y |
| Source of data | http://dcc.gov.vn/van-ban-phap-luat/1102/Nghien-cuu,-xay-dung-he-so-phat-thai-(EF)-cua-luoi-dien-Viet-Nam-nam-2021-(k%C3%A8m-CV-1278/BDKH-TTBVTOD).html |
| Value(s) applied | 0.8230 |
| Choice of data or Measurement methods and procedures | The EFgrid,CM,y is calculated by using Operating Margin (EFgrid,OM,y) and Build Margin (EFgrid,BM,y) CO2 emission factors published (1278/BDKH-TTBVTOD) by Department of Climate Change - Ministry of Natural Resources and Environment on 31/12/2022 as per “Tool to calculate the emission factor for an electricity system (Version 07.0)”the computation as follows: - EFgrid,CM,y=EFgrid, OM,y×WOM+EFgrid,BM,y×WBM - Where: - WOM = Weighting of OM emissions factor (%) = 75% WBM= Weighting of BM emissions factor (%) = 25% |
| Purpose of data | To compute baseline emission for greenfield grid connected wind power plants |
| Additional comment | None |

B.6.3 Ex ante estimation of SDG Impact

>>

SDG 7: Affordable and Clean Energy

The baseline outcome benefit is zero.

The project activity generates 305,500 MWh which will be monitored by electricity meters. No ex-ante estimation required for SDG 7.

SDG 8: Decent Work and Economic Growth

The baseline outcome benefit is zero.

The project provides employment 33 local people (11 women & 22 men) at present, and it may vary according to the operation needs throughout project lifetime. This will help to achieve SDG 8 with indicators 8.5.2 “Unemployment rate, by sex, age and persons with disabilities” and following target: 8.5 “By 2030, achieve full and productive employment and decent work for all women and men, including for young people and persons with disabilities, and equal pay for work of equal value”. Number of jobs created and wages for both men and women to demonstrate that the project provides decent work for all women and men and equal pay for equal work. The pay for men and women will be proportional to nature of work which they engaged for and the working hours is similar to all employees. This will be monitored through payroll records and attendance system which is available at the project site.

SDG 13: Climate Action

Baseline Emission:

As per Equation (11) of ACM0002 (Version 20.0), the baseline emissions are to be calculated as follows:

BEy=EGPJ,y×EFgrid,CM,y

Where:

| BEy | = | Baseline emissions in year y (tCO2/yr) |
| --- | --- | --- |
| EGPJ,y | = | Quantity of net electricity generation that is produced and fed into the grid as a result of the implementation of the CDM project activity in year y (MWh/yr) |
| EFgrid,CM,y | = | Combined margin CO2 emission factor for grid connected power generation in year y calculated using the latest version of “TOOL07: Tool to calculate the emission factor for an electricity system” (tCO2/MWh) |

As per Equation (12) of ACM0002 (Version 20.0), the EGPJ,y for the greenfield power plant is calculated as follows:

EGPJ,y=EGfacility,y

| EGfacility,y | = | Quantity of net electricity generation supplied by the project plant/unit to the grid in year y (MWh/yr) |
| --- | --- | --- |

The est. electricity generation by the proposed project (EGPJ,y) = 305,500 MWh/yr

The CM emission factor for the Vietnam’s electricity system (EFgrid,CM,y) = 0.8230 tCO2/MWh.

Hence the baseline is:

| EGPJ,y | × | EFgrid,CM,y | - = | - BEy |
| --- | --- | --- | --- | --- |
| 305,500 MWh/yr | × | 0.8230 tCO2/MWh | - = | - 251,419 tCO2/yr |

Project Emission:

The proposed project activity involves the generation of electricity by development of wind power project. The generation of electricity does not result in greenhouse gas emissions and therefore the project emission (PEy) is zero.

Emission Reduction:

As per Equation (17) of ACM0002 (Verison 20.0), the emission reduction (ERy) of the proposed project as follows:

| BEy | – | PEy | - = | - ERy |
| --- | --- | --- | --- | --- |
| 251,419 tCO2/yr | – | 0 tCO2/yr | - = | - 251,419 tCO2/yr |

B.6.4 Summary of ex ante estimates of each SDG Impact

SDG 7: Affordable and Clean Energy – Ensure access to affordable, reliable, sustainable and modern energy for all

| Year | Baseline estimate (MWh) | Project estimate (MWh) | Net benefit (MWh) |
| --- | --- | --- | --- |
| 2021 (4 months only) | 0 | 101,833 | 101,833 |
| 2022 | 0 | 305,500 | 305,500 |
| 2023 | 0 | 305,500 | 305,500 |
| 2024 | 0 | 305,500 | 305,500 |
| 2026 | 0 | 305,500 | 305,500 |
| 2027 (8 months only) | 0 | 203,667 | 203,667 |
| Total | 0 | 1,527,500 | 1,527,500 |

| Total number of crediting years | 5 |  |  |
| --- | --- | --- | --- |
| Annual average over the crediting period | 0 | 305,500 | 305,500 |

SDG 8: Decent Work and Economic Growth – Promote inclusive and sustainable economic growth, employment and decent work for all

The project leads to employment opportunities which would not have been possible in

the baseline scenario. The project is expected to provide permanent employment to 33 people (includes 11 women & 22 men).

SDG 13: Climate Action – Take urgent action to combat climate change and its impacts

| Year | Baseline estimate (tCO2e) | Project estimate (tCO2e) | Net benefit (tCO2e) |
| --- | --- | --- | --- |
| 2021 (4 months only) | 83,806 | 0 | 83,806 |
| 2022 | 251,419 | 0 | 251,419 |
| 2023 | 251,419 | 0 | 251,419 |
| 2024 | 251,419 | 0 | 251,419 |
| 2026 | 251,419 | 0 | 251,419 |
| 2027 (8 months only) | 167,613 | 0 | 167,613 |
| Total | 1,257,095 | 0 | 1,257,095 |

| Total number of crediting years | 5 |  |  |
| --- | --- | --- | --- |
| Annual average over the crediting period | 251,419 | 0 | 251,419 |

##### B.7. Monitoring plan

B.7.1 Data and parameters to be monitored

SDG 13: Climate Action

| Data / Parameter | ERy - Annual emission reduction |
| --- | --- |
| Unit | tCO2e/year |
| Description | The project will generate renewable energy, which will displace part of the electricity otherwise supplied by fossil fuel fired power plants, thereby the emission reductions are achieved by this project |
| Source of data | Calculated by net electricity generated by the project activity and combined margin emission factor of electricity system. |
| Value(s) applied | 251,419 tCO2e/year |
| Measurement methods and procedures | Computed as recommended by the ACM0002 (Version 20.0) |
| Monitoring frequency | Once for each monitoring period |
| QA/QC procedures | This parameter is cross checked with the electricity supplied to grid |
| Purpose of data | Direct emission reductions of the Project over the Baseline |
| Additional comment | None |

SDG 8: Decent Work and Economic Growth (Indicator 8.5.1 – Average hourly earnings of female and male employees, by occupation, age and persons with disabilities)

| Data / Parameter | Employees’ monthly wages |
| --- | --- |
| Unit | Vietnam Dong per employee |
| Description | Monthly wages to all employees by the project activity |
| Source of data | Job contracts (or) Pay slip (or) Payroll records |
| Value(s) applied | Employees’ wages record available during validation. |
| Measurement methods and procedures | Randomly choose employees with similar designation from payroll records and compare their wages and experience |
| Monitoring frequency | Once for each monitoring period |
| QA/QC procedures | Payroll records, job contracts and staff register will be provided annually. After first verification, only changes in employees will be reported. |
| Purpose of data | To demonstrate contribution to SDG8- 8.5 “By 2030, achieve full and productive employment and decent work for all women and men, including for young people and persons with disabilities, and equal pay for work of equal value’ |
| Additional comment | None |

SDG 7: Affordable and Clean Energy (Indicator 7.2.1: Renewable energy share in the total final energy consumption)

| Data / Parameter | EGPJ,grid,y |
| --- | --- |
| Unit | MWh/yr |
| Description | Net electricity supplied to national grid by the project activity |
| Source of data | Direct measurement by energy meters at point of electricity supplied to the national grid |
| Value(s) applied | 305,500 MWh/year (this value may change in future due to fluctuation in power generation) |
| Measurement methods and procedures | Electricity generated would be monitored by the electricity meters on a continuous basis and readings will be taken every month for billing purpose. The meters have the capability to store data as well as transfer them to server which is located at the control room in switch station. It can be downloaded on a monthly basis for billing purpose. Archive of the data would be available for entire lifetime of the project. |
| Monitoring frequency | Continuous measurement with daily recording (The meters can capture half hourly generation data and the data would be logged through the SCADA system in a daily basis. However, readings will be downloaded on a monthly basis for record keeping and billing to the EVN.) |
| QA/QC procedures | Electricity meters will be calibrated once in three years as per EVN requirement. The electricity generation can also be cross-checked using the invoices billed to EVN EPTC for payment |
| Purpose of data | Calculation of baseline emissions to demonstrate contribution to SDG7’-7.2 By 2030, increase substantially the share of renewable energy in the global energy mix” |
| Additional comment | None |

Safeguarding Principle Assessment

| Data / Parameter | 3. Community Health, Safety and Working Conditions |
| --- | --- |
| Unit | Not applicable |
| Description | The Project shall avoid community exposure to increased health risks and shall not adversely affect the health of the workers and the community. |
| Source of data | Training records (either attendance register or event photos of the training) |
| Value(s) applied | Not applicable |
| Measurement methods and procedures | The training records include attendance registers and photos will be available to measure the level of safety trainings conducted for workers and awareness training provided to community regarding climate change and mitigation measures. |
| Monitoring frequency | Once for each monitoring period |
| QA/QC procedures | Cross checking by interviews |
| Purpose of data | Improve employee’s health and safety |
| Additional comment | None |

| Data / Parameter | 9.4. Release of pollutants – Waste Disposal |
| --- | --- |
| Unit | Not applicable |
| Description | Release of pollutants to the environment |
| Source of data | Onsite waste management (collection & disposal mechanism) process |
| Value(s) applied | Not applicable |
| Measurement methods and procedures | Not Applicable. |
| Monitoring frequency | Not applicable |
| QA/QC procedures | Onsite verification by the plant manager/director to ensure that waste management in place and followed as per the EPP |
| Purpose of data | Not applicable |
| Additional comment | None |

| Data / Parameter | 9.4. Release of pollutants – Noise |
| --- | --- |
| Unit | Not Applicable |
| Description | Noise Pollution |
| Source of data | Nosie level inspection report |
| Value(s) applied | Not applicable |
| Measurement methods and procedures | Not Applicable. |
| Monitoring frequency | Every 6 months |
| QA/QC procedures | Inspection of the noise level of works has been carried out twice a year by third party contractor |
| Purpose of data | Submit the report to environment authority |
| Additional comment | None |

| Data / Parameter | 9.4. Release of pollutants – Shadow Flickering |
| --- | --- |
| Unit | Not applicable |
| Description | Shadow Flickering |
| Source of data | complains/grievances by local stakeholders |
| Value(s) applied | Not applicable |
| Measurement methods and procedures | Not Applicable. |
| Monitoring frequency | Not applicable |
| QA/QC procedures | Monitoring and reporting of any complains/grievances on shadow flicker effect during operational phase |
| Purpose of data | Not applicable |
| Additional comment | None |

| Data / Parameter | Principle 9.10 High Conservation Value Areas and Critical Habitats – Bat Curtailment |
| --- | --- |
| Unit | Not applicable |
| Description | Bat Curtailment |
| Source of data | Manual reading of curtailment periods |
| Value(s) applied | Not applicable |
| Measurement methods and procedures | Not Applicable. |
| Monitoring frequency | Daily |
| QA/QC procedures | As per the Environment Management plan, A Bat Curtailment plan is conducted on a daily basis of all turbines. |
| Purpose of data | Not applicable |
| Additional comment | None |

| Data / Parameter | 9.5. Hazardous and Non-hazardous waste |
| --- | --- |
| Unit | Not applicable |
| Description | Manufacture, trade, release, and/ or use of hazardous and non-hazardous chemicals and/or materials |
| Source of data | Onsite hazardous and non-hazardous waste management (collection & disposal mechanism) process |
| Value(s) applied | Not applicable |
| Measurement methods and procedures | Not Applicable. |
| Monitoring frequency | Not applicable |
| QA/QC procedures | Onsite verification by the plant manager/director to ensure that hazardous and non-hazardous waste management in place and followed as per the EPP |
| Purpose of data | Not applicable |
| Additional comment | None |

B.7.2 Sampling plan

>>

Not applicable.

B.7.3 Other elements of monitoring plan

>>

Monitoring data is collected in accordance with the agreement done between the project owner and Vietnam Electricity Corporation (EVN) which provides the infrastructure for the connection to the national grid. Data will be stored electronically, during the crediting period and at least two years after the last issuance of credits for the solar power project activity in the concerning crediting period. The Project Participant will be responsible for storage of data received from the measuring devices.

Monitoring procedures

The metering flow diagram as follows:

NATIONAL GRID

33kV Bus Bar

33kV Bus Bar

373

375

171

Thuan Nam Substation

Main Power Meter

379

377

331A

277

277

Back-up Power Meter

220kV    Bus Bar

371

381

171

Meters information as follows:

| Meter Number | Serial Number | Calibration Frequency (years) | Last Calibration Month |
| --- | --- | --- | --- |
| 371 | 20036887 | 3 | Mar-21 |
| 373 | 20036888 | 3 | Mar-21 |
| 375 | 20036886 | 3 | Mar-21 |
| 377 | 20036889 | 3 | Mar-21 |
| 379 | 20036885 | 3 | Mar-21 |
| 381 | 21000036 | 3 | Jul-21 |
| 331A | 19058860 | 3 | Feb-21 |
| 271 | 20036711 | 3 | Feb-21 |
| 231 | 20036712 | 3 | Feb-21 |
| 277 Main | 19058842 | 1 | Aug-22 |
| 277 Back-up | 20036713 | 3 | Feb-21 |

All the installed electricity meters have an internal memory that can be stored monitored data. The electricity meters will obtain half hourly data and send to the server located at control room in switch station on a daily basis. The data can be downloaded from the computer at control room. The operator will collate and compile all operational data on a monthly basis and send to plant director. The plant director will verify the data and endorse them with digital signature (registered with EVN) before proceeding the billing to power purchaser.

Period of archiving

The data will be archived electronically every half hour in meters and the meters will send the data to server on a daily basis. Monthly data can be downloaded for billing purposes.

Calibration of equipment

The calibration of electricity meters will be done as per supplier recommendation by an independent accredited third party. However, the tariff meters are validated by EVN as per their requirement in three years interval for back meters and yearly for main meter. The relevant calibration certificates will be made available at the time of verification. The specification of electricity meter is as follows:

| Brand | Code | Rated Current | Rated Voltage | Accuracy class |
| --- | --- | --- | --- | --- |
| Elster | A1700 | 1-2A, 5-6A, and 5-10A | 57.5-415V | 0.2S (Main) & 0.5S (Backup) |

Training requirements

The suppliers of the equipment will train the staff in-charge during erection, to operate and maintain the equipment efficiently. Apart from this, the equipment supplier will provide complete manuals and documentation providing details for the maintenance schedule and the required activities associated with it.

Roles & Responsibilities

The following roles and responsibilities will be used to monitor the project activity as per procedures:

Operation Manager

- Review data

- Submit for endorsement

- Billing after endorsement

Control Room Operator

- Monitoring data

- Submit for review

Top Management

- Endorse reviewed data

Project Coordinator

- Cross check data

- Compute emission reduction

EVN

- Endorse on remotely monitored data

The monitored data will be reported on a yearly basis for the calculation and estimation of emission reductions. This data will be checked against the billing to EVN. If the project is not performing as expected or if there are any negative impacts on the volume of emission reductions obtained, identify the where the project is deviating in its generation of emission reductions and the immediate measures which need to be undertaken to maintain the expected generation of emission reductions from the operation of this project. Should there be any significant changes in plant operation, these will be notified and amendments to the PDD will be requested during the following verification by GS-VVB.

For each verification period, project owner will prepare a monitoring report that will be submitted to a GS-VVB for verification. Project owner ensures that the procedures and monitoring plan are being followed. All data will be kept for a minimum of 2 years following issuance of certified emission reductions or the end of the crediting period, whichever is later, and the storage of this data will be the responsibility of the project owners.

#### SECTION C. DURATION AND CREDITING PERIOD

##### C.1. Duration of project

C.1.1 Start date of project

>>

29 May 2020

C.1.2 Expected operational lifetime of project

>>

25 years

##### C.2. Crediting period of project

C.2.1 Start date of crediting period

>>

01 October 2021 (first crediting period – CP1)

C.2.2 Total length of crediting period

>>

Renewable crediting period with total length of 15 years as follows:

CP1 – 5 years 0 months

CP2 – 5 years 0 months

CP3 – 5 years 0 months

#### SECTION D. SUMMARY OF SAFEGUARDING PRINCIPLES AND GENDER SENSITIVE ASSESSMENT

##### D.1 Safeguarding Principles that will be monitored

A completed Safeguarding Principles Assessment is in Appendix 1, ongoing monitoring is summarized below.

| Principles | Mitigation Measures added to the Monitoring Plan |
| --- | --- |
| Principle 3 – Community Health, Safety and Working Conditions | Refer section B.7.1 |
| 9.4. Release of Pollutants – Waste Disposal |  |
| 9.4. Release of Pollutants – Nosie Pollution |  |
| 9.4. Release of Pollutants – Shadow Flickering |  |
| 9.5 Hazardous and Non-hazardous Waste |  |
| 9.10 High Conservation Value Areas and Critical Habitats – Bat Curtailment |  |

##### D.2. Assessment that project complies with GS4GG Gender Sensitive requirements

| Question 1 - Explain how the project reflects the key issues and requirements of Gender Sensitive design and implementation as outlined in the Gender Policy? | The project owner has prepared the project activity to ensure the contribution to generate livelihood benefits and enable women and men to equitably contribute to the local development through the project implementation. |
| --- | --- |
| Question 2 - Explain how the project aligns with existing country policies, strategies and best practices | The project activity totally complies with the Gender Equality Law approved by the National Assembly in 2006 and the Government’s Decree No. 48/2009/ND-CP providing measures to assure gender equality. |
| Question 3 - Is an Expert required for the Gender Safeguarding Principles & Requirements? | No. The Project Owner has its own internal regulations including their commitments to implement the gender equality. All the GS mandatory requirements of the gender equality and women’s right are met properly. |
| Question 4 - Is an Expert required to assist with Gender issues at the Stakeholder Consultation? | To be confirmed upon completion of stakeholder consultation. Anyhow, project owner is planning 3 different consultation meetings and “women only” meeting one of them. |

#### SECTION E. SUMMARY OF LOCAL STAKEHOLDER CONSULTATION

The below is a summary of the 2 step GS4GG Consultation for monitoring purposes. Please refer to the separate Stakeholder Consultation Report for a complete report on the initial consultation and stakeholder feedback round.

##### E.1 Summary of stakeholder mitigation measures

>>

A total of three different stakeholder consultation meetings have been planned and organized as follows:

| No | Date (hours) | Type of Stakeholders | Number of Attendees |
| --- | --- | --- | --- |
| 1 | 09.05.2022 (0830) | Local community people | 24 |
| 2 | 09.05.2022 (1100) | Women only | 15 |
| 3 | 09.05.2022 (1500) | Government authorities, NGOs & GS | 4 |
| Total |  |  | 43 |

The meeting agenda was established in line with the guidelines of GS4GG as follows:

- Opening of the meeting – Welcome address & self-introductions

- Explanation about the project in non-technical terms

- Question & Answer Session – Clarifications/feedback/opinions on the project

- Potential impacts (positive & negative) on the sustainable development

- Stakeholders’ opinion on safeguarding principles & project’s contributions

- Discussion on monitoring plan of sustainable development impacts

- Discussion of continuous input /grievance mechanism

- Evaluation & Closure of the meeting

The meeting has started with welcome address by the presenter Mr Nguyen Phuong Trung (herein after Trung). Subsequently, presenter has requested to the stakeholders to introduce themselves individually as well as sign the attendance register. The entire presentation was carried out in local language (Vietnamese) by the Mr Trung.

The presentation has begun with the purpose of the meeting, the gold standard process & certification, following by the brief explanation about the Binary Power Plant.

Since the plant already under operation at the time of consultation meeting events, the stakeholders were only informed only on the commercial operation date instead of detailed timetable of project implementation.

Mr Trung has opened the floor to obtain the questions and feedbacks from stakeholders about the project.

After clarified all the questions (refer section iii. Assessment of all comments), the “Continuous input mechanism” was explained and discussed with stakeholders on the suitable way to register their complaints/feedbacks/requests. Many of the stakeholders have chosen the phone call instead of grievance book to register their complaints/feedbacks. Even though they chose to make phone call, Mr Trung has explained to the stakeholders that they will place a grievance book at the office, and it would be regularly checked by either by site staff or the project development team.

Following the grievance mechanism, the sustainable development goals and the impacts created by the project on them were clearly explained to stakeholders. Following that, a short video about the Sustainable Development Goals (SDGs) was played in local language. The positive impact by the project was clearly explained in conjunction with the SDGs. The SDGs monitoring parameters and how they will be monitored also being discussed. Finally, the meeting feedback forms were circulated and closed the meeting in brief. End of the meeting, the contact details of the project owner and the Gold Standard Office were shared with the participants. Local stakeholders were encouraged to give feedback about the project.

No mitigation measures are required as per the comments below:

| Gender of Stakeholder | Stakeholder comment | Was comment taken into account (Yes/ No)? | Explanation/ Justification (Why? How?) |
| --- | --- | --- | --- |
| Male | The design of the protection corridor, when operating and maintaining, ensuring the safety of the people around and the constructions below the turbine? | No | The project has applied strictly according to Vietnam's standards and regulations on the arrangement of turbines, the turbines will be at least 300m away from residential areas to ensure safety for people. |
| Male | What benefits does the project bring to local people, does it support electricity prices for people? | No | The project is located in this area to help create jobs for local people during the construction and operation phase and promote the regional economy. There is no incentive mechanism to support electricity prices for people in areas with renewable energy projects. |
| Female | What about the project’s drainage system? | No | The project's drainage system is also built and improved based on a natural drainage system. The project doesn’t discharge any wastewater and thereby only focused on the rainwater drainage and built the system. |
| Female | How will be lightning protection for the project? | No | The project has been designed with a full range of lightning protection and collection devices to limit the risk of lightning strikes in bad weather. |

##### E.2 Final continuous input / grievance mechanism

| Method | Include all details of Chosen Method (s) so that they may be understood and, where relevant, used by readers. |
| --- | --- |
| Continuous Input / Grievance Expression Process Book (mandatory) | Grievance expression process book will be placed at the site office (Address: Quan the 1 village, Phuoc Minh commune, Thuan Nam district, Ninh Thuan province, Vietnam) which can be access by public. The location and purpose of process book will be clearly explained during the meeting. Project Participant will be respectful to the stakeholders’ views and suggest alternative solutions or compromises whenever possible. |
| GS Contact (mandatory) | help@goldstandard.org |
| Telephone access | Mr Nguyen Phuong Trung (Project Coordinator): +84 936 656 694 This information will be explained at the consultation meeting and provided in the Continuous Input/Grievance Expression Process Book and invitation. |
| Internet/email access | Mr Nguyen Phuong Trung (Project Coordinator): trung.np@bimgroup.com This information will be explained at the consultation meeting and provided in the Continuous Input/Grievance Expression Process Book and invitation. |

Project owner will inform to the stakeholders about the above input & grievance mechanism before the commencement of validation.

The comments received from the stakeholder consultation will be incorporated into SCR and PDD before the validation as well as necessary action will be taken as per the comments received.

### Appendix 1 - Safeguarding Principles Assessment

Complete the Assessment below and copy all Mitigation Measures for each Principle into SECTION D above. Please refer to the instructions in the Guide to Completing this Form.

| Assessment Questions/ Requirements | Justification of Relevance (Yes/ potentially/ no) | How Project will achieve Requirements through design, management or risk mitigation. | Mitigation Measures added to the Monitoring Plan (if required) |
| --- | --- | --- | --- |
| Principle 1. Human Rights |  |  |  |
| - The Project Developer and the Project shall respect internationally proclaimed human rights and shall not be complicit in violence or human rights abuses of any kind as defined in the Universal Declaration of Human Rights - The Project shall not discriminate with regards to participation and inclusion | No | - The Project Proponent and the Project does not conflict with the economic livelihood or other factors of the local communities. Thus, the Project does not cause any human rights abuse and respects internationally proclaimed human rights issue. - As a member of United Nations and part of UN Agreement on Human Rights, it is ensured by law in Vietnam that no action can be taken against human rights. | Not required |
| Principle 2. Gender Equality |  |  |  |
| - The Project shall not directly or indirectly lead to/contribute to adverse impacts on gender equality and/or the situation of women - Projects shall apply the principles of nondiscrimination, equal treatment, and equal pay for equal work - The Project shall refer to the country’s national gender strategy or equivalent national commitment to aid in assessing gender risks - (where required) Summary of opinions and recommendations of an Expert Stakeholder(s) | No | - The project will benefit to local residents regardless of gender and it does not create any adverse effect on the local community. Also, it does not discriminate the local community on basis of gender or caste or religion and therefore equally serve to all. Hence, it will not expose women and girls to further risks or hazards. - The Project has equal opportunity for women and men to contribute both in volunteer and working positions. Project owner has a specified HR policy that considers participation by both men and women. - The project does not involve in any form of discrimination in any kind. Vietnam also ratified relevant ILO core conventions on equality, namely Equal Remuneration Convention (Convention No 100) and Discrimination (Employment and Occupation) Convention (Convention No 111) in 1997. - Not applicable | Not required |
| Principle 3. Community Health, Safety and Working Conditions |  |  |  |
| - The Project shall avoid community exposure to increased health risks and shall not adversely affect the health of the workers and the community | Yes | - The project proponent is committed to the employee’s workplace health & safety during all phases of the project. All employees will attend health & safety trainings. This issue is protected by Labor code and UN Agreement on Human Rights. | Workplace Health & Safety trainings will be conducted regularly during the project operation. This is a monitoring parameter under section B.7.1 |
| Principle 4.1 Sites of Cultural and Historical Heritage |  |  |  |
| Does the Project Area include sites, structures, or objects with historical, cultural, artistic, traditional or religious values or intangible forms of culture? | No | The construction and operation of the project will not be creating any damage, alteration and/or removal to the critical cultural heritage. Law on Cultural heritage is protected against alteration, damage or removal by the “law on cultural heritage”. | Not required |
| >> |  |  |  |
| Principle 4.2 Forced Eviction and Displacement |  |  |  |
| Does the Project require or cause the physical or economic relocation of peoples (temporary or permanent, full or partial)? | No | The project has been established at the salt farm area. Thus, this project does not cause the physical or economic relocation of peoples. The project activity does not involve any alteration of existing roads as well as it does not add additional traffic. Since the site located isolated area, the traffic volume is negligible. Hence, there is no additional burden to the existing traffic. | Not required |
| >> |  |  |  |
| Principle 4.3 Land Tenure and Other Rights |  |  |  |
| - Does the Project require any change, or have any uncertainties related to land tenure arrangements and/or access rights, usage rights or land ownership? - For Projects involving land use tenure, are there any uncertainties with regards to land tenure, access rights, usage rights or land ownership? | No | - BIM Wind Power JSC is legal owner of the land and thereby no land tenure involved. - This is not applicable as the land is fully owned by BIM Wind Power JSC. | Not required |
| >> |  |  |  |
| Principle 4.4 - Indigenous people |  |  |  |
| Are indigenous peoples present in or within the area of influence of the Project and/or is the Project located on land/territory claimed by indigenous peoples? | No | This is not applicable as the land is fully owned by BIM Wind Power JSC. | Not required |
| >> |  |  |  |
| Principle 5. Corruption |  |  |  |
| - The Project shall not involve, be complicit in or inadvertently contribute to or reinforce corruption or corrupt Projects | No | - The project does not involve and is not complicit in any kind of corruption. Vietnam has ratified UN convention against Corruption in 2007. | Not required |
| Principle 6.1 Labour Rights |  |  |  |
| - The Project Developer shall ensure that all employment is in compliance with national labour occupational health and safety laws and with the principles and standards embodied in the ILO fundamental conventions - Workers shall be able to establish and join labour organisations - Working agreements with all individual workers shall be documented and implemented and include: - Working hours (must not exceed 48 hours per week on a regular basis), AND - Duties and tasks, AND - Remuneration (must include provision for payment of overtime), AND - Modalities on health insurance, AND - Modalities on termination of the contract with provision for voluntary resignation by employee, AND - Provision for annual leave of not less than 10 days per year, not including sick and casual leave. - No child labour is allowed (Exceptions for children working on their families’ property requires an Expert Stakeholder opinion) - The Project Developer shall ensure the use of appropriate equipment, training of workers, documentation and reporting of accidents and incidents, and emergency preparedness and response measures | No | - BIM Wind Power JSC and appointed contractors will not involve in any form of forced or compulsory labour. Vietnam has ratified ILO “C0 Forced Labour Convention” - Workers should have the right to establish and join the organization that they consider necessary in a climate of complete security based on Decree No. 8/2014/ND-CP. - The project owner follows regulations of Labour Code of Viet Nam. Thus, they always ensure the participation of women and men in project activities and benefits. The employees of the project are trained on technical aspects relating to the operation of the wind power plant and provided with labour contracts, medical insurance and regular health-check as well as social insurance and unemployment insurance. (Ref. Labour Code). - BIM wind power JSC and their subcontractors complying with all relevant national laws regarding child labor. BIM wind power JSC will not employ children in any shape or form for their works. Vietnam has ratified ILO “C138 – Minimum Age Conventions” and “C182 – Worst Forms of Child Labour Convention” - The project does not require labour force for implementation of the project. Trained technicians are involved in construction and operation and maintenance of plants. | Not required |
| Principle 6.2 Negative Economic Consequences |  |  |  |
| - Does the project cause negative economic consequences during and after project implementation? | No | The project doesn’t cause any negative economic consequences during and after project implementation. | Not required |
| >> |  |  |  |
| Principle 7.1 Emissions |  |  |  |
| Will the Project increase greenhouse gas emissions over the Baseline Scenario? | No | The project will reduce 251,419 tCO2 per annum compared to the baseline scenario through replacing fossil fuel based electricity by renewable energy based electricity which has nil emissions. |  |
| >> |  |  |  |
| Principle 7.2 Energy Supply |  |  |  |
| Will the Project use energy from a local grid or power supply (i.e., not connected to a national or regional grid) or fuel resource (such as wood, biomass) that provides for other local users? | No | The Project’s purpose is to supply clean energy from the wind power plant to the national grid. It does not use energy from a local grid or power supply or fuel resource that provides for other local users. | Not required |
| >> |  |  |  |
| Principle 8.1 Impact on Natural Water Patterns/Flows |  |  |  |
| Will the Project affect the natural or pre-existing pattern of watercourses, ground-water and/or the watershed(s) such as high seasonal flow variability, flooding potential, lack of aquatic connectivity or water scarcity? |  | The proposed project is located on the salt farm, where no population and agriculture lands are located inside and around the project site. It is concluded that the Project will not affect the natural or pre-existing pattern of watercourses and ground-water | Not required |
| >> |  |  |  |
| Principle 8.2 Erosion and/or Water Body Instability |  |  |  |
| - Could the Project directly or indirectly cause additional erosion and/or water body instability or disrupt the natural pattern of erosion? - Is the Project’s area of influence susceptible to excessive erosion and/or water body instability? | No | - During the construction period, topsoil erosion may occur due to the excavation activities. However, proper mitigation measures will be applied. During the operational period, there are no excavation activities and the vegetation cover will be restored. Therefore, the proposed project doesn’t cause any additional erosion and/or water body instability or disrupt the natural pattern of erosion - The risk of erosion is unlikely by the project. | Not required |
| >> |  |  |  |
| Principle 9.1 Landscape Modification and Soil |  |  |  |
| Does the Project involve the use of land and soil for production of crops or other products? | No | The project has been established at the salt farm area which was not used for production of crops and other products for a long time. | Not required |
| >> |  |  |  |
| Principle 9.2 Vulnerability to Natural Disaster |  |  |  |
| Will the Project be susceptible to or lead to increased vulnerability to wind, earthquakes, subsidence, landslides, erosion, flooding, drought or other extreme climatic conditions? | No | The purpose of the project activity is to generate clean energy from wind. Hence, it’s not susceptible to increased vulnerability to wind, earthquakes, subsidence, landslides, erosion, flooding, drought or other extreme conditions. | Not required |
| >> |  |  |  |
| Principle 9.3 Genetic Resources |  |  |  |
| Could the Project be negatively impacted by or involve genetically modified organisms or GMOs (e.g., contamination, collection and/or harvesting, commercial development, or take place in facilities or farms that include GMOs in their processes and production)? | No | The Project’s purpose is to supply clean energy from the wind power plant to the national grid. Hence, the proposed project doesn’t negatively impact by GMOs | Not required |
| >> |  |  |  |
| Principle 9.4 Release of pollutants |  |  |  |
| Could the Project potentially result in the release of pollutants to the environment? | No | The project takes a precautionary approach regarding environmental challenges and is not complicit in practices contrary to the precautionary principle. The environment is protected by several Laws and Regulations in Vietnam. The purpose of the “Law on Environmental Protection” is to protect the environment with principles of sustainable development and environment. The project owner also follows necessary procedures for environmental safety at the project. For Noise: - Arrange working time reasonably, limit material transportation on traffic routes at rush hours, transport vehicles must not exceed the permitted speed, especially when passing through residential areas or at rest hours. In addition, the machinery having a loud noise do not operate after 8 pm. - Regularly maintain equipment, machinery and vehicles, and do not use vehicles and machinery that do not meet environmental standards. - Check the noise level in the construction area to arrange the construction schedule accordingly and reach the allowable noise level. - For noisy equipment, it will be designed to reduce the noise for the machine during operation, periodically maintain the equipment, and replace degraded components. - All vehicles parking at the construction site will turn off the engine. - Limit the simultaneous operation of high noise equipment. - Crank at the height of 2-2.5m to reduce noise. - Equip with noise-canceling tools for workers working in high-noise areas such as using ear cups, earplugs. For Shadow Flickering: - The likelihood of direct line of sight to the location of proposed turbine locations will be assessed visually and the potential for using screening such as higher fencing and planting trees will be explored at problematic houses. - The use of curtains or blinds will also be explored. | Oil & Lubricants disposal: The oil/lubricants are disposed as described in page 79 of EPP (Environmental Protection Plan) as follows: Do not perform oil changes or repairs at the construction site to limit the spillage of engine oils containing harmful ingredients into the water environment. In case of force majeure, the waste machine oils are collected into a container at the site (200 liter drums, with lids, labels, with wheels); store in the temporary hazardous waste storage (built behind the material storage), in accordance with the provisions of Circular 36:2015/TT-BTNMT on management of hazardous waste and hire a unit to collect and transport wastes for disposal. For noise: Inspection of the noise level of works should be carried out at all times; and Reporting of any excessive levels of noise and the measures taken to minimize the impact and prevent from occurring again. For Shadow Flickering: Monitoring and reporting of any complains/grievances on shadow flicker effect during operational phase so as appropriate mitigation measures can be assessed and provided to the sensitive receptors |
| >> |  |  |  |
| Principle 9.5 Hazardous and Non-hazardous Waste |  |  |  |
| Will the Project involve the manufacture, trade, release, and/ or use of hazardous and non-hazardous chemicals and/or materials? | No | As defined in page 82 of EPP: The Hazardous Waste (HW) generated during construction and operation phase will be managed in accordance with Circular 36/2015/TT-BTNMT on management of hazardous wastes. | - Minimize repair of machines in the project area. - The Project Owner carries out the construction of a temporary HW warehouse during the construction period, the warehouse is located on flat ground, near the construction material gathering area, far from surface water sources, an area of about 25 m2. - HWs are collected into closed containers and concentrated in temporary storage. When the volume is significant, the unit will be hired to collect and dispose the wastes. It is planned to arrange 3 HW containers with a capacity of 120 liters each, with labels for each worker base box that is easy to identify and classify, including: 1 container of oil-contaminated rags, sticky gloves, broken fluorescent bulbs; 01 paint tank, paint residue waste; 01 waste oil tank. |
| >> |  |  |  |
| Principle 9.6 Pesticides & Fertilisers |  |  |  |
| Will the Project involve the application of pesticides and/or fertilisers? | No | Not applicable for wind power plants. | Not required |
| >> |  |  |  |
| Principle 9.7 Harvesting of Forests |  |  |  |
| Will the Project involve the harvesting of forests? | No | The project area is salt farm and thereby no harvesting of forests | Not required |
| >> |  |  |  |
| Principle 9.8 Food |  |  |  |
| Does the Project modify the quantity or nutritional quality of food available such as through crop regime alteration or export or economic incentives? | No | The project does not modify the quantity or nutritional quality of food available | Not required |
| >> |  |  |  |
| Principle 9.9 Animal husbandry |  |  |  |
| Will the Project involve animal husbandry? | No | Not applicable for wind project. | Not required |
| >> |  |  |  |
| Principle 9.10 High Conservation Value Areas and Critical Habitats |  |  |  |
| Does the Project physically affect or alter largely intact or High Conservation Value (HCV) ecosystems, critical habitats, landscapes, key biodiversity areas or sites identified? | No | EPP report states that the projected area of the Project is saline soils, not located in sensitive ecological zones, biodiversity conservation areas, and there are no rare and valuable plant and animal species. However, the plant has adopted a BAT Curtailment plan in order to protect the Bat. | As per the Environment Management plan, A Bat Curtailment plan is conducted on a daily basis of all turbines. |
| >> |  |  |  |
| Principle 9.11 Endangered Species |  |  |  |
| a. Are there any endangered species identified as potentially being present within the Project boundary (including those that may route through the area)? - Does the Project potentially impact other areas where endangered species may be present through transboundary affects? | No | - There were no endangered species found in the project boundary as per EPP report. - The project does not impact other areas where endangered species may be present. | Not required |
| >> |  |  |  |

### Appendix 2- Contact information of project participants

| Organization name | BIM Wind Power Joint Stock Company |
| --- | --- |
| Registration number with relevant authority | 4500624821 |
| Street/P.O. Box | Quan the 1 village, Phuoc Minh commune |
| Building | – |
| City | Thuan Nam district |
| State/Region | Ninh Thuan province |
| Postcode | – |
| Country | Viet Nam |
| Telephone | 02593960273 |
| E-mail | – |
| Website | https://bimgroup.com/en |
| Contact person | Nguyen Phuong Trung |
| Title | Project Coordinator |
| Salutation | Mr |
| Last name | Trung |
| Middle name | Phuong |
| First name | Nguyen |
| Department | Renewable Energy |
| Mobile | +84 936 656 694 |
| Direct tel. | – |
| Personal e-mail | trung.np@bimgroup.com |

### Appendix 3- LUF Additional Information

Not Applicable

### Appendix 4-Summary of Approved Design Changes

Not applicable.

##### Revision History

| Version | Date | Remarks |
| --- | --- | --- |
| 1.2 | 14 October 2020 | Hyperlinked section summary to enable quick access to key sections Improved clarity on Key Project Information Inclusion criteria table added Gender sensitive requirements added Prior consideration (1 yr rule) and Ongoing Financial Need added Safeguard Principles Assessment as annex and a new section to include applicable safeguards for clarity Improved Clarity on SDG contribution/SDG Impact term used throughout Clarity on Stakeholder Consultation information required Provision of an accompanying Guide to help the user understand detailed rules and requirements |
| 1.1 | 24 August 2017 | Updated to include section A.8 on ‘gender sensitive’ requirements |
| 1.0 | 10 July 2017 | Initial adoption |

In the beginning, PO was planned to increase up to 99 MW (18 nos. of 5.5 MW) however the approval on the investment policy and PPA limited to 88 MW. The planned capacity of 99 MW has been removed in all sections of the revised PDD. This has been justified through a letter issued by EPC contractor dated on 26 March 2021.

As per “Option (c)” of “TOOL10: Tool to determine the remaining lifetime of the equipment (Version 01)”

https://datahelpdesk.worldbank.org/knowledgebase/articles/906519

VN Prime Minister issued Document 196/TTg-CN on 18 Feb 2021 regarding review the list of wind power projects

EVN Annual Report 2018 (https://en.evn.com.vn/d6/news/-Annual-Report-2018-6-13-2118.aspx)

https://unfccc.int/process/parties-non-party-stakeholders/parties-convention-and-observer-states

https://thuvienphapluat.vn/van-ban/Tai-nguyen-Moi-truong/Nghi-dinh-06-2022-ND-CP-giam-nhe-phat-thai-khi-nha-kinh-va-bao-ve-tang-o-don-500104.aspx

https://en.evn.com.vn/

http://dcc.gov.vn/van-ban-phap-luat/1059/Nghien-cuu,-xay-dung-heso-phat-thai-(EF)-cua-luoi-dien-Viet-Nam-(K%C3%A8m-CV-263/BDKH).html

- Slide 216 of Corporate Finance, Second Edition of Aswath Damodaran & https://en.wikipedia.org/wiki/Nominal_interest_rate

https://www.sbv.gov.vn/webcenter/portal/en/home/sbv/news/Latestnews/Latestnews_chitiet?leftWidth=20%25&showFooter=false&showHeader=false&dDocName=SBV403529&rightWidth=0%25&centerWidth=80%25&_afrLoop=28497701191216466#%40%3F_afrLoop%3D28497701191216466%26centerWidth%3D80%2525%26dDocName%3DSBV403529%26leftWidth%3D20%2525%26rightWidth%3D0%2525%26showFooter%3Dfalse%26showHeader%3Dfalse%26_adf.ctrl-state%3D1dpqjaw9k0_486

- https://www.sbv.gov.vn/TyGia/faces/Aiber.jspx?_afrLoop=33777021381882577&_afrWindowMode=0&_adf.ctrl-state=49df93rbs_106

https://www.imf.org/external/datamapper/PCPIPCH@WEO/WEOWORLD/VNM

https://devi-renewable.com/

https://registry.goldstandard.org/projects/details/1954

http://dcc.gov.vn/van-ban-phap-luat/1059/Nghien-cuu,-xay-dung-heso-phat-thai-(EF)-cua-luoi-dien-Viet-Nam-(K%C3%A8m-CV-263/BDKH).html

- The net power generation is estimated through energy yield assessment report which is done by K2 Management.

- Estimated (for 1st year) by using power simulations variant – Done by ERS Energy Sdn Bhd (Malaysia)

As per the the circular 07_2019_TT-BKHCN.

- Supplier’s Data/specification sheet

Date of Engineering, Procurement & Construction (EPC) contract signed

- https://thuvienphapluat.vn/van-ban/Quyen-dan-su/Tuyen-ngon-quoc-te-nhan-quyen-1948/65774/noi-dung.aspx

- https://www.ilo.org/dyn/normlex/en/f?p=1000:11200:0::NO:11200:P11200_COUNTRY_ID:103004

- https://thuvienphapluat.vn/van-ban/Lao-dong-Tien-luong/Bo-Luat-lao-dong-2012-142187.aspx

- https://www.ohchr.org/EN/Countries/AsiaRegion/Pages/VNIndex.aspx

- https://thuvienphapluat.vn/van-ban/Van-hoa-Xa-hoi/Van-ban-hop-nhat-10-VBHN-VPQH-2013-hop-nhat-Luat-di-san-van-hoa-204826.aspx

- https://treaties.un.org/Pages/ViewDetails.aspx?src=IND&mtdsg_no=XVIII-14&chapter=18&clang=_en#EndDec

- https://www.ilo.org/dyn/normlex/en/f?p=1000:11200:0::NO:11200:P11200_COUNTRY_ID:103004

- https://www.ilo.org/dyn/normlex/en/f?p=1000:11200:0::NO:11200:P11200_COUNTRY_ID:103004

- https://thuvienphapluat.vn/van-ban/Tai-nguyen-Moi-truong/Luat-bao-ve-moi-truong-2014-238636.aspx