Key Project Information & project Design Document (PDD)

###### PUBLICATION DATE  14.10.2020
VERSION  v. 1.2 
RELATED SUPPORT

###### - TEMPLATE GUIDE Key Project Information & Project Design Document v.1.2

This document contains the following Sections

Key Project Information

SECTION A – Description of project

SECTION B - Application of approved Gold Standard Methodology (ies) and/or    demonstration of SDG Contributions

SECTION C – Duration and crediting period

SECTION D – Summary of Safeguarding Principles and Gender Sensitive Assessment

SECTION E – Outcome of Stakeholder Consultations

Appendix 1 – Safeguarding Principles Assessment (mandatory)

Appendix 2 - Contact information of Project participants (mandatory)

Appendix 3 - LUF Additional Information (project specific)

Appendix 3 - Summary of Approved Design Changes (project specific)

### KEY PROJECT INFORMATION

| GS ID of Project | GS11426 |
| --- | --- |
| Title of Project | Installation of Household Biogas plants in Punjab - 1 |
| Time of First Submission Date | 10/11/2021 |
| Date of Design Certification | N/A |
| Version number of the PDD | 2 |
| Completion date of version | 28/02/2022 |
| Project Developer | Greneity Infocom Service Private Limited |
| Project Representative | Greneity Infocom Service Private Limited |
| Project Participants and any communities involved | Greneity Infocom Service Private Limited Dhaneswari Mahila Sewa Sansthan |
| Host Country (ies) | India |
| Activity Requirements applied | Community Services Activities Renewable Energy Activities Land Use and Forestry Activities/Risks & Capacities N/A |
| Scale of the project activity | Micro scale Small Scale Large Scale |
| Other Requirements applied | N/A |
| Methodology (ies) applied and version number | AMS-I.E. Switch from non-renewable biomass for thermal applications by the user - Version 12 |
| Product Requirements applied | GHG Emissions Reduction & Sequestration Renewable Energy Label N/A |
| Project Cycle: | Regular Retroactive |

Table 1 – Estimated Sustainable Development Contributions

| ## Sustainable Development Goals Targeted | ## SDG Impact (defined in B.6.) | ## Estimated Annual Average | ## Units or Products |
| --- | --- | --- | --- |
| ## 13 Climate Action (mandatory) | ## Emission reductions | ## 33,001 | ## tCO2e |
| ## 7 Affordable and Clean Energy | ## Access to affordable and clean energy services | ## 6,468 biogas plant users are expected to access clean energy | ## Number of affected users |
| ## 8 Decent Work and Economic Growth | ## Unemployment rate, by sex, age and persons with disabilities | ## 10 permanent employments, excluding more than 15 contractual employments. | ## Number of employment created. |
| ## 8 Decent Work and Economic Growth | ## Unemployment rate, by sex, age and persons with disabilities | ## 2 trainings per year. | ## Number of trainings provided. |
| ## 3 Good health and well being | ## Improvement in health and decrease in illness | ## 6468 biogas plant users will have improved health conditions. | ## Number of affected users. |

#### SECTION A. DESCRIPTION OF PROJECT

##### A.1 Purpose and general description of project

>> The project activity aims to replace the commonly used inefficient wood fired mud stove technology, with efficient biogas based cook stove, which is clean and sustainable. The project activity involves bundling household biogas plants located in the state of Punjab, India with varying capacities – 4m3 and 6m3. All the 6,468 digesters which are included in the project activity were implemented between 15th November, 2020 and 25th October, 2021.

Each household will utilize the dung of its cows to feed the digester for the production of biogas for domestic purposes. The generated biogas from the plants to be used for the purpose of cooking and other thermal energy needs in households. The thermal energy generated from the project activity replaces the equal amount of thermal energy which otherwise would have been supplied from the woody biomass (fire-wood) based stove technology. Therefore, the project activity replaces firewood for equal thermal energy needs and thereby reduces harmful greenhouse gases in the atmosphere. In addition, the hygienic conditions in the rural areas will be improved by an appropriate disposal of waste. Further, residue from the bio digesters is used as organic fertilizer and will improve soil conditions in rural areas.

Pre project Scenario:

Household survey was conducted to assess the baseline fuel and quantity used. As per the Survey, firewood was the main fuel used to suffice domestic energy needs (cooking). It was sourced from nearby forests and open market. Usage of firewood in inefficient cooking system (conventional system) leads to indoor pollution and land use patterns have been showing a decrease in forest land cover and increase in degraded land. A trend of forests turning into open scrubs has been observed. Degradation of forest lands has exacerbated the already existing problem of desertification. There is a need to maintain adequate forest cover in the state to mitigate climate change effects.

Project Scenario:

The project activity involves bundling of 6,468 household biogas plants located in the state of Punjab, India with varying capacities – 4m3 and 6m3. The project development is performed & maintained by Dhaneshwari Mahila Sewa Sansthan and its supported NGOs. Project activity involves installation of new biogas plant in the households of Punjab and produced biogas will be used in the biogas stoves for thermal energy needs.  Hence, the project activity is a Greenfield project activity.  The size of the bio digesters varies, depending on the number of people and number of cattle available per household.

Residues from the bio digesters are used as organic fertilizer in the garden area and fields of the local people.

Project activity will result in average saving of 33,001 tCO2e/year.

Eligibility of the project under Gold Standard

The project is eligibility under GS4GG as the project activity involves dissemination of household biogas which is eligible under Gold Standard pursuant to clause 3.1.1 of the GS4GG Principles and Requirements document. The eligibility compliance under GS4GG is detailed as follows;

| Eligibility Criteria Category | Eligibility criterion - Required condition | Justification |
| --- | --- | --- |
| 1. Types of Project | Eligible projects shall include physical action/implementation on the ground. Pre-identified eligible project types are identified in the Eligibility Principles and Requirements section. | The project is already implemented since its start date. As per section 3.1.1 (a) of GG4GG Community Services Activity Requirements ver.1.2,” Renewable energy types such as solar (photovoltaic and solar thermal electricity generation), tidal/wave, wind, hydropower, geothermal, waste to energy and renewable biomass that are connected to mini grid3 or off grid solutions for targeted users and/or applications.” Since, project activity includes utilization of cow dung as feed to bio-digesters along with other organic waste to generate biogas Hence, it meets the criterion. Thus, the project technology/measure falls under GG4GG Community Services Activity Requirements. Section 4.1.3 of the GS4GG Principles and Requirements document states following for automatic eligibility for a project “A Project type is automatically eligible for Gold Standard Certification if there are Gold Standard approved Activity Requirements and/or Impact Quantification Methodologies associated with it or it’s referenced in the Gold Standard Product Requirements.” The Gold Standard has published Community Services Activity Requirements; therefore, the project activity falls under the list of Pre-identified eligible project and is automatically eligible for Gold standard certification. |
| 2. Location of Project | Projects may be located in any part of the world. | Location of the project is the state of Punjab, India. |
| 3. Project Area, Project Boundary and Scale | The Project Area and Project Boundary shall be defined. Projects may be developed at any scale although certain rules, requirements and limitations may apply under specific Activity Requirements, Impact Quantification Methodologies and Products Requirements. In order to avoid double counting the Project shall not be included in any other voluntary or compliance standards programme unless approved by Gold Standard (for or compliance standard programme of a similar nature, the Project shall demonstrate that there is no double counting of impacts at design and performance certification (for example use of similar technology or practices through which the potential arises for double counting or misestimation of impacts amongst projects) example through dual certification). Also, if the Project Area overlaps with that of another Gold Standard or other voluntary | The boundary for the project activity in terms of a geographical area is defined as the territorial boundary of the state of Punjab, India. The project falls under waste handling and disposal with emission reductions 33,001 tCO2 with installed energy output of 34.36 MWth (Below the threshold of 45 MWth). Hence, the project falls under small scale projects. To avoid inclusion of any bio-digester which is a part of another registered carbon project/ programme, all the 6468 bio-digesters under this project activity have a unique ID number to uniquely identify the bio-digesters avoiding any double counting and trace its user, later during monitoring and verification. |
| 4. Host Country Requirements | Projects shall be in compliance with applicable Host Country’s legal, environmental, ecological and social regulations. | The project activity is in compliance with the host country’s legal, environmental, ecological & social regulation. |
| 5. Contact Details | As part of the Project Documentation the Project Developer shall provide (i) name and (ii) contact details of all Project Participants; AND in case of an organisation (iii) the legal registration details and (iv) documentation by the governing jurisdiction that proves that the entity is in good standing (defined as being a legal or other appropriate entity registered in or allowed to operate within the required jurisdiction and with no evidence of insolvency or legal/criminal notices placed against it or any of its Directors). Gold Standard retains the right (at its own discretion) to refuse use of the Standard where reputational concerns are highlighted. | Name and Contact details of organisation is given in the Appendix 1. |
| 6. Legal Ownership | Full and uncontested legal ownership of any Products that are generated under Gold Standard Certification, (for example carbon credits) shall be demonstrated. Where such ownership is transferred from project beneficiaries this must be demonstrated transparently and with full, prior and informed consent (FPIC). Note that for certain Project types there is a requirement for full and uncontested legal land title/tenure to be demonstrated. These are contained within specific Activity or Product Requirements. All projects shall immediately report to Gold Standard any land title/tenure disputes arising. | The project activity includes installation of household biogas plants in the rural areas of Punjab. The owners of the biodigester have transferred their all legal rights, interests, credits, entitlements, benefits or allowances arising from or in connection with any greenhouse gas emissions reductions arising from the operation of the digester (Emission Reduction)in the lieu of free operation and maintenance of the plant to Dhaneshwari Mahila Sewa Sansthan. Same is mentioned in the Commissioning certificates co-signed by the respective plant owners. Also, Dhaneswari Mahila Sewa Sansthan has signed agreement with Greneity Infocom Service Pvt Ltd to take all necessary action required for registering the project under suitable Voluntary Standard along with complete assistance during operation and maintenance of the biogas plants included in the project activity. Also feedback has been collected during the physical local stakeholder consultation meeting as well as SFR round for the project activity. |
| 7. Other Rights | As well as legal title and ownership, the Project Developer shall also demonstrate where required uncontested legal rights and/or permissions concerning changes in use of other resources required to service the Project (for example, access rights, water rights etc.). Any known disputes or contested rights must be declared immediately to Gold Standard by the Project Developer and resolved prior to further project implementation in affected areas. | Not Applicable. |
| 8. Official Development Assistance (ODA) Declaration | All Project Developers applying for project activities located in a country named by the OECD Development Assistance Committee’s ODA recipient list and seeking Gold Standard Certification for carbon credits shall declare the Official Development Assistance (ODA) support. The Project Developer shall follow the GHG Emissions Reduction & Sequestration Product Requirements and submit the declaration at the time of Design Certification. | No ODA is involved in the Project activity. A declaration is being submitted. |

Eligibility under Gold Standard Community Services Activity (CSA) Requirements

>>

As per section 3.1.1 of GS4GG Principles & Requirements, Eligibility criteria is defined below:

| Eligibility Criteria Category | Eligibility criterion - Required condition | Justification |
| --- | --- | --- |
| 1. Eligible Project Types | All CSA Projects shall lead to climate change mitigation and/or adaptation by providing or improving access to services/resources at the household or community or institution level. Eligible services include electricity and energy, water and sanitation, waste management, housing, etc. | The project is already implemented since its start date. As per section 3.1.1 (a) of GG4GG Community Services Activity Requirements ver.1.2,” Renewable energy types such as solar (photovoltaic and solar thermal electricity generation), tidal/wave, wind, hydropower, geothermal, waste to energy and renewable biomass that are connected to mini grid3 or off grid solutions for targeted users and/or applications.” Since, project activity includes utilization of cow dung as feed to bio-digesters along with other organic waste to generate biogas Hence, it meets the criterion. |
| 2. GENERAL ELIGIBILITY CRITERIA - Type of project | (a) Renewable energy: Renewable energy types such as solar (photovoltaic and solar thermal electricity generation), tidal/wave, wind, hydropower, geothermal, waste to energy and renewable biomass that are connected to mini grid3 or off grid solutions for targeted users and/or applications. | As per section 3.1.1 (a) of GG4GG Community Services Activity Requirements ver.1.2,” Renewable energy types such as solar (photovoltaic and solar thermal electricity generation), tidal/wave, wind, hydropower, geothermal, waste to energy and renewable biomass that are connected to mini grid3 or off grid solutions for targeted users and/or applications.” Since, project activity includes utilization of cow dung as feed to bio-digesters along with other organic waste to generate biogas Hence, it meets the criterion. |
| 3. GENERAL ELIGIBILITY CRITERIA – Project Area, Boundary and scale | Project Area and Boundary shall be defined in line with the applicable Impact Quantification Methodologies and Product Requirements. | The boundary for the project activity in terms of a geographical area is defined as the territorial boundary of the state of Punjab, India. The project falls under waste handling and disposal with emission reductions 33,001 tCO2 with installed energy output of 34.36 MWth (Below the threshold of 45 MWth). Hence, the project falls under small scale projects. To avoid inclusion of any bio-digester which is a part of another registered carbon project/ programme, all the 6468 bio-digesters under this project activity have a unique ID number to uniquely identify the bio-digesters avoiding any double counting and trace its user, later during monitoring and verification. |
| 4. GENERAL ELIGIBILITY CRITERIA – Legal Ownership | (a) Projects involving the distribution of a large number of devices for services such as heating, cooking, lighting, electricity generation, water treatment technology such as water filter, etc. shall provide a clear description of the ownership of the Products that are generated under Gold Standard Certification all along the investment chain. In line with the FPIC requirement, the proofs that end-users are aware of and willing to give up their rights on Products shall be provided. (b) The transfer of Product ownership shall be discussed during local stakeholder consultations for projects. | The project activity includes installation of household biogas plants in the rural areas of Punjab. The owners of the biodigester have transferred their all legal rights, interests, credits, entitlements, benefits or allowances arising from or in connection with any greenhouse gas emissions reductions arising from the operation of the digester (Emission Reduction)in the lieu of free operation and maintenance of the plant to Dhaneshwari Mahila Sewa Sansthan. Same is mentioned in the Commissioning certificates co-signed by the respective plant owners. Also, Dhaneswari Mahila Sewa Sansthan has signed agreement with Greneity Infocom Service Pvt Ltd to take all necessary action required for registering the project under suitable Voluntary Standard along with complete assistance during operation and maintenance of the biogas plants included in the project activity. Also feedback has been collected during the physical local stakeholder consultation meeting as well as SFR round for the project activity. |

Legal ownership of products generated by the project and legal rights to alter use of resources required to service the project

>> The project activity is the household level biogas plants and the owner of the technology is the particular household using biogas plants. The owners of a digester signed an agreement with Dhaneshwari Mahila Sewa Sansthan by transferring all legal rights, interests, credits, entitlements, benefits or allowances arising from or in connection with any greenhouse gas emissions reductions arising from the operation of the digester (Emission Reduction), and agrees to take all necessary action required to ensure the transfer of those Emission Reductions to the Dhaneswari Mahila Sewa Sansthan or its nominee, including executing any relevant documents.

##### A.2 Location of project

Country: India

State: Punjab

| District | Geo-coordinates |
| --- | --- |
| Amritsar | 31.63°N, 74.87°E |
| Fateh garh sahib | 30.38°N, 76.23°E |
| Taran Taaran | 31.44°N, 74.92°E |
| Hoshiarpur | 31.53°N, 75.92°E |
| Kapurthala | 31.38°N, 75.38°E |
| Patiala | 30.34°N, 76.38°E |
| Rupnagar | 30.97°N, 76.51°E |
| SAS Nagar | 30.69°N, 76.69°E |
| Sangrur | 30.15°N, 75.50°E |

##### A.3 Technologies and/or measures

>> The existing traditional stoves used in the baseline are simple structures made from clay or having stone or metal tripods with poor combustion air supply or flue gas ventilation system i.e. without a grate or a chimney. These stoves use non-renewable biomass (firewood). The target group of the project are households with at least one head of cattle (generally cows or buffalos) who currently use non-renewable woody biomass (firewood) for cooking purposes. The project aims to utilize the methane produced from household biogas digesters to replace the current non-renewable energy sources that is used by households.

The project activity involves bundling of household bio-digesters with Deenbandhu model technology installed in the state of Punjab. The bio-digester models of various sizes (4m3and 6m3). The major feed cattle dung is mixed with water and fed into the plant through the inlet chamber of the plant. This waste is converted into biogas with the help of a special type of anaerobic bacteria. The digested material, which comes out of the plant, is enriched manure.

Deenbandhu Model:

The major feed cow dung and other organic waste material is mixed with water and fed into the plant through the inlet chamber of the plant. This waste is converted into biogas with the help of a special type of anaerobic bacteria. The digested material, which comes out of the plant, is enriched manure.

The main feature of a Deenbandhu biogas plant is the fixed underground digester chamber, constructed with a layer of bricks and an additional layer of cement mortar forming the roof above. Connected to the underground chamber is an inlet tank (labelled on diagram as “Mixing Tank”), through which manure is fed into the plant. The manure then ferments separating the slurry from the methane gas which rises and collects at the top of the digester tank, and is released through the gas outlet pipe. The slurry passes into the outlet tank where it is ejected from the plant and can be used as fertilizer on the field

Figure1: Deenbandhu type Biogas plant

Function

A fixed-dome plant comprises of a closed, dome-shaped digester with an immovable, rigid gas-holder and a displacement pit, also named 'compensation tank'. The gas is stored in the upper part of the digester. When gas production commences, the slurry is displaced into the compensating tank. Gas pressure increases with the volume of gas stored, i.e. with the height difference between the two slurry levels. If there is little gas in the gas-holder, the gas pressure is low.

Digester

The digesters of fixed-dome plants are usually masonry structures, structures of cement and ferro-cement exist. Main parameters for the choice of material are:

- Technical suitability (stability, gas- and liquid tightness);

- availability in the region and transport costs;

- availability of local skills for working with the particular building material.

Fixed-dome Gasholder

A fixed-dome gas-holder in deenbandhu model is a hemispherical digester. The top part of a fixed-dome plant (the gas space) must be gas-tight. Concrete, masonry and cement rendering are not gas-tight. The gas space must therefore be painted with a gas-tight layer (e.g. 'Water-proofer', Latex or synthetic paints). A possibility to reduce the risk of cracking of the gas-holder consists in the construction of a weak-ring in the masonry of the digester. This "ring" is a flexible joint between the lower (water-proof) and the upper (gas-proof) part of the hemispherical structure. It prevents cracks that develop due to the hydrostatic pressure in the lower parts to move into the upper parts of the gas-holder.

The cattle dung otherwise would have been left to decay in open which leads to methane emissions and by utilizing the same for generation of biogas leads to capture of methane and utilize as fuel. The biogas thus generated replaces firewood that otherwise would have been used for cooking and heating purposes. The project is expected to reduce 33,001 tCO2 per annum.

The projects operational life time is expected to be around 20 years.

##### A.4 Scale of the project

>> The project activity is a small scale project activity.  The project falls under waste handling and disposal with emission reductions 33,001 tCO2 with installed energy output of 34.36 MWth (Below the threshold of 45 MWth). Hence, the project falls under small scale projects.

##### A.5 Funding sources of project

>> No public funding from parties included in Annex I to the UNFCCC, is available to the project. No Official Development Assistance involves in the project activity. The end users invest in procuring biogas technology with special assistance given by Dhaneswari Mahila Sewa Sansthan.

#### SECTION B. APPLICATION OF APPROVED GOLD STANDARD METHODOLOGY (IES) AND/OR DEMONSTRATION OF SDG CONTRIBUTIONS

##### B.1. Reference of approved methodology (ies)

>> Methodology: AMS-I.E - Switch from non-renewable biomass for thermal applications by the user.

Version : Version 12.0

Reference: https://cdm.unfccc.int/methodologies/DB/BLVN9ULDP1FRUVS2LYWW6WPYN9W78E

##### B.2. Applicability of methodology (ies)

>> AMS-I. E. "Switch from Non-Renewable Biomass for Thermal Applications by the User" Version 12.0 is applicable to the proposed project because the project meets all applicability criteria. The applicability of the methodology is outlined as below:

| Criteria AMS-I.E. | Applicability |
| --- | --- |
| Project participants4 are able to show that non-renewable biomass has been used in the project region since 31 December 1989, using survey methods or referring to published literature, official reports or statistics. | Survey was conducted to check since when villagers were using firewood. It is evident from the survey that all villagers have been using firewood for more than 35 years hence meets the criteria. |
| In the case that technologies using renewable biomass are used under the project activity, this methodology is applicable where all emissions related to processing of biomass are fully accounted for and biomass is sourced from biomass residues and/or a dedicated plantation of the CDM project activity, meeting the following conditions: (a) For projects that use biomass residues, prior to the implementation of the project activity, the biomass residues have not been collected and used but been left for decay and would, in the absence of the project activity, continue to be left for decay; and (b) For projects that use biomass residues from a production process (e.g. production of sugar or wood panel boards), the implementation of the project does not result in an increase of the processing capacity of raw input (e.g. sugar, rice, logs, etc.) or in other substantial changes (e.g. product change) in this process; and (c) The biomass used by the project facility is not stored for more than one year; and (d) In the case biomass from dedicated plantations are used, the applicability conditions of TOOL16 “Project and leakage emissions from biomass” are satisfied. | Biomass is not used in the project activity. Hence, the conditions are not applicable. |
| For electric cook stoves with integrated renewable energy device or with grid connected renewable energy system employing net metering, project participants shall demonstrate that, on an annual basis, at least 80% of the electricity generated is consumed by the electric cook stoves (i.e. 20% or less of electricity is consumed by other loads connected). | The project does not involve any electric cook stove and hence the condition is not applicable for the project. |
| For electric cook stoves, in all cases under paragraph 2(d) above where back-up diesel generators are used, this methodology is only applicable when no more than 1% of total electricity supply occurs from back up diesel generators on an annual basis. | The project does not involve any electric cook stove and hence the condition is not applicable for the project. |
| Under this methodology, emission reductions cannot be claimed only due to fuel-switch aspect and proposed project activities shall introduce new renewable energy based technologies, i.e. technology switch is also involved. | The project activity involves technology switch from conventional firewood based cooking system to renewable biogas based cooking system. Hence, the criteria are met. |
| Project participants shall describe in the PDD/PoA-DD the proposed method for distribution of project devices and how the double counting of emission reductions has been addressed, for example, using methods such as unique identifications of product and end-user locations (e.g. programme logo), to prevent double counting of emission reductions from the project devices (e.g. between end users, distributors and producers of stoves, producers of renewable energy, producers of processed renewable biomass). | Each of the bio-digesters shall be allocated a unique id against each end users. End user and project implementer shall have an agreement to avoid any double counting. |
| For project activities introducing bio ethanol cook stove, project participants shall demonstrate that the bioethanol cook stove are designed, constructed and operated to the requirements (e.g. with regard to safety) of a relevant national or local standard or comparable literature. Latest guidelines issued by a relevant national authority or an international organization may also be used. | The Project is using biogas digesters and bio- ethanol is not used and safety requirement does not arise here. Hence, the criteria is met. |

The section 3.1.1 of GS4GG Principles & Requirements states that ‘A Project type is automatically eligible for Gold Standard Certification if there are Gold Standard published Activity Requirements and/or Gold Standard Approved Methodologies associated with it or as referenced in Gold Standard Product Requirements’. The GS has published Community Services Activity Requirement (Version 1.2, October 2019) therefore the proposed project falls under the automatic eligibility list of projects. The projects lead to climate change mitigation by providing access to resources (biogas) to households. The project falls under ‘Renewable energy” type- Waste management and handling: Management of animal waste (cattle dung) to deliver biogas.

##### B.3. Project boundary

>> According to AMS-I.E, the project boundary is the physical, geographical site of the use of biomass or the renewable energy. At the unit level, the project boundary is defined by the individual sites and refers to the operation of the biogas digester units at the household level. Table below shows the emission sources that are under the control of the project participants and attributable to biogas digesters.

| Source |  | GHGs | Included? | Justification/Explanation |
| --- | --- | --- | --- | --- |
| Baseline scenario | Firewood | CO2 | Yes | Main emission source. |
|  |  | CH4 | No | Excluded for simplification. |
|  |  | N2O | No | Excluded for simplification. |
|  | Emissions from fossil fuel use for cooking | CO2 | No | Excluded for simplification. |
|  |  | CH4 | No | Excluded for simplification. |
|  |  | N2O | No | Excluded for simplification. |
| Project scenario | Firewood | CO2 | Yes | Project emissions form continued use of fire-wood shall be accounted. |
|  | Emission from digester and biogas cooking stove | CO2 | No | Not required by the methodology |
|  |  | CH4 | No | Not required by the methodology |
|  |  | N2O | No | Not required by the methodology |

##### B.4. Establishment and description of baseline scenario

>> As per “AMS I.E. Switch from non-renewable biomass for thermal applications by the user, Version 12” ‘A baseline scenario would be the use of fossil fuels for meeting similar thermal energy needs’.

The project activity involves the installation of anaerobic bio digesters for the production of biogas which replaces non-renewable biomass, used as a fuel for household cooking purposes. A baseline survey has been conducted on June 2020, to estimate the average firewood consumption prior to project activity at user point following sampling standard as per UNFCCC ‘Sampling and surveys for CDM project activities and programmes of activities’ version 08 and guideline ‘Sampling and surveys for CDM project activities and programmes of activities’ version 04.

The baseline survey was performed by a third party covering various districts of Punjab. A 90% confidence interval and 10% error margin applied for sample calculation.

To understand the cooking method pattern especially with focus on rural areas of Punjab, to establish a baseline with respect to the fuel used for cooking; and to facilitate a clean cooking programme for the state of Punjab. As part of this objective, a primary survey of households was carried out to gauge the extent of the use of non-renewable biomass for cooking in households. The primary survey covers the targeted rural households in the districts of Amritsar, Fatehgarh Sahib, Patiala Roop Nagar, Sangrur and Taran Taaran in the state of Punjab, India.

To understand the level of detail and data points required, a conclusive research methodology, through structured face-to-face interviews, was used to arrive at statistically-valid estimates for the relevant parameters. In consonance with the objective of the study, the following methodology was adopted for carrying out the assignment.

• Preparation of detailed Questionnaire for household survey

• Mobilization of appropriate teams for village and household survey

• Orientation of the team members about the survey objectives

• Collection of primary and secondary data.

• Compilation and analysis of the collected data

Sampling Plan:

To establish a statistically representative sample size, from the total targeted rural households in Punjab, Guidelines for sampling and surveys for CDM project activities and programmes of activities (Ver04.0, CDM-EB67-A06-GUID) issued by UNFCCC, is used. As per the guidelines provided by UNFCCC, for the representative sampling methods, sample size shall be chosen for a 90/10 precision (90% confidence interval and 10% margin of error) for parameter values used to determine emission reductions.

The project activity is envisaging to implement in the districts Amritsar, Fatehgarh Sahib, Hoshiarpur, Kapurthala, Patiala, Rupnagar, Sahibzada Ajit Singh Nagar (Mohali), Sangrur and Tarn Taran of the state Punjab, India. As there are more districts involved, the team opted for simple random sampling. As the project activity is planned to implement in 9 different districts of Punjab; since, the population are homogeneous in nature, a simple random sampling is performed. For this survey, 6 districts which are Amritsar, Fatehgarh Sahib, Patiala, Rupnagar, Sangrur and Tarn Taran are surveyed. These 6 districts cover around 99.8% of the planned biogas plants. The following equation is used for arriving the sample size.

The following equation is used for arriving the sample size.

Where:

n= Sample size

N = Total number of biogas system of type i installed under the project

p = expected proportion

1.645 = represents the 90% confidence required

0.1 = represents the 10% relative precision

Using the above formula, the sample size arrived is around 261.

However, on a conservative note, the project developer, has decided to go for 327 household sampling. The total sample size decided is 327 households.

Baseline assessment:

This study examines the patterns of domestic energy consumption of households in 9 districts of Punjab, India over the period of past three and half decades. Potential Fuels for cooking in the survey area are as follows;

Kerosene: Dependence on kerosene as a primary fuel for cooking or for lighting has decreased in India, and it is used much less for cooking than for lighting. It is evident from the survey , that none of the households utilize Kerosene as the primarily source of energy for cooking owing to is higher price and limited quantity is available is Fair price shop which is not sufficient to meet their energy demands. Prices in the open market shop is very high and hence making it out of the reach of the local population.

LPG: During this survey it was found that Fuel wood has remained the principal component of rural domestic energy. From the survey conducted it is very evident that, In India, the transition to modern fuel is slow and still a large number of households are using traditional fuels for cooking and lighting. So, it is important to understand the household level behavior towards use of different types of traditional and modern fuel.  More than 80 per cent of the households still bank upon the traditional energy sources for cooking i.e. firewood With the slow rate of growth of LPG, the Government of India launched the Pradhan Mantri Ujjwala Yojana (PMUY) in May 2016 to facilitate underprivileged households' access to LPG for cooking (Cabinet Committee on Economic Affairs 2018). The scheme aimed to empower women and safeguard health. But still, LPG has not become the primarily source of fuel because of the following reasons:

•High upfront costs

• High refill costs

•Extended refilling time

•Lack of adequate infrastructure

During the survey it was found, there were lack of LPG distributors within a reasonable distance and therefore can be considered as major problem, as carrying cylinders over long distances (5–11 km) is exhausting, especially in view of the lack of roads in some rural areas are acting as the main barriers for the non-adoption of LPG.

Firewood: It was found during survey firewood provided the main fuel source for home heating, domestic hot water, and food preparation. The forest has been the dominant source for the supply of wood. However, due to increase in demand, fuel wood is also sourced from Trees outside forest, shrubs, dead logs. Being cost effective, relative affordability and easily accessible makes its primary fuel choice.

The baseline survey study concludes that, firewood is the primary source of fuel in targeted rural households in the various districts of Punjab, India.

The kitchen performance test: The kitchen performance test is performed to understand an average consumption of fuelwood usage per household. The kitchen performance test allows the testers to understand the rate of daily fuelwood consumption per household, as they are used in the normal household environment over a period of time. The KPT is a prolonged test conducted with the willing cooperation of the individual families. More than 90% of the approached families are willing to participate in the kitchen performance test. It is done by conducting daily measurement as families use the traditional stove for a period of 3 days. Results are compiled at the end of the test period. The kitchen performance test household data helps to derive the monthly and yearly consumption of biomass usage per household.

Total number of individuals in a household reflects the rationality of the quantity of biomass fuel consumed. From survey it was found that per households has 5 individuals on an average.

For 3 experimental days, it is evidenced that the average frequency for food preparation is 2-3 times a day. The practice of additional preparation of food items/thermal applications viz. snacks, preparation of tea, hot water etc is also been carried out.

Households selected for the experimental purpose were asked to gather the required quantity of biomass fuel of different categories envisaged to be consumed for three experimental days. After that the weight of individual content was taken every morning and evening for 3 days. Further, average of the quantity measured for 3 days is taken to arrive at final value of the firewood consumption per day per household. On an average, the surveyed households were found to consume 14.95 kg fuelwood daily i.e. 5.4 tons of fuelwood annually.

##### B.5. Demonstration of additionality

| Specify the methodology, activity requirement or product requirement that establishes deemed additionality for the proposed project (including the version number and the specific paragraph, if applicable). | The project activity involves the installation of anaerobic bio digesters for the production of biogas which replaces non-renewable biomass, used as a fuel for household cooking purposes. The methodology used is AMS I.E. Switch from non-renewable biomass for thermal applications by the user, Version 12. The project falls under GG4GG Community Services Activity Requirements. As per Annex-B Positive list under ‘GG4GG Community Services Activity Requirements’ the project meets the criteria 3 ‘Project activities solely composed of isolated units where the users of the technology/measure are households or communities or institutions and where each unit results in <= 600 MWh of energy savings per year or <=600 tonnes of emission reductions per year’. |
| --- | --- |
| Describe how the proposed project meets the criteria for deemed additionality. | The emission reduction for an individual unit is far below 600 tonnes of emission reduction per year. Hence, the proposed project activity falls under the ‘Positive List’ and hence under Principle 5 – Financial Additionality & Ongoing Financial Need, the project is considered deemed additional. |

B.5.1 Prior Consideration

>> As per GS4GG rule for retroactive projects, project documents need to submit to GS within one year of the project start date to meet prior consideration. Only biogas plants commissioned from one year prior to GS submission shall be considered.

B.5.2 Ongoing Financial Need

>> As per the clause 3.5.2.2 of GS4GG ‘principle and requirements’, the ongoing financial needs will be demonstrated during the Design Certification Renewal stage.

##### B.6. Sustainable Development Goals (SDG) outcomes

Relevant Target/Indicator for each of the three SDGs

| Sustainable Development Goals Targeted | Most relevant SDG Target | SDG Impact |
| --- | --- | --- |
|  |  | Indicator (Proposed or SDG Indicator) |
| 13 Climate Action (mandatory) | SDG target (13.2)- Integrate climate change measures into national policies, strategies and planning | Quantity of emission reductions achieved from the project activity. |
| 8 Decent Work and Economic Growth | SDG target 8.5.2- Quality of Employment; and Quantitative employment and income generation | Number of Training Programmes provided to the employees and the Maintenance & Service staff. Number of Employees for Maintenance & Service. |
| 7 Affordable and clean energy | 7.1- By 2030, ensure universal access to affordable, reliable and modern energy services | 7.1.2-Proportion of population with primary reliance on clean fuels and technology |
| 3 Good Health and Well Being | 3.9 By 2030, substantially reduce the number of deaths and illnesses from hazardous chemicals and air, water and soil pollution and contamination. | 3.9.1Mortality rate attributed to household and ambient air pollution |

B.6.1 Explanation of methodological choices/approaches for estimating the SDG Impact

>>

As per “AMS I.E- Switch from non-renewable biomass for thermal applications by the user, Version 12, the baseline emissions (BEy) are calculated as:

𝐵𝐸𝑦 = 𝐵𝑦 × 𝑓𝑁𝑅𝐵,𝑦 × 𝑁𝐶𝑉𝑏𝑖𝑜𝑚𝑎𝑠𝑠 ×𝐸𝐹𝑝𝑟𝑜𝑗𝑒𝑐𝑡𝑒𝑑_𝑓𝑜𝑠𝑠𝑖𝑙_𝑓𝑢𝑒𝑙

Where,

𝐵𝐸𝑦 = Baseline emissions during the year y in t CO2e

𝐵𝑦   = Quantity of woody biomass that is substituted or displaced in tonnes

𝑓𝑁𝑅𝐵, = Fraction of woody biomass used in the absence of the project activity in year y that can be established as non-renewable biomass (fNRB)

𝑁𝐶𝑉𝑏𝑖𝑜𝑚𝑎𝑠𝑠 = Net calorific value of the non-renewable woody biomass that is substituted (IPCC default for wood fuel, 0.0156 TJ/tonne)

𝐸𝐹𝑝𝑟𝑜𝑗𝑒𝑐𝑡𝑒𝑑_𝑓𝑜𝑠𝑠𝑖𝑙 𝑓𝑢𝑒𝑙 = Emission factor for the substitution of non-renewable woody biomass by similar consumers. Use a value of 64.4 tCO2/TJ

By is determined by using option (a) paragraph 27 of the methodology as follows:

“Calculated as the product of the number of households multiplied by the estimate of average annual consumption of woody biomass per household that is displaced by the project activity (tonnes/ household/year)”;

𝐵𝑦 =𝑁𝐻𝐻×(𝐵𝐶𝐵𝐿,𝐻𝐻,𝑦−𝐵𝐶𝑃𝐽,𝐻𝐻,𝑦)

Where,

𝑁𝐻𝐻 = Number of households in the project activity, number

𝐵𝐶𝐵𝐿,,𝑦 = Average annual consumption of woody biomass per household before the start of the project activity, tonnes/household/year

𝐵𝐶𝑃𝐽,𝐻𝐻,𝑦 = If it is found that pre-project devices were not completely displaced but continue to be used to some extent, average annual consumption of woody biomass per household in the pre-project devices during the project activity, tonnes/household/year

BCBL,HH,y for the project have been considered based on previous survey and publicly available reports as discussed in above section.

Fraction of woody biomass used in the absence of the project activity in year y that can be established as nonrenewable biomass (fNRB,y) is determined as per methodological tool ‘Calculation of the fraction of nonrenewable biomass’ version 03 as follows:

The fraction of woody biomass that can be established as non-renewable, is:

fNRB =NRB NRB+RB

fNRB = Fraction of non-renewable biomass (fraction or %)

NRB = Quantity of non-renewable biomass (t/yr)

RB = Quantity of renewable biomass (t/yr)

Estimation of consumption of woody biomass (H/Bold,total) is done following paragraph 11 (a) of the tool ‘Official Statistics’.

As per Forest Survey of India report 2011, annual wood fuel consumption in household sector for the state of Punjab = 3,377,000(Ton/year)

As per Forest Survey of India report 2011, non-domestic wood consumption (Annual fuelwood consumption in household sector and consumption of wood in House construction, Furniture and Agriculture) = 32,221,700(ton/year)

As per Forest Survey of India report 2011, Total wood consumption = 35,598,700(ton/year)

Procedure to estimate RB:

Renewable biomass (RB) in the country/region/area is estimated using the equation below:

Where:

MAIforest,i = Mean Annual Increment of woody biomass growth per hectare in subcategory i of forest areas (t/ha/yr).

MAIother,i = Mean Annual Increment of woody biomass growth per hectare in subcategory i of other wooded land areas (t/ha/yr). This value is not considered due to non-availability of data.

Fforest,i = Extent of forest in sub-category i (ha). India state of of forest report, 2019 has been referred for this parameter.

Fother,i = Extent of other wooded land in sub-category i (ha). This is not used following footnote 4 of the tool.

Pforest = Extent of non-accessible area (e.g. protected area where extraction of wood is prohibited, geographically remote area) within forest areas (ha). This parameter is optional and not considered.

Pother = Extent of non-accessible area (e.g. protected area where extraction of wood is prohibited, geographically remote area) within other wooded land areas (ha). This parameter is optional and not considered.

I = Sub-category i of forest areas and other wooded land areas

The value of MAIforest,i is 0.5 for Punjab. This parameter has been referred from the report issued by Ministry of Environment and Forest, Govt. of India.

The value of Fforest,i is 256,000 ha. The value is referred from the India state of of forest report, 2019.

Therefore, Renewable biomass (RB) =  1,28,000

The fraction of woody biomass that can be established as non-renewable, is:

fNRB =NRB NRB+RB

i.e. NRB = (35598700 – 128000) =  3,54,70,700

fNRB =  3,54,70,700 /( 3,54,70,700 +128,000) = 99.64% for a conservative estimation the same is round down to 99%.

Project Emissions (PEy):

As per applied methodology AMS-I.E, version 12, project emissions are accounted for below activities:

- - CO2 emissions from on-site consumption of fossil fuels due to the project activity

- - CO2 emissions from electricity consumption by the project activity

- - Methane emission from solid waste disposal or waste water

- - Project emissions related to cultivation of feedstock

- - Project emissions from transportation

The project activity does not involve any of the above activity and hence, project emissions for the project activity is not applicable. However, while determining By as per equation 3 of the applied methodology, firewood consumed by pre-project devices during the project activity shall be monitored and applied ex-post. This is to be accounted.

Leakage Emissions (LEy):

Leakage emissions (related to the non-renewable woody biomass saved by the project activity shall be assessed based on ex post surveys of users and the areas from which this woody biomass is sourced (using 90/30 precision for a selection of samples). The following potential source of leakage shall be considered: The use/diversion of non-renewable woody biomass saved under the project activity by non-project households/users that previously used renewable energy sources. If this leakage assessment quantifies an increase in the use of non renewable woody biomass used by the non-project households/users that is attributable to the project activity, then By is adjusted to account for the quantified leakage. Alternatively, By is multiplied by a net to gross adjustment factor of 0.95 to account for leakages, in which case surveys are not required.

PP has opted default option, and By shall be adjusted with adjustment factor of 0.95 to account leakage.

Emission reductions:

Emission reductions are to be estimated based on the equation below:

ERy = BEy – PEy – LEy

SDG 8: The employment records and training records will by monitoried and maintained to estimate impact of SDG 8.

SDG 7: Sample survey to be applied to estimate impact of SDG 7.

B.6.2 Data and parameters fixed ex ante

SDG13

| Data/parameter | NHH |
| --- | --- |
| Unit | Number |
| Description | Number of households in the project activity in year y |
| Source of data | Project Proponent’s project database |
| Value(s) applied | 6,468 |
| Choice of data or Measurement methods and procedures | Project database as per the commissioning reports and end user agreements |
| Purpose of data | Estimation of Baseline |
| Additional comment | NIL |

| Data/parameter | BCBL,HH,y |
| --- | --- |
| Unit | tonnes/household/year |
| Description | Average annual consumption of woody biomass per household before the start of the project activity |
| Source of data | Baseline survey |
| Value(s) applied | 5.4 |
| Choice of data or Measurement methods and procedures | To know the baseline fuel/technologies which is the firewood and its consumption pattern, a survey was conducted in the state of Punjab. The survey was conducted in line with the methodology and following UNFCCC sampling standard ‘Sampling and surveys for CDM project activities and programmes of activities’ version 07 and guideline ‘Sampling and surveys for CDM project activities and programmes of activities’ version 04. A 90% confidence interval and 10% error margin applied for sample calculation. |
| Purpose of data | Estimation of Baseline |
| Additional comment | NIL |

| Data/parameter | fNRB,y |
| --- | --- |
| Unit | Percentage |
| Description | Fraction of woody biomass saved by the project activity during year y that can be established as non-renewable biomass |
| Source of data | Calculated as per the requirements of methodological tool - Calculation of the fraction of non-renewable biomass, version 03. |
| Value(s) applied | 99% |
| Choice of data or Measurement methods and procedures | Calculation of the fraction of non-renewable biomass, version 03 (Methodological tool) |
| Purpose of data | Estimation of Baseline |
| Additional comment | NIL |

| Data/parameter | NCVbiomass |
| --- | --- |
| Unit | TJ/tonne |
| Description | Net calorific value of the non-renewable woody biomass that is substituted |
| Source of data | Default value from the methodology, AMS-I.E - Switch from non-renewable biomass for thermal applications by the user, Version 12.0. |
| Value(s) applied | 0.0156 TJ/tonne |
| Choice of data or Measurement methods and procedures | Default value from the methodology, AMS-I.E - Switch from non-renewable biomass for thermal applications by the user, Version 12.0. |
| Purpose of data | Estimation of Baseline |
| Additional comment | NIL |

| Data/parameter | 𝐸𝐹𝑝𝑟𝑜𝑗𝑒𝑐𝑡𝑒𝑑_𝑓𝑜𝑠𝑠𝑖𝑙𝑓𝑢𝑒l |
| --- | --- |
| Unit | tCO2/TJ |
| Description | Emission factor for the substitution of non-renewable woody biomass |
| Source of data | Default value from the methodology, AMS-I.E - Switch from non-renewable biomass for thermal applications by the user, Version 12.0. |
| Value(s) applied | 64.4 tCO2/TJ |
| Choice of data or Measurement methods and procedures | Default value from the methodology, AMS-I.E - Switch from non-renewable biomass for thermal applications by the user, Version 12.0. |
| Purpose of data | Estimation of Baseline |
| Additional comment | NIL |

B.6.3 Ex ante estimation of SDG Impact

>> SDG 13:

Baseline Emissions:

𝐵𝐸𝑦=𝐵𝑦×𝑓𝑁𝑅𝐵,𝑦×𝑁𝐶𝑉𝑏𝑖𝑜𝑚𝑎𝑠𝑠×𝐸𝐹𝑝𝑟𝑜𝑗𝑒𝑐𝑡𝑒𝑑_𝑓𝑜𝑠𝑠𝑖𝑙_𝑓𝑢𝑒𝑙

Where,

𝐵𝑦=𝑁𝐻𝐻×(𝐵𝐶𝐵𝐿,𝐻𝐻,𝑦−𝐵𝐶𝑃𝐽,𝐻𝐻,𝑦)

Total population (NHH)  = 6,468 nos.

Average firewood consumption (BCBL,HH,y) = 5.4 tonne/year

BCPj,HH,y (if it is found that pre-project devices were not completely displaced but continue to be used to some extent, average annual consumption of woody biomass per household in the pre-project devices during the project activity, tonnes/household/year) is considered zero during design certification stage. The same shall be monitored ex-post certification.

𝐵𝑦 = 6,468 × 5.4 = 34927 tonne/year

Fraction of woody biomass that can be established as non-renewable for Punjab (𝑓𝑁𝑅𝐵,) = 99%

BEy would be:

𝐵𝐸𝑦=𝐵𝑦×𝑓𝑁𝑅𝐵,𝑦×𝑁𝐶𝑉𝑏𝑖𝑜𝑚𝑎𝑠𝑠×𝐸𝐹𝑝𝑟𝑜𝑗𝑒𝑐𝑡𝑒𝑑_𝑓𝑜𝑠𝑠𝑖𝑙_𝑓𝑢𝑒𝑙

𝐵𝑦 = 34,927 tonne/year

𝑓𝑁𝑅𝐵, = 99%

𝑁𝐶𝑉𝑏𝑖𝑜𝑚𝑎𝑠𝑠 = 0.0156 TJ/tonne

𝐸𝐹𝑝𝑟𝑜𝑗𝑒𝑐𝑡𝑒𝑑_𝑓𝑜𝑠𝑠𝑖𝑙_𝑓𝑢𝑒𝑙 = 64.4 tCO2/TJ

Therefore, 𝐵𝐸𝑦 = 34,738 tCO2e

Project Emissions (PEy): As explained in above, project emission is zero.

Leakage Emissions: By is multiplied by a net to gross adjustment factor of 0.95 to account for leakages.

Emission reductions = BEy – PEy – LEy

= 34,738  * 0.95

= 33,001 tCO2e

| SDG 7: |  |
| --- | --- |
| Baseline estimation: All users used fire-wood based cooking system which is not clean energy. | 6,468 users are accessed to clean energy sources. This shall be monitored. It is assumed that with 100% operational status all users are accessed to clean energy source. |
| SDG 3: |  |
| Prone to illness due to smoke released from burning of fire-wood. | Reduction in smoke related illness due to use of biogas instead of firewood. Since, biogas is smokeless; 6,468 users are expected to get rid of kitchen smoke related illness. |
| SDG 8: |  |
| Less local employment opportunities for youth. | 10 permanent employments, excluding more than 15 contractual employments. Creates more local employment opportunities. |
| Unskilled local villagers; and less employability. | 2 trainings per year. More trained/skilled youth. |

B.6.4 Summary of ex ante estimates of each SDG Impact

| Year | Baseline estimate | Project estimate | Net benefit (after applying leakage adjustment factor) |
| --- | --- | --- | --- |
| Year 1 | 34,738 | 0 | 33,001 |
| Year 2 | 34,738 | 0 | 33,001 |
| Year 2 | 34,738 | 0 | 33,001 |
| Year 4 | 34,738 | 0 | 33,001 |
| Year 5 | 34,738 | 0 | 33,001 |
| Total | 173,690 | 0 | 165,005 |

| Total number of crediting years | 5 |
| --- | --- |
| Annual average over the crediting period | 34,738 | 0 | 33,001 |

##### B.7. Monitoring plan

B.7.1 Data and parameters to be monitored

Greneity infocom Service Pvt Ltd (hereafter Greneity) facilitates the End User families to set up village level institutions to take care of repairs, maintenance and the social controls/peer support needed to cope with various exigencies that crops up.

Biogas Project Management Unit

A dedicated team is set up within Greneity for management and monitoring of the Biogas Project. This Project Management & Monitoring Unit consist of the following staff:

Biogas Field Workers/RETs:

Biogas field workers oversee the operation & maintenance of biogas units.

Each field worker is in-charge of approximately 500 unit. They report to the Project Manager and their tasks are to:

- Inspect the bio-digesters in periodic intervals to ensure proper functioning of all the plants

- Identify local suppliers of material for village clusters

- Record the working of bio-digesters in specially designed formats and handover the data to manager on a regular basis

- Assist in selection of Volunteers, train and support them

- Contract Masons and attend to major repairs that cannot be handled by the village level institutions

Management system at village level:

Village level Volunteers maintain the Monitoring log Books where the reasons for non-usage are recorded. Every participating village have a Volunteer to monitor usage of biogas units. Volunteer(s)/mason(s) will be responsible for monitoring of usage and is the first to identify dysfunctional units. The Volunteer either prompts the End User to set right a problem or bring it to the notice of the Biogas Technician.

Each Biogas Unit is marked with a unique Identification Number and date of construction.

A Monitoring log Book is maintained for the project activity. If any Biogas Unit is faulty or not functional, the problem report is automatically passed on to Volunteer(s)/mason(s). There is a continuous database maintained of all the Biogas Units not operational on a regular basis where number of non-operational days, and the reasons of non-operation are recorded. Mostly, volunteers are able to handle the problem arises, in case of major issues, Biogas technicians are informed to handle the situation.

Information flow diagram is as follows;

SDG 13

| Data / Parameter | BCPJ,HH,y |
| --- | --- |
| Unit | tonnes/household/year |
| Description | Average annual consumption of woody biomass per household in the pre-project devices during the project activity, if it is found that pre-project devices were not completely displaced but continue to be used to some extent. |
| Source of data | Survey |
| Value(s) applied | To be monitored |
| Measurement methods and procedures | Monitoring shall consist of estimation of all project devices or a representative sample thereof, at least once every two years. |
| Monitoring frequency | At least once in every two years. |
| QA/QC procedures | Third party survey to be conducted following standard sampling approach. |
| Purpose of data | Estimation of Baseline Emissions. |
| Additional comment | Nil |

| Data / Parameter | NHH |
| --- | --- |
| Unit | Number |
| Description | Number of households (biogas system) in the project activity in operational per year |
| Source of data | Survey |
| Value(s) applied | To be monitored |
| Measurement methods and procedures | Monitoring consist of checking of representative sample, to ensure that bio-digesters operating. Calculated as NHH*Py, where NHH is the number of biodigesters installed in the project and Py is Proportion of Bio-digesters operational estimated based on the sample survey. |
| Monitoring frequency | At least once in every two years. |
| QA/QC procedures | Third party survey to be conducted following standard sampling approach |
| Purpose of data | Estimation of Baseline Emissions. |
| Additional comment |  |

SDG 8:

| Data / Parameter | Unemployment rate, by sex, age and persons with disabilities |
| --- | --- |
| Unit | Numbers |
| Description | Provided training for improving employability. Thereby improving Quality of Employment. |
| Source of data | Records of the training programmes, awareness generation activities, photographs, interviews etc. |
| Value(s) applied | 2 |
| Measurement methods and procedures | - |
| Monitoring frequency | Annual |
| QA/QC procedures | Continuation of regular trainings/workshops. |
| Purpose of data | Assessment of contribution towards ‘Safeguarding Principles’ and to monitor the contribution to SDG 8 |
| Additional comment |  |

| Data / Parameter | Quantitative employment and income generation (8.5.2) |
| --- | --- |
| Unit | Numbers |
| Description | Number of people employed for the maintenance and servicing of biodigesters. |
| Source of data | Attendance Sheet, employment records, employment contracts maintained by Project Developer |
| Value(s) applied | 10 permanent employment |
| Measurement methods and procedures | Manually by Project Developer |
| Monitoring frequency | Annual |
| QA/QC procedures | Income generation to be enhanced by creating job opportunities through training and capacity building. |
| Purpose of data | To monitor the contribution to SDG 8 |
| Additional comment |  |

SDG 7

| Data / Parameter | Access to affordable and clean energy services (7.1.2) |
| --- | --- |
| Unit | Numbers |
| Description | Number of biogas system operational under the project activity |
| Source of data | Biogas User Survey |
| Value(s) applied | To be monitored |
| Measurement methods and procedures | Sample survey to confirm if project biogas systems are operational. Operational status will confirm that the users are accessed to affordable and clean energy |
| Monitoring frequency | At least once in two years |
| QA/QC procedures | Required sample size shall be determined following UNFCCC sampling standard |
| Purpose of data | To monitor the contribution to SDG 7 |
| Additional comment | Nil |

B.7.2 Sampling plan

>>

A third-party sample survey has performed. The survey is conducted in line with the methodology and following UNFCCC sampling standard ‘Sampling and surveys for CDM project activities and programmes of activities’ version 08 and guideline ‘Sampling and surveys for CDM project activities and programmes of activities’ version 04. A 90% confidence interval and 10% error margin applied for sample calculation.

B.7.3 Other elements of monitoring plan

>>N/A

#### SECTION C. DURATION AND CREDITING PERIOD

##### C.1. Duration of project

C.1.1 Start date of project

>> 15th November,2020

C.1.2 Expected operational lifetime of project

>> 20 Years

##### C.2. Crediting period of project

C.2.1 Start date of crediting period

>>15/11/2020 or two years prior to the date of project design certification whichever

is later.

C.2.2 Total length of crediting period

>>The project falls under GS community service activity and hence eligible for total 15 years issuance with two times certification renewal every 5 years. So the length of crediting period will be 15 years.

#### SECTION D. SUMMARY OF SAFEGUARDING PRINCIPLES AND GENDER SENSITIVE ASSESSMENT

##### D.1 Safeguarding Principles that will be monitored

A completed Safeguarding Principles Assessment is in Appendix 1, ongoing monitoring is summarized below.

| Principles | Mitigation Measures added to the Monitoring Plan |
| --- | --- |
| Not applicable | No safeguarding principle is adversely impacted by the project activity. |

##### D.2. Assessment that project complies with GS4GG Gender Sensitive requirements

| Question 1 - Explain how the project reflects the key issues and requirements of Gender Sensitive design and implementation as outlined in the Gender Policy? | The project is aimed to replace conventional wood fired mud stoves technology with biogas primarily for cooking. This will result in reducing use of firewood consumption or replacement of complete firewood usage at user place. Primarily in rural areas of India, cooking activity at household level is managed by women. Therefore, women are more exposed to the indoor air pollution and the associated hazard. Women in rural areas in most cases are also responsible for taking care of their children specially infants who need mother’s support most of the time are bound to accompany their mother in kitchen. This situation leads to enhanced exposure of the women and children to kitchen smoke and associated health consequences. Since the project aims to replace the polluting traditional cooking stoves with biogas cooking system, the primary beneficiary would be the women and children. Furthermore, the project is focused to the economically disadvantaged group of people, which also justifies the dimension of social inclusion in the project design. |
| --- | --- |
| Question 2 - Explain how the project aligns with existing country policies, strategies and best practices | Ministry of women & child development, Govt. of India has taken various measures for gender equality/socio-economic development/empowerment of women. Out of these, the project positively contributes towards the national mission for empowerment of women through improvement of health and attaining vision for empowerment of wowen under national policy for women 2016 (Women participation will be ensured in the efficient use and spreading the use of solar energy, biogas, smokeless chulas and other technological applications to have positive influence on their life styles and a long term impact on meeting sustainable development goals). |
| Question 3 - Is an Expert required for the Gender Safeguarding Principles & Requirements? | N/A |
| Question 4 - Is an Expert required to assist with Gender issues at the Stakeholder Consultation? | N/A |

#### SECTION E. SUMMARY OF LOCAL STAKEHOLDER CONSULTATION

The below is a summary of the 2 step GS4GG Consultation for monitoring purposes. Please refer to the separate Stakeholder Consultation Report for a complete report on the initial consultation and stakeholder feedback round.

##### E.1 Summary of stakeholder mitigation measures

Greneity and its partner NGOs invited all the relevant stake holders like Gold standard supporting NGOs, DNA, local Government officials, elected representatives and the villagers for this meeting. The local stake holder consultation meeting was successfully held on 12/01/2022 at Community center, Near SBI Bank, Village Dhotian, Taran Taaran, Punjab, India

The project idea and its components were thoroughly discussed, feedback from the participants has been documented and sustainable development aspects were discussed during the proceedings. Participants got clarification on many of the project aspects.

Mr. Ajmer Singh, Sarpanch, explained about the biogas programme of the Government and its limitations and implementation style. He stated that the present project is much useful to the farming community in general and women in particular. Further, the positive impact on the women health by using the clean fuel technology is also explained.

Mr. Jasbeer Singh, RET , introduced the present project to the participants and clarified many of their queries. The RET, also explained the transfer of ownership of Carbon credits, to provide free operation and maintenance of the biogas plant. Staff members of the Dhaneswari Mahilla Sewa Sansthan recorded attendees’ details in a special register. Further, he explained how this Project activity is supporting the installation of biogas plants to rural households and provide participating households with clean, convenient and efficient means for cooking and heating purposes. He also provided insights on how the monitoring of installed domestic biogas units could lead to positive sustainable development goals.

They had distributed feedback forms among the participants and collected once they were duly filled. The minutes of the meeting were recorded and meeting photos were taken to document the event.

The following were the sample questions raised in the meeting:

How long the project will be monitored and maintained?

Basic qualification required to be technician?

The following were the sample comments from the feedback forms:

1. The meeting is well organised and very informative.

2. The project is a really needed and useful for both farmers and women.

3. There is nothing to dislike in the project.

The participants overwhelmingly supported the project and No negative comments were raised. Stakeholder feedback round was initiated on 18/01/2022 till 19/03/2022. However, no comments were received during this period.

##### E.2 Final continuous input / grievance mechanism

| Method | Include all details of Chosen Method (s) so that they may be understood and, where relevant, used by readers. |
| --- | --- |
| Continuous Input / Grievance Expression Process Book (mandatory) | The project details and grievance logbook has been maintained in the panchayat office. |
| GS Contact (mandatory) | help@goldstandard.org |
| Other | shivani.garg@greneity.com |

### Appendix 1 - Safeguarding Principles Assessment

Complete the Assessment below and copy all Mitigation Measures for each Principle into SECTION D above. Please refer to the instructions in the Guide to Completing this Form.

| Assessment Questions/ Requirements | Justification of Relevance (Yes/potentially/no) | How Project will achieve Requirements through design, management or risk mitigation. | Mitigation Measures added to the Monitoring Plan (if required) |
| --- | --- | --- | --- |
| Principle 1. Human Rights |  |  |  |
| - The Project Developer and the Project shall respect internationally proclaimed human rights and shall not be complicit in violence or human rights abuses of any kind as defined in the Universal Declaration of Human Rights - The Project shall not discriminate with regards to participation and inclusion | No | India adopted ‘The protection of human rights Act’ 1993 and the project is bound to follow the rules and regulation of host country. In addition, India has ratified ‘International Convention on the Elimination of All Forms of Racial Discrimination :1969’ ‘International Covenant on Civil and Political Rights :1976’, ‘International Covenant on Economic, Social and Cultural Rights :1976’ ‘Convention on the Elimination of All Forms of Discrimination against Women :1981’. Therefore, the project developer and the project do respect nationally and internationally proclaimed human rights and shall not be complicit in violence or human rights abuses of any kind. | Not Applicable |
| Principle 2. Gender Equality |  |  |  |
| - The Project shall not directly or indirectly lead to/contribute to adverse impacts on gender equality and/or the situation of women - Projects shall apply the principles of nondiscrimination, equal treatment, and equal pay for equal work - The Project shall refer to the country’s national gender strategy or equivalent national commitment to aid in assessing gender risks - (where required) Summary of opinions and recommendations of an Expert Stakeholder(s) | No | 1. The project is implemented at households. It does not involve any women workforce which may lead to sexual harassment. 2. Yes. The project involves construction of biogas digesters at households. Trained labors are used for the same. Local people are engaged for the same. No discrimination either in gender or any other form is followed to engage local people. 3. The project does not have any scope to apply gender strategy as such. Although the project positively contributes towards the national mission for empowerment of women through improvement of health and attaining vision for empowerment of women under national policy for women 2016 | Not Applicable. |
| Principle 3. Community Health, Safety and Working Conditions |  |  |  |
| - The Project shall avoid community exposure to increased health risks and shall not adversely affect the health of the workers and the community | No | The project leads to safe working condition and improvement in health as it will replace firewood as fuel with biogas which is clean and safe. | Not applicable |
| Principle 4.1 Sites of Cultural and Historical Heritage |  |  |  |
| Does the Project Area include sites, structures, or objects with historical, cultural, artistic, traditional or religious values or intangible forms of culture? | No | The project area covers households which does not have any structures, or objects with historical, cultural, artistic, traditional or religious values or intangible forms of culture. Hence, not applicable. | Not applicable |
| >> |  |  |  |
| Principle 4.2 Forced Eviction and Displacement |  |  |  |
| Does the Project require or cause the physical or economic relocation of peoples (temporary or permanent, full or partial)? | No | The project activity takes place within individual households. Therefore, no relocation of people takes place. | Not applicable |
| >> |  |  |  |
| Principle 4.3 Land Tenure and Other Rights |  |  |  |
| - Does the Project require any change, or have any uncertainties related to land tenure arrangements and/or access rights, usage rights or land ownership? - For Projects involving land use tenure, are there any uncertainties with regards to land tenure, access rights, usage rights or land ownership? | No | The project area covers households which does not have any structures, or objects with historical, cultural, artistic, traditional or religious values or intangible forms of culture. Hence, not applicable | Not applicable |
| >> |  |  |  |
| Principle 4.4 - Indigenous people |  |  |  |
| Are indigenous peoples present in or within the area of influence of the Project and/or is the Project located on land/territory claimed by indigenous peoples? | No | No. The project involves household biogas digesters. Therefore, it does not involve any influence towards indigenous people | Not applicable |
| >> |  |  |  |
| Principle 5. Corruption |  |  |  |
| - The Project shall not involve, be complicit in or inadvertently contribute to or reinforce corruption or corrupt Projects | No | The project benefits households with clean fuel (biogas). There is no corruption provision in the project activity. | Not applicable |
| Principle 6.1 Labour Rights |  |  |  |
| - The Project Developer shall ensure that all employment is in compliance with national labour occupational health and safety laws and with the principles and standards embodied in the ILO fundamental conventions - Workers shall be able to establish and join labour organisations - Working agreements with all individual workers shall be documented and implemented and include: - Working hours (must not exceed 48 hours per week on a regular basis), AND - Duties and tasks, AND - Remuneration (must include provision for payment of overtime), AND - Modalities on health insurance, AND - Modalities on termination of the contract with provision for voluntary resignation by employee, AND - Provision for annual leave of not less than 10 days per year, not including sick and casual leave. - No child labour is allowed (Exceptions for children working on their families’ property requires an Expert Stakeholder opinion) - The Project Developer shall ensure the use of appropriate equipment, training of workers, documentation and reporting of accidents and incidents, and emergency preparedness and response measures | No | The project does not require labour force for implementation of the project. Trained technicians are involved in construction and operation and maintenance of plants. Therefore, no forced labour is involved in the project. No child labour is involved. | Not applicable |
| Principle 6.2 Negative Economic Consequences |  |  |  |
| - Does the project cause negative economic consequences during and after project implementation? | No | The project has lifetime of 15 years and expected to run beyond the project certification period. With carbon finance it is further strengthen to operate 100% project systems with continuous maintenance. | Not Applicable |
| >> |  |  |  |
| Principle 7.1 Emissions |  |  |  |
| Will the Project increase greenhouse gas emissions over the Baseline Scenario? | No | No. The project will replace firewood use with biogas. Hence, it will reduce greenhouse gas emissions over the Baseline Scenario. | Not Applicable |
| >> |  |  |  |
| Principle 7.2 Energy Supply |  |  |  |
| Will the Project use energy from a local grid or power supply (i.e., not connected to a national or regional grid) or fuel resource (such as wood, biomass) that provides for other local users? | No | No. The project uses inhouse cattle dung only. | Not Applicable |
| >> |  |  |  |
| Principle 8.1 Impact on Natural Water Patterns/Flows |  |  |  |
| Will the Project affect the natural or pre-existing pattern of watercourses, ground-water and/or the watershed(s) such as high seasonal flow variability, flooding potential, lack of aquatic connectivity or water scarcity? | No | No. Water in huge quantity is not required for the project which can impact the ground water level or any seasonal flow. Water is needed to mix with cattle dung while feeding in digester. | Not Applicable |
| >> |  |  |  |
| Principle 8.2 Erosion and/or Water Body Instability |  |  |  |
| - Could the Project directly or indirectly cause additional erosion and/or water body instability or disrupt the natural pattern of erosion? - Is the Project’s area of influence susceptible to excessive erosion and/or water body instability? | No | No. Water in huge quantity is not required for the project which can impact the ground water level or any seasonal flow. Water is needed to mix with cattle dung while feeding in digester | Not Applicable |
| >> |  |  |  |
| Principle 9.1 Landscape Modification and Soil |  |  |  |
| Does the Project involve the use of land and soil for production of crops or other products? | No | No. Since the project takes place in individual households, no net degradation in existing landscape function and services takes place. | Not Applicable |
| >> |  |  |  |
| Principle 9.2 Vulnerability to Natural Disaster |  |  |  |
| Will the Project be susceptible to or lead to increased vulnerability to wind, earthquakes, subsidence, landslides, erosion, flooding, drought or other extreme climatic conditions? | No | No. The project happens at individual households. There is no activity which can affect adversely the natural system to cause earthquake, landslides, erosion, flooding, draught or other extreme climatic conditions. | Not Applicable |
| >> |  |  |  |
| Principle 9.3 Genetic Resources |  |  |  |
| Could the Project be negatively impacted by or involve genetically modified organisms or GMOs (e.g., contamination, collection and/or harvesting, commercial development, or take place in facilities or farms that include GMOs in their processes and production)? | No | Not applicable. The project does not involve any crop production or cultivation. | Not Applicable |
| >> |  |  |  |
| Principle 9.4 Release of pollutants |  |  |  |
| Could the Project potentially result in the release of pollutants to the environment? | No | No. The project does not release any pollutants to the environment. | Not Applicable |
| >> |  |  |  |
| Principle 9.5 Hazardous and Non-hazardous Waste |  |  |  |
| Will the Project involve the manufacture, trade, release, and/ or use of hazardous and non-hazardous chemicals and/or materials? | No | Not applicable. The project does not involve any production process. | Not Applicable |
| >> |  |  |  |
| Principle 9.6 Pesticides & Fertilisers |  |  |  |
| Will the Project involve the application of pesticides and/or fertilisers? | No | Not applicable. The project does not involve any crop production or cultivation. | Not Applicable |
| >> |  |  |  |
| Principle 9.7 Harvesting of Forests |  |  |  |
| Will the Project involve the harvesting of forests? | No | Not applicable. The project happens at individual households. |  |
| >> |  |  |  |
| Principle 9.8 Food |  |  |  |
| Does the Project modify the quantity or nutritional quality of food available such as through crop regime alteration or export or economic incentives? | No | Not applicable. The project activity is biogas digester installation happens at individual households. | Not Applicable |
| >> |  |  |  |
| Principle 9.9 Animal husbandry |  |  |  |
| Will the Project involve animal husbandry? | No | Not applicable. The project activity is biogas digester installation happens at individual households. | Not Applicable |
| >> |  |  |  |
| Principle 9.10 High Conservation Value Areas and Critical Habitats |  |  |  |
| Does the Project physically affect or alter largely intact or High Conservation Value (HCV) ecosystems, critical habitats, landscapes, key biodiversity areas or sites identified? | No | Not applicable. The project activity is biogas digester installation happens at individual households. | Not Applicable |
| >> |  |  |  |
| Principle 9.11 Endangered Species |  |  |  |
| a. Are there any endangered species identified as potentially being present within the Project boundary (including those that may route through the area)? - Does the Project potentially impact other areas where endangered species may be present through transboundary affects? | No | Not applicable. The project activity is biogas digester installation happens at individual households. | Not Applicable |
| >> |  |  |  |

### Appendix 2- Contact information of project participants

| Organization name | Greneity Infocom Service Pvt Ltd |
| --- | --- |
| Registration number with relevant authority | CIN-U74900DL2013PTC253219 under Companies Act,1956 |
| Street/P.O. Box | Dilshad Garden |
| Building | 137-B, Pocket-A |
| City | Delhi |
| State/Region | Delhi |
| Postcode | 110095 |
| Country | India |
| Telephone | +91-9886252062 |
| E-mail | Shivani.garg@greneity.com |
| Website | www.greneity.com |
| Contact person | Shivani Garg |
| Title | Managing Director |
| Salutation | Ms. |
| Last name | Garg |
| Middle name |  |
| First name | Shivani |
| Department |  |
| Mobile | +91- 9886252062 |
| Direct tel. |  |
| Personal e-mail | Shivani.garg@greneity.com |

### Appendix 3- LUF Additional Information

| Risk of change to the Project Area during Project Certification Period: |  |
| --- | --- |
| Risk of change to the Project activities during Project Certification Period: |  |
| Land-use history and current status of Project Area: |  |
| Socio-Economic history: |  |
| Forest management applied (past and future) |  |
| Forest characteristics (including main tree species planted) |  |
| Main social impacts (risks and benefits) |  |
| Main environmental impacts (risks and benefits) |  |
| Financial structure |  |
| Infrastructure (roads/houses etc): |  |
| Water bodies: |  |
| Sites with special significance for indigenous people and local communities ‐ resulting from the Stakeholder Consultation: |  |
| Where indigenous people and local communities are situated: |  |
| Where indigenous people and local communities have legal rights, customary rights or sites with special cultural, ecological, economic, religious or spiritual significance: |  |

### Appendix 4-Summary of Approved Design Changes

Please refer to Design Change Requirements for more information on procedures governing Design Changes

##### Revision History

| Version | Date | Remarks |
| --- | --- | --- |
| 1.2 | 14 October 2020 | Hyperlinked section summary to enable quick access to key sections Improved clarity on Key Project Information Inclusion criteria table added Gender sensitive requirements added Prior consideration (1 yr rule) and Ongoing Financial Need added Safeguard Principles Assessment as annex and a new section to include applicable safeguards for clarity Improved Clarity on SDG contribution/SDG Impact term used throughout Clarity on Stakeholder Consultation information required Provision of an accompanying Guide to help the user understand detailed rules and requirements |
| 1.1 | 24 August 2017 | Updated to include section A.8 on ‘gender sensitive’ requirements |
| 1.0 | 10 July 2017 | Initial adoption |

Commissioning certificates co-signed by the respective plant owners and Dhaneswari Mahila Sewa Sansthan.

https://www.ctae.ac.in/images/editorFiles/file/CTAE-2017/Biogas%20FAQ_English.pdf (Refer the page no. 14 point 25 of the document.)

https://fsi.nic.in/cover_2011/chapter7.pdf (page no. 76 and 77)

https://fsi.nic.in/cover_2011/chapter7.pdf (page no. 76 and 77)

https://fsi.nic.in/cover_2011/chapter7.pdf (page no. 76 and 77)

https://fsi.nic.in/isfr19/vol2/isfr-2019-vol-ii-punjab.pdf (page no. 212 of the document)

CDM Methodological Tool -30: Calculation of the fraction of non-renewable biomass, version 03

https://moef.gov.in/wp-content/uploads/2019/06/Pacific.pdf (page no. 16 of Asia-Pacific Forestry Sector Outlook Study II India Outlook Study 2020)